3GPP TSG-RAN WG4 Meeting # 106	R4-2303635

Athens, Greece, 27 February –03 March, 2023

Title: 	WF on Inconsistency Issue for Intra-band EN-DC Band Combinations

Agenda Item:	12.5

Source: 	AT&T

Document for:	Approval

# Case 3 solutions/updates

## Update solution for Option 2c

Update solution for Option 2c as below.

    - Option 2c:From Rel-16, UE shall also support non-contiguous operation in the DL (DC_48C_n48A or DC_48D_48A), then the network can configure DL_(n)48CA or DC_(n)48DA with the middle LTE cell DL-only and the UL with a gap (non-contiguous)

<Agreement>: RAN4 to check if the updated Option 2c can be agreed in the next meeting.

## Extend the meaning of ‘both’ in the IE intraBandENDC-Support for B48+n48 EN-DC in both case 3 and case 4

-	Option 1: Extend the meaning of ‘both’ in the IE intraBandENDC-Support for B48+n48 EN-DC in both case 3 and case 4.

-	Option 2: Keep existing RAN4 understanding of ‘both’ as communicated in the RAN4 LS to RAN2 in R4-2220837.

<Way forward>: Companies are requested to provide further input next meeting

# Case 4 solutions/updates

## Update solution for Option 2b

Update solution for Option 2b from the WF in R4-2220589 as below.

    - Option 2b: From Rel-16, bandcombinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e.

      - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A

      - The Case 4 is limited to one NR sub-block (band entry) with one or more E-UTRA sub-blocks, the intraBandENDC-Support still indicting the relation between any one of the E-UTRA sub-blocks (band entries) and the single NR sub-block

    - Check with RAN2 about option 2b and new signalling is not precluded for case 4.

<Agreement>: RAN4 to check if the updated Option 2b can be agreed in the next meeting.

## Future extensions

-	Option 1: Consider future extension with intra-band EN-DC configurations having three band entries with two entries from NR band and one entry from E-UTRA band. Decide whether the following configurations in ‘Case x’ and ‘Case y’ will be possible or not in future extensions

- ‘Case x’:    DL: DC_(n)XCA    UL: DC_XA_nXA

- ‘Case y’:    DL: DC_YA-(n)YAA    UL: DC_YA_nYA.

-	Option 2: Defer until RAN4 develops common understanding on existing cases.

-	Option 3: Wait for response from RAN2 to RAN4 LS in R4-2220837.

<Way forward>: Companies are requested to provide further input next meeting

# RAN4 Specification Updates

## Remove non-contiguous UL EN-DC configuration DC_41A_n41A for contiguous DL EN-DC configurations

<Agreement>:

Agree to ZTE CRs in R4-2302560, R4-2302561 (CAT-A), R4-2302562 (CAT-A), and R4-2302563 (CAT-A).

## Remove non-contiguous UL EN-DC configuration DC_48A_n48A for contiguous DL EN-DC configurations

<Agreement>:

Revise Apple CR in R4-2300379 to remove the DC_41A_n41A UL changes which will be covered in the ZTE CRs from Rel-15. Update the corresponding Cat-A CRs in R4-2300380 (CAT-A) and R4-2300381 (CAT-A). The revised CRs can be agreed.

## Handling of Intra-band EN-DC configurations in the specification

-	Option 1: RAN4 to specify Config#1 (both UL and DL has contiguous spectrum), Config#2 (both UL and DL has non-contiguous spectrum) and Config#3 (UL and DL has different contiguity) in different sub-clauses.

-	Option 2: Keep existing clauses and handle any combinations with intra-band DL contiguous EN-DC with intra-band UL non-contiguous EN-DC in mixed contiguous and non-contiguous tables.

<Way forward>: Companies are requested to provide further input next meeting

# LS to RAN2

-	Option 1: Option 1: Send an LS to RAN2 to list a set of scenarios in the LS to evaluate if they could be implemented under the new interpretation of “both” in RAN4 and to consider a future extension with intra-band EN-DC configurations having three band entries with two entries from NR band and one entry from E-UTRA band. (R4-2302568, ZTE).

-	Option 2: Defer any further LS to RAN2 until getting the feedback from RAN2 concerning the LS sent in RAN4#105.

<Agreement>: Option 2