3GPP TSG-RAN WG4 Meeting #106-bis-e                          R4-2304065

Online, April 17 – 26, 2023

Source: 	 SK Telecom, Murata Manufacturing Co. Ltd

Title: 		 TP for TR 37.718-11-21 to include DC_1-5_n40-n77

Agenda Item: 	 4.6.2

Document for:  Approval

- Introduction

A text proposal for TR 37.718-11-21 to include DC_1-5_n40-n77.

- Text Proposal

※ Please note the following status regarding the fallback combinations of DC_1A-5A_n40A-n77A and DC_1A-5A_n40A-n77(2A).

(1) The following band combinations were specified in TS 38.101-3 V18.0.0 (2022-12).

- DC_1A-5A_n40A

- DC_1A-5A_n77A

- DC_1A-5A_n77(2A)

(2) The following band combinations were included in R4-2300762(RP-230540), which was agreed(approved) in the last RAN4#106(RAN#99) meeting.

- DC_1A_n40A-n77A

- DC_5A_n40A-n77A

(3) R4-2304063, the draft CR for the following band combinations, is submitted to RAN4#106-bis-e.

- DC_1A_n40A-n77(2A)

- DC_5A_n40A-n77(2A)

--- Start of Changes ---

7.x	DC_1-5_n40-n77

7.x.1	Operating bands for DC

Table 7.x.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-5_n40-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 7.x.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-5A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |

7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-5A_n40A-n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-5A_n40A-n77(2A) |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.x.3	UE co-existence studies

Co-existence studies of DC_1-5_n40-n77 are already covered in the constituent fallback modes. Therefore, there is no additional harmonic and inter-modulation impact for the additional band receiver.

7.x.4	∆TIB and ∆RIB values

Referring to DC_3_n5-n40-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.x.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-5_n40-n77 | 0.6 | 0.6 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.x.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-5_n40-n77 | 0.2 | 0.2 | 0.48 | 0.5 |
| NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.x.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

--- End of Changes ---

- Reference

[1] 3GPP TS 38.101-1 V18.0.0 (2022-12), User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)

[2] 3GPP TS 38.101-3 V18.0.0 (2022-12), User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 18)

[3] 3GPP TS 36.101 V18.0.0 (2022-12), User Equipment (UE) radio transmission and reception (Release 18)

[4] 3GPP TR 37.718-11-21 V0.3.0 (2022-11), Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) (Release 18)