3GPP TSG-RAN WG4 Meeting # 106-bis-e				     R4-2304208

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.14.3

Source: 	CMCC

Title: 	Discussion on ATG BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN4#106 meeting, WF [1] was approved. In this contribution, we’d like to provide our views on BS RF requirements.

# 2. Discussion

1. ATG operating band

After last meeting discussion, n3, n34 and n41 had been added as the operating band of ATG. We use 2GHz and 4Ghz as simulation carriers, and TDD band n39 is around these frequencies. So the same RF requirements of ATG operating bands can be used for n39, such as ACLR and ACS requirements. Consider of the operator deployment, we suggest to add TDD band n39 as ATG operating band.

Proposal 1: It’s suggested to add TDD bands n39 as the operating band of ATG.

2. Tx IM requirements

Rx IM requirements had been defined to reuse the existing requirement in TS 38.104. In our opinion, Tx IM requirements can guarantee the performance of the ATG BS. So regardless of the deployment scenario, Tx IM requirements will be needed and the existing requirement defined in TS 38.104 could be reused.

Proposal 2: The existing requirement in TS 38.104 for Tx IM requirements could be reused.

# 3. Conclusions

In this contribution, ATG BS RF requirements are discussed with the following proposals:

Proposal 1: It’s suggested to add TDD bands n39 as the operating band of ATG.

Proposal 2: The existing requirement in TS 38.104 for Tx IM requirements could be reused.

# 4. References

[1] R4-2302904, WF for ATG BS RF requirements, ZTE