3GPP TSG-RAN WG4 Meeting #106bis-e		R4-2304259

Online, April 17 – 26, 2023

Agenda item:	5.13.5.1

Source:	Intel Corporation

Title:	General aspects on FR2 HST simultaneous multi-panel demodulation

Document for:	Discussion

# Introduction

According to WID [1], the following objectives of performance part of FR2 HST enhancement WI have been agreed:

- Specify the necessary RRM test cases based on the outcome on corresponding core part. - Investigate and if needed specify the demodulation performance requirements for intra-band carrier aggregation (CA) HST scenario. - Specify the necessary demodulation performance requirements for simultaneous multi-panel reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Specify the other necessary RRM and demodulation performance requirements depending on the outcome of core part.

In RAN4 #106, the following agreements [2] on transmission scheme and deployment scenario for RRM requirements study of FR2 HST with multi-panel reception have been reached:

Agreements: - RAN4 to focus on bi-directional deployment in Scenario-A and Scenario-B to identify RRM requirement. - RAN4 to follow Rel-17 NR FR2 HST for CPE configuration assumptions where the CPE UE is equipped with 2 panels each for both TX and RX pointing in opposite directions. - RAN4 to focus on NC JT as the main transmission scheme in Rel-18 HST FR2 enhanced deployments.

In this paper, we provide our view on transmission scheme, receiver assumption and deployment scenario for DL demodulation performance simulation of FR2 HST simultaneous multi-panel reception.

# Discussion

## Transmission scheme and receiver assumption

Regarding multi-panel reception, as highlighted above in WID, this WI shall focus on FR2 HST specific aspects and avoid the overlap with the scope of FR2 multi-Rx DL reception WI. This implies that the WI should leverage the synergy with FR2 multi-Rx DL reception WI and the study should base on general aspects agreed in FR2 multi-Rx DL reception WI. According to [2], NC JT is considered as the main transmission scheme for RRM requirements study in this WI. Moreover, mDCI with full-overlapping resources based mTRP transmission scheme has been agreed in [3] for UE RF requirements study of FR2 multi-Rx DL reception. Based on the related agreements [4] on UE demodulation performance requirements, we proposed to focus on the same mTRP transmission scheme for UE demodulation performance study of FR2 multi-Rx DL reception in our companion contribution [5]. As such, we also propose to focus on the same mTRP transmission scheme, i.e., mDCI with full-overlapping resources based mTRP transmission scheme, for DL demodulation performance study of FR2 HST simultaneous multi-panel reception.

Proposal 1:	Consider both sDCI with SDM and mDCI with full-overlapping resource based mTRP schemes, and prioritize mDCI with full-overlapping resources based mTRP scheme for DL demodulation performance study of FR2 HST simultaneous multi-panel reception.

According to the agreements [3] on UE receiver assumption, no joint detection/decoding algorithm are considered for FR2 multi-Rx DL reception study. Instead, two Rx processing branches are assumed to receive two TRP-to-RX links separately. Moreover, based on the related agreements in [4], we further proposed in [5] to adopt the same receiver assumption for UE demodulation performance evaluation for FR2 multi-Rx DL reception. Currently no justification is identified to adopt different receiver assumption for FR2 HST simultaneous multi-panel reception. Therefore, we propose to adopt the same receiver assumption for FR2 HST simultaneous multi-panel study as that made in FR2 multi-Rx DL reception WI.

Proposal 2:	Adopt the receiver assumption agreed in FR2 multi-Rx DL reception WI, i.e, UE processing with 2x2 per TRP per Rx branch, for DL demodulation performance study of FR2 HST simultaneous multi-panel reception.

## Deployment scenario

In RAN4 spec [6], both uni- and bi-directional deployment scenarios have been specified for the propagation channel conditions used for FR2 HST-DPS performance requirement evaluations. Both deployment scenarios share the same RRH location parameter values in terms of inter-RRH distance Ds and the minimum distance Dmin from RRH to rail track. It is reasonable to reuse these RRH location parameters’ values for DL demodulation performance study of FR2 HST simultaneous multi-panel reception as well.

Proposal 3:	Reuse the current RRH location parameters’ values of FR2 HST-DPS performance requirements evaluation for DL demodulation performance study of FR2 HST simultaneous multi-panel reception.

Figure 1. Bi-directional deployment of FR2 HST dual-panel simultaneous reception

According to the above agreements [2] on deployment scenario, RRM requirements study for FR2 HST simultaneous multi-panel reception will focus on RRH bi-directional deployment. As illustrated in Fig. 1, bi-directional deployment provisions dual RRH coverage at every location on the rail track, and consequently the FR2 HST CPE UE with multi-panel can simultaneously receive PDSCH from the two serving RRHs. Specifically, each RRH provides two beam serving ranges, namely left-beam serving range (L-range) and right-beam serving range (R-range), which are in the left and the right sides of the RRH, respectively, and symmetrical about the RRH. The gap between the L- and the R-range of an RRH forms a “shadow” region around the RRH, which is in the middle of the “shadow” region. However, the “shadow” region of the RRH is under the coverage of the two closest neighbour RRHs. It is shown from Fig. 1 that shadow region of an RRH starts from the RRH left-beam switching point and ends at the RRH right-beam switching point. It is noted the L-ranges of bi-directional deployment in Fig. 1 correspond to beam serving ranges in unidirectional deployment for FR2 HST-DPS performance requirements evaluation [6]. Therefore, the distance from a beam switching point to the respective RRH, namely Ds_offset, in uni-directional deployment for FR2 HST-DPS performance evaluation [6] can be also reused to determine the locations of left- and right-beam switching points in bi-directional deployment in Fig. 1.

Proposal 4:	Reuse the beam serving range of uni-directional FR2 HST-DPS performance evaluation as the left-beam serving range of an RRH in bi-directional deployment, and right-beam serving range is symmetrical to the left-beam serving range about the RRH.

## Channel model

It is obvious that the channel model for bi-directional deployment FR2 HST scenario illustrated in Fig. 1 is similar to that considered in the existing FR2 HST DPS test case in [6], i.e., single path non-fading channel. As describe in [6], when HST is moving on the rail track, the timing offset of the received signals from two serving RRHs depends on the propagation delay difference of two signals corresponding to the respective serving RRHs, and Doppler shifts of two received signals depend on the HST velocity and its relative location with respect to the serving RRHs. As such, both timing offset and Doppler shifts are time varying and location dependent characteristics. We propose reuse the existing FR2 HST DPS channel model including the effects of time-varying location-dependent timing offset and Doppler shifts for FR2 HST with simultaneous multi-panel reception performance study.

Proposal 5:	Reuse the existing FR2 HST DPS channel model to perform FR2 HST simultaneous multi-panel reception performance evaluation.

## Scope of demodulation performance requirements

Since this WI is about HST reception performance study, it is clear that only UE PDSCH demodulation performance requirements will be studied, and BS demodulation performance requirement are not in the scope.

Proposal 6:	Only UE PDSCH demodulation requirements are studied in this WI. And BS demodulation requirements are out of the scope.

# Conclusion

In this paper we provided view on transmission scheme, receiver assumption and deployment scenario for DL demodulation performance simulation of FR2 HST simultaneous multi-panel reception and made the following proposals:

Proposal 1:	Focus on mDCI with full-overlapping resources based mTRP scheme for DL demodulation performance study of FR2 HST simultaneous multi-panel reception.

Proposal 2:	Adopt the receiver assumption agreed in FR2 multi-Rx DL reception WI, i.e, UE processing with 2x2 per TRP per Rx branch, for DL demodulation performance study of FR2 HST simultaneous multi-panel reception.

Proposal 3:	Reuse the current RRH location parameters’ values of FR2 HST-DPS performance requirements evaluation for DL demodulation performance study of FR2 HST simultaneous multi-panel reception.

Proposal 4:	Reuse the beam serving range of uni-directional FR2 HST-DPS performance evaluation as the left-beam serving range of an RRH in bi-directional deployment, and right-beam serving range is symmetrical to the left-beam serving range about the RRH.

Proposal 5:	Reuse the existing FR2 HST DPS channel model to perform FR2 HST simultaneous multi-panel reception performance evaluation.

Proposal 6:	Only UE PDSCH demodulation requirements are studied in this WI. And BS demodulation requirements are out of the scope.

# References

- RP-220985, “New WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2)”, Samsung, RAN #94, March 2022.

- R4-2303173, “WF on NR FR2 HST Enhancements RRM requirements”, Samsung, RAN4 #106, February 2023.

- R4-2303708, “WF on NR FR2 multi-Rx chain DL reception”, Samsung and Apple, RAN4 #106, February 2023.

- R4-2302941, “WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception” Qualcomm, RAN4 #106, February 2023.

- R4-2304258, “General aspects on NR FR2 multi-Rx DL demodulation”, Intel, RAN4 #106b-e, April 2023.

- TS 38.101-4, NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, Release 17, January 2023.