3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304383

# Electronic Meeting, April 17 – April 26, 2023

Agenda Item:	5.10.3.2

Source: 		Qualcomm Incorporated

Title: 			Discussion on requirements of inter-RAT measurement without gaps.

Document for:	Discussion

# 1.	Introduction

In this paper, we discuss remaining issue about the requirements for the requirements of Inter-RAT measurements without gap from the last meeting [1].

# 2. 	Discussion

## UE capability

### Issue 2-2-2: On top of which UE capability to support the inter-RAT LTE measurement requirements when UE has vacant RF chain available(Case b-1) < Way forward >: - Option 1: Apple, Qualcomm, Intel, CATT, vivo, OPPO, Huawei, Ericsson, MTK, xiaomi - ONLY on top of ‘nogap-noncsg’ in NeedForNCSG-InfoEUTRAN to define the UE capability to support Case b-1 - Option 2: CMCC - Both NeedForNCSG-InfoEUTRAN and NeedForGap shall be considered also. ### Issue 2-2-4: On top of which UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2) < Way forward/ >: - Option 1: OPPO, Xiaomi - extend the capability of NeedForGaps - Option 2: Apple, Qualcomm, Intel, CATT, CMCC, vivo,Huawei, Ericsson, MTK - A new per-UE capability should be defined - FFS on the detailed design on this new IE ### Issue 2-2-5: Additional capability to support inter-RAT measurement without gap with mixed numerology < Way forward >: - Option 1: CATT, Huawei, MTK - No additional UE capability is defined for inter-RAT measurement with mixed numerology; instead it can be considered for scheduling restriction - Option 2: MTK - a UE capability can be defined for mixed numerology incapable UEs (FFS) - Option 2a: Intel, vivo - a UE capability for inter-RAT NR can be defined (FFS)

For Inter-RAT LTE measurements, there is existing reporting capability in R17. However, the delay requirements are not defined when UE report ‘nogap-noncsg’ via ‘NeedForNCSG-InfoEUTRAN’. RAN4 should consider using R17 reporting framework and define the missing delay requirements.

Proposal: RAN4 only consider NeedforNCSG-InfoEUTRAN capability to indicate inter-RAT LTE measurement without gap when UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements.

For case b-2, LTE CRS is in UE’s active BWP. From our understanding, UE already performs measurement without gap for LTE CRS as rate matching is applied for LTE CRS when UE is in DSS. To avoid such confusion, we think RAN4 shall define applicability rule when the requirements for case b-2 is applied.

Observation: UE already performs measurement without gap for LTE CRS as LTE CRS is protected by rate matching when UE is in DSS. Scenario needs to be specified to apply the requirements.

Proposal: RAN4 shall define the applicability rule of the requirements for inter-RAT LTE measurement without gap for caseb-2 as UE is in DSS but LTE cell is not serving.

Since active BWP can be changed dynamically, it is difficult to have dedicate reporting capability. Similar to R16 inter-frequency measurement without gap when SSB is located at active BWP, new capability needs to be introduced to support this scenario.

Observation: It is difficult to have dedicate reporting capability because active BWP can be changed dynamically. Same mechanism for inter-frequency measurement without gap when SSB is located at active BWP can be applied.

Proposal: New capability [interRAT-frequencyMeas-NoGap-R18] shall be defined indicate whether the UE can perform inter-RAT frequency measurements without gap if the LTE CRS are fully contained in the active BWP.

Regarding additional capability to support inter-RAT measurement without gap with mixed numerology, it is not clear the motivation of defining additional capability and what is the problem not having such capability. From our understanding, caseb-2 should not be considered as DSS is only supported for same numerology.

Observation: It is not clear the motivation of defining additional capability for mixed numerology and what is the problem not having the capability. From our understanding, caseb-2 should not be considered as DSS is only supported for same numerology.

Proposal: No need to define additional capability for mixed numerology.

## Measurement requirements

### Inter-RAT NR measurements without gap

In 36.133, there are two measurement requirements for inter-RAT NR based on EN-DC is configured or not. At 8.2.4.21.1, 36.133, the requirement is applied for NR capable UE when not configured with EN-DC. The requirement is applicable for measurement without gap for FR2 as following:

8.1.2.4.21, 36.133 E-UTRAN FDD – NR measurements For per-FR measurement gap capable UE, when serving cells are in E-UTRA and measurement objects are only in FR2, - UE can perform such measurements without gap, and - UE fulfils the requirements for FR2 measurement objects based on effective MGRP = 20 ms.

However, the requirement for FR1 is not defined.

At 8.17.4, inter-RAT NR measurement for NR capable UE when EN-DC is configured. The current spec indicates that the requirement can be applicable for measurements without gap.

8.17.4.1.2, 36.133 NR Inter-RAT measurement When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting NR SS-RSRP, NR SS-RSRQ and NR SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses 9.11.1, 9.11.2 and 9.11.3, respectively, with a measurement period given by:

However, it is not clear what effective MGRP can be used in current spec.

Observation: Legacy Inter-RAT NR measurement can be reused for inter-RAT NR measurement without gap, but effective MGRP is not yet defined.

Proposal: RAN4 consider using 8.17.4, 36.133 inter-RAT NR measurement delay requirements with No DRX as baseline and define effective MGRP for measurements without gap for FR1 and FR2. (e.g MGRP = 80ms)

### Inter-RAT LTE measurement without gap

From R17 NCSG, there was discussion to define delay requirements for inter-RAT LTE measurement without gap. The requirements in 9.4 inter-RAT measurements can be used as baseline but RAN4 needs update parameters.

    - TIdentify,  E-UTRAN FDD=TBasicIdentify∙480TInter1∙CSSFinterRAT      ms

Proposal: RAN4 consider the requirements in 9.4 inter-RAT LTE measurements as baseline and RAN4 needs to update definition of Tinter1 and CSSFinterRAT.

Proposal: Same delay requirements can be applicable for both case b-1 and case b-2.

Depending on UE implementation both tight and relaxed delay requirements can be considered. Tight delay requirements can be applicable when UE prioritizes performing measurement for LTE carriers with consuming more power. On the other hand, relaxed delay requirements can be applicable when UE prioritize balancing energy consumption while performing LTE carrier measurements.

Proposal: Multiple Tinter1 (30ms, 60ms) can be considered, and new indicator needs to be introduced for UE to indicate NW what Tinter1 is applied.

# 3. Conclusions

Proposal: RAN4 only consider NeedforNCSG-InfoEUTRAN capability to indicate inter-RAT LTE measurement without gap when UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements.

Observation: UE already performs measurement without gap for LTE CRS as LTE CRS is protected by rate matching when UE is in DSS. Scenario needs to be specified to apply the requirements.

Proposal: RAN4 shall define the applicability rule of the requirements for inter-RAT LTE measurement without gap for caseb-2 as UE is in DSS but LTE cell is not serving.

Observation: It is difficult to have dedicate reporting capability because active BWP can be changed dynamically. Same mechanism for inter-frequency measurement without gap when SSB is located at active BWP can be applied.

Proposal: New capability [interRAT-frequencyMeas-NoGap-R18] shall be defined indicate whether the UE can perform inter-RAT frequency measurements without gap if the LTE CRS are fully contained in the active BWP.

Observation: It is not clear the motivation of defining additional capability for mixed numerology and what is the problem not having the capability. From our understanding, caseb-2 should not be considered as DSS is only supported for same numerology.

Proposal: No need to define additional capability for mixed numerology.

Observation: Legacy Inter-RAT NR measurement can be reused for inter-RAT NR measurement without gap, but effective MGRP is not yet defined.

Proposal: RAN4 consider using 8.17.4, 36.133 inter-RAT NR measurement delay requirements with No DRX as baseline and define effective MGRP for measurements without gap for FR1 and FR2. (e.g MGRP = 80ms)

Proposal: RAN4 consider the requirements in 9.4 inter-RAT LTE measurements as baseline and RAN4 needs to update definition of Tinter1 and CSSFinterRAT.

Proposal: Same delay requirements can be applicable for both case b-1 and case b-2.

Proposal: Multiple Tinter1 (30ms, 60ms) can be considered, and new indicator needs to be introduced for UE to indicate NW what Tinter1 is applied.

# 4.    References

[1] R4-2303305, WF on NR_MG_enh2 Part 2, Intel, RAN4 #106