3GPP TSG-RAN4 Meeting #106-bis-e	R4-2304391

Online,, 17th April – 26th April 2023

Title:	Scenarios for dual SUL

Source:	Qualcomm Incorporated

Agenda item:	8.2

Release:	Rel-18

Document for:	Approval

# 1.	Introduction

RAN4 was tasked by RAN#99 [1] to assess a specification and UE implementation impact when UE is configured with two serving cells, each with SUL. In this paper we discuss more in detail the assignment of the RAN task and its impact.

# 2. 	Discussion

The further detail in the RAN task was clarified by sub bullets in the agreed slide. The slide is copied to the annex if this document .

The sub bullets indicate the following cases:

- Can two SUL carriers (on different bands) transmit back-to-back

- Can a SUL carrier and non-corresponding NUL carrier transmit back-to-back

Observation 1: Case 1 contains the assumption that two SUL carrier (on different bands) can not transmit at the same time since if this were the case, the question would be moot.

For the same reason,

Observation 2: Case 2 contains the assumption that SUL carrier and non-corresponding NUL carrier can not transmit at the same time.

Further assumption is written in the second sub-bullet and it means this scenario of dual SUL uplink is applicable for TX switching, according to plenary agreement. It should be noted that assumptions is that UE can be configured with two SULs for uplink, and what is written in to dual SUL WID [2]:

“NOTE 2: No simultaneous uplink configuration on the two SUL bands according to Table 1.”

is no longer valid.

One more aspect to be noted is that

Observation 3: Two NUL carriers can transmit at the same time

In conclusion, RAN4 is tasked to analyse what are the time relations of the actual transmission between the two SUL configurations when UE is configured for dual SUL configurations.

To illustrate the scenario, we drew a timing diagram. To ensure the terminology is common: in an example configuration CA_SUL_n41A-n95A_SUL_n79A-n98A, the naming is as follows:

- n95: SUL1 and corresponding NUL1: n41

- n98: SUL1 and corresponding NUL2: n79

And also n41 is the non-corresponding NUL for the n98.

Figure 1: Illustration of the scenarios to be discussed in RAN4

Rel-15 SUL design assumes UE has two TX chains and uses those to transmit SUL and NUL without any delay, as stated in by the note 1 in Table 5.2C-1. 38.101-1 does not define any requirements for ON-OFF time mask for the SUL design which then means network can assume the transmission starts from the first sample of the grant without any transient period. In RAN4 terms, transient periods are assumed to be excluded from the transmissions as stated in clause 6.3.3.2 and in Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR1. The Figure 2 shows the specified output power behaviour between SUL and NUL for rel-15 behaviour.

Figure 2. Specified timing relations between SUL and corresponding NUL

Observation 4: If UE supports Rel-15 SUL configuration as defined containing SUL and corresponding NUL, both need their own TX chains.

The UE supporting back-to-back transmissions between two SUL carriers or between SUL and non-correstponding NUL can be implemented in two ways:

- UE is limited to two TX chains and can retune the second TX chain during the transmission of SUL1 from NUL1 to SUL2 or NUL2.

- UE has four TX chains.

The first option assumes that only one transmission can be scheduled at a time and retuning during a transmission has earlier concluded as problematic in ran4 due to glitches and other issue that increase EVM to the carrier where the transmissions are ongoing. As a conclusion, the first option converges to almost same functionality as TX switching option SwitchedUL without switching time so with more detailed analysis this option is not without problems and could be concluded that RAN1 involvement for this kind of behaviours is needed.

UE with four TX chains would be a new implementation assumption in RAN4.

Observation 5: Back-to-back transmissions between SUL and non-corresponding NUL and between two SULs are feasible but means new UE implementation assumptions in RAN4

If 4Tx UE work will be started in ran4, then the potential of such implementation should be fully utilized and this is best done with a new dedicated WI. Note that 4Tx work that is ongoing now is for 4 Tx chains for same frequency. This new WI would mean 4 TX chains on different frequency.

Proposal: RAN4 will respond to the LS [1] that back-to-back transmissions between SUL and non-corresponding NUL and between two SULs are feasible but specifying this means new UE implementation assumptions in RAN4 and in-order to properly specify UE with those new assumptions, new non-spectrum related work item is needed.

# Conclusion

We made five observations:

Observation 1: Case 1 contains the assumption that two SUL carrier (on different bands) can not transmit at the same time since if this were the case, the question would be moot.

Observation 2: Case 2 contains the assumption that SUL carrier and non-corresponding NUL carrier can not transmit at the same time.

Observation 3: Two NUL carriers can transmit at the same time

Observation 4: If UE supports Rel-15 SUL configuration as defined containing SUL and corresponding NUL, both need their own TX chains.

Observation 5: Back-to-back transmissions between SUL and non-corresponding NUL and between two SULs are feasible but means new UE implementation assumptions in RAN4

And one proposal:

Proposal: RAN4 will respond to the LS [1] that back-to-back transmissions between SUL and non-corresponding NUL and between two SULs are feasible but specifying this means new UE implementation assumptions in RAN4 and in-order to properly specify UE with those new assumptions, new non-spectrum related work item is needed.

# Reference

[1]	RP-230810, “Offline discussion summary on RAN1-MultiCarrier”, NTT DOCOMO, INC, 3GPP TSG RAN #99, Rotterdam, Netherlands, March 20th - 23rd, 2023

[2]	RP-223553, “NR CA band combinations with two SUL cells in Rel-18”, 	CMCC, 3GPP TSG RAN Meeting #98-e, Electronic Meeting, December 12-16, 2022

# Annex I: agreed slide from RAN#99