3GPP TSG-RAN WG4 Meeting #106-bis-e 	R4-2304399

Electronic meeting, 17th – 26th April, 2023

Source:	Ericsson

Title:	Initial simulation results for FR1 4Tx demodulation requirements

Agenda item:	9.6.6.2

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the FR1 4Tx demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

## 2.1 Simulation assumptions

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0, FFS on 4 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2.1-2 Simulation results with 70% of maximum throughput for 15kHz SCS, 5MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 1+1 | TDLB100-400 | 2 | A | -1.10 |
|  |  |  |  |  | B | -0.93 |
|  |  |  | TDLC300-100 | 12 | A | 12.14 |
|  |  |  |  |  | B | 12.11 |
|  |  |  |  | 16 | A | 18.18 |
|  |  |  |  |  | B | 18.44 |
|  |  |  | TDLA30-10 | 16 | A | 15.74 |
|  |  |  |  |  | B | 15.75 |
|  |  |  |  | 17 | A | 16.27 |
|  |  |  |  |  | B | 16.28 |
|  |  |  |  | 20 | A | 19.44 |
|  |  |  |  |  | B | 19.44 |
| 4 | 8 |  | TDLB100-400 | 2 | A | -4.65 |
|  |  |  |  |  | B | -4.56 |
|  |  |  | TDLC300-100 | 12 | A | 5.29 |
|  |  |  |  |  | B | 5.24 |
|  |  |  |  | 16 | A | 9.40 |
|  |  |  |  |  | B | 9.42 |
|  |  |  | TDLA30-10 | 16 | A | 7.86 |
|  |  |  |  |  | B | 7.86 |
|  |  |  |  | 17 | A | 8.56 |
|  |  |  |  |  | B | 8.56 |
|  |  |  |  | 20 | A | 11.18 |
|  |  |  |  |  | B | 11.18 |

Table 2.1-3 Simulation results with 70% of maximum throughput for 15kHz SCS and 20MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 1+1 | TDLB100-400 | 2 | A | -0.88 |
|  |  |  |  |  | B | -0.70 |
|  |  |  | TDLC300-100 | 12 | A | 12.17 |
|  |  |  |  |  | B | 12.24 |
|  |  |  |  | 16 | A | 18.46 |
|  |  |  |  |  | B | 19.16 |
|  |  |  | TDLA30-10 | 16 | A | 16.82 |
|  |  |  |  |  | B | 16.80 |
|  |  |  |  | 17 | A | 17.28 |
|  |  |  |  |  | B | 17.27 |
|  |  |  |  | 20 | A | 21.55 |
|  |  |  |  |  | B | 21.54 |
| 4 | 8 |  | TDLB100-400 | 2 | A | -4.61 |
|  |  |  |  |  | B | -4.51 |
|  |  |  | TDLC300-100 | 12 | A | 5.26 |
|  |  |  |  |  | B | 5.26 |
|  |  |  |  | 16 | A | 9.44 |
|  |  |  |  |  | B | 9.47 |
|  |  |  | TDLA30-10 | 16 | A | 8.24 |
|  |  |  |  |  | B | 8.24 |
|  |  |  |  | 17 | A | 8.90 |
|  |  |  |  |  | B | 8.90 |
|  |  |  |  | 20 | A | 11.78 |
|  |  |  |  |  | B | 11.78 |

Table 2.1-4 Simulation results with 70% of maximum throughput for 15kHz SCS and 50MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 1+1 | TDLB100-400 | 2 | A | -0.92 |
|  |  |  |  |  | B | -0.67 |
|  |  |  | TDLC300-100 | 12 | A | 12.26 |
|  |  |  |  |  | B | 12.34 |
|  |  |  |  | 16 | A | 19.12 |
|  |  |  |  |  | B | 20.29 |
|  |  |  | TDLA30-10 | 16 | A | 18.07 |
|  |  |  |  |  | B | 18.08 |
|  |  |  |  | 17 | A | 18.42 |
|  |  |  |  |  | B | 18.43 |
|  |  |  |  | 20 | A | 23.34 |
|  |  |  |  |  | B | 23.33 |
| 4 | 8 |  | TDLB100-400 | 2 | A | -4.66 |
|  |  |  |  |  | B | -4.55 |
|  |  |  | TDLC300-100 | 12 | A | 5.23 |
|  |  |  |  |  | B | 5.25 |
|  |  |  |  | 16 | A | 9.64 |
|  |  |  |  |  | B | 9.73 |
|  |  |  | TDLA30-10 | 16 | A | 8.64 |
|  |  |  |  |  | B | 8.64 |
|  |  |  |  | 17 | A | 9.33 |
|  |  |  |  |  | B | 9.33 |
|  |  |  |  | 20 | A | 12.13 |
|  |  |  |  |  | B | 12.14 |

Table 2.1-5 Simulation results with 70% of maximum throughput for 30kHz SCS and 10MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 1+1 | TDLB100-400 | 2 | A | -1.05 |
|  |  |  |  |  | B | -1.04 |
|  |  |  | TDLC300-100 | 12 | A | 12.49 |
|  |  |  |  |  | B | 12.60 |
|  |  |  |  | 16 | A | 19.34 |
|  |  |  |  |  | B | 19.20 |
|  |  |  | TDLA30-10 | 16 | A | 15.71 |
|  |  |  |  |  | B | 15.73 |
|  |  |  |  | 17 | A | 16.20 |
|  |  |  |  |  | B | 16.22 |
|  |  |  |  | 20 | A | 19.12 |
|  |  |  |  |  | B | 19.13 |
| 4 | 8 |  | TDLB100-400 | 2 | A | -4.42 |
|  |  |  |  |  | B | -4.40 |
|  |  |  | TDLC300-100 | 12 | A | 5.26 |
|  |  |  |  |  | B | 5.27 |
|  |  |  |  | 16 | A | 9.91 |
|  |  |  |  |  | B | 10.03 |
|  |  |  | TDLA30-10 | 16 | A | 8.06 |
|  |  |  |  |  | B | 8.06 |
|  |  |  |  | 17 | A | 8.69 |
|  |  |  |  |  | B | 8.70 |
|  |  |  |  | 20 | A | 11.19 |
|  |  |  |  |  | B | 11.19 |

Table 2.1-6 Simulation results with 70% of maximum throughput for 30kHz SCS and 100MHz CBW

| Tx | Rx | DM-RS | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 1+1 | TDLB100-400 | 2 | A | -1.25 |
|  |  |  |  |  | B | -1.19 |
|  |  |  | TDLC300-100 | 12 | A | 13.51 |
|  |  |  |  |  | B | 13.75 |
|  |  |  |  | 16 | A | 20 |
|  |  |  |  |  | B | 19.53 |
|  |  |  | TDLA30-10 | 16 | A | 17.33 |
|  |  |  |  |  | B | 17.33 |
|  |  |  |  | 17 | A | 17.73 |
|  |  |  |  |  | B | 17.73 |
|  |  |  |  | 20 | A | 22.39 |
|  |  |  |  |  | B | 22.40 |
| 4 | 8 |  | TDLB100-400 | 2 | A | -4.48 |
|  |  |  |  |  | B | -4.69 |
|  |  |  | TDLC300-100 | 12 | A | 5.50 |
|  |  |  |  |  | B | 5.52 |
|  |  |  |  | 16 | A | 10.25 |
|  |  |  |  |  | B | 10.37 |
|  |  |  | TDLA30-10 | 16 | A | 8.55 |
|  |  |  |  |  | B | 8.56 |
|  |  |  |  | 17 | A | 9.28 |
|  |  |  |  |  | B | 9.28 |
|  |  |  |  | 20 | A | 12.22 |
|  |  |  |  |  | B | 12.22 |

Table 2.1-7 Simulation results with 70% of maximum throughput for 15kHz SCS, 5MHz CBW for TPMI 4

| Tx | Rx | TPMI | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 4 | TDLB100-400 | 2 | A | -1.09 |
|  |  |  |  |  | B | -0.92 |
|  |  |  | TDLC300-100 | 12 | A | 12.14 |
|  |  |  |  |  | B | 12.11 |
|  |  |  |  | 16 | A | 18.14 |
|  |  |  |  |  | B | 18.47 |

Table 2.1-8 Simulation results with 70% of maximum throughput for 30kHz SCS, 10MHz CBW for TPMI 4

| Tx | Rx | TPMI | Channel model | MCS | Mapping type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 | 4 | TDLB100-400 | 2 | A | -1.04 |
|  |  |  |  |  | B | -1.03 |
|  |  |  | TDLC300-100 | 12 | A | 12.48 |
|  |  |  |  |  | B | 12.63 |
|  |  |  |  | 16 | A | 19.34 |
|  |  |  |  |  | B | 19.20 |

# Conclusions

Simulation results for FR1 4Tx demodulation requirements are delivered.

# References

[1]	R4-2302926, WF for FR1 4Tx demodulation requirements, Ericsson, RAN4#106-e