3GPP TSG-RAN WG4 Meeting # 106bis-e                                                       R4-2304432

# Online, April 17 – April 26, 2023

Title: 	TP for TR 38.858:“Impact on BS RF requirements”

Source: 	CATT

Agenda item:	5.20.2.3

Document for:	Discussion

Background

This contribution provides a draft TP for TR 38.858: Impact on BS RF requirements. The technical background and the summary of the SI discussion are provided in the TP.

Reference

[1]R4-2302887, “draft TR skeleton of RAN4 part for 38858 and work splitting for TR drafting”, CMCC

[2]R4-2302969, “WF for BS RF requirements impact for SBFD operation”, CATT

TP for TR 38.858

11	Impact on RF requirements

11.1 Impact on BS RF requirements

For Tx, SBFD transmits signal in the sub-band. In the SI discussion, sub-band bandwidth is still the legacy bandwidth. But as the SBFD operation is different with legacy operation, the Tx chain implementation is different, i.e. sub-band filter may be needed, etc. Then the sub-band Tx signal performance should be guaranteed.  The existing RF requirements as below are still applicable within SBFD time slot:

Table 11.1.1 : Analysis Requirements are applicable within SBFD time slot

|  | Requirements |
| --- | --- |
| Conducted RF requirement | BS output power |
|  | Output power dynamics |
|  | Transmitted signal quality |
|  | Occupied bandwidth |
|  | Dynamic range |
| Radiated RF requirement | Radiated transmit power |
|  | OTA base station output power |
|  | OTA output power dynamics |
|  | OTA transmitted signal quality |
|  | OTA occupied bandwidth |
|  | OTA dynamic range |

For SBFD, the biggest challenge is REFSENS degradation when simultaneous Tx and Rx happens. The detail analysis and evaluation are provided in clause 10. For both FR1 and FR2, the interference from Tx sub-band comes from the air, so antenna isolation helps much for the interference cancellation. Therefore, REFSENS should be tested as OTA sensitivity. In SI phase, the conclusion for new requirements is as following.

The following requirements are new requirements:

- OTA sensitivity within SBFD time slot

- Other requirements not precluded

There’re some other requirements which should be defined with different requirements comparing legacy BS. For transmitter intermodulation and OTA transmitter intermodulation requirements, 30 dB coupling loss is assumed between two co-location gNBs. For SBFD BS, Rx REFSENS requirements for this scenario needs to be discussed further, i.e. if the same interference level is assumed and if Tx sub-band is active for this test. For the similar reason, co-location out of band blocking requirements also needs to be discussed further. In SI phase, the followings were agreed.

The following requirements may be impacted:

- Transmitter intermodulation

- Co-location out of band blocking

There are some requirements which are not applicable. Transmit ON/OFF power requirement is not applicable for SBFD BS because transmit OFF power and transmitter transient period requirements apply only to TDD operation of the BS. OTA receiver spurious emission can’t be defined as it is superseded by the OTA TX spurious emissions requirement. This is due to the fact that TX and RX spurious emissions cannot be distinguished in OTA domain. In SI phase, the followings were agreed.

The following requirements are not applicable:

- Transmit ON/OFF power requirement within SBFD time slot

- OTA receiver spurious emissions