3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304492

# Electronic Meeting, April 17 – April 26, 2023

Title: 	Discussion on new IoT NTN FDD band (extended L-band) UE RF requirements

Agenda Item:	6.5.3

Source: 	MediaTek Inc.

Document for:	Approval

Introduction

Regarding the new approved WI of “New WID on introduction of the Extended L-band for IoT NTN” in RAN-Plenary#99 meeting, to define the UE RF TX and RX requirements for the new IoT NTN band (extended L-band) is needed. In the contribution, the consideration for UE RF TX and RX requirements are provided.

RF requirements

- 2.1	General view for band agnostic UE RF requirements

The new IoT NTN FDD band (extended L-band) spectrum WI [1] was approved.

First, regarding UE RF, the objective of the WI core part is show below for reference.

- Specify a new E-UTRA NTN FDD band, with initial scope for Region 1, with UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz

  - NOTE: The introduction of this new NTN band through this WID has no impact on existing 3GPP terrestrial band specifications.

- Support UE Categories NB1, NB2, M1;

- Support UE Power class 3 (+23dBm) and Power Class 5 (+20dBm);

For the eMTC/NB-IoT UE RF requirements, we distinguish band agnostic requirements from TS 36.102. In the table below, we provide comments on reusing these requirements if possible.

Table 2.1-1

| Requirement name | Band-agnostic? | Reuse 36.102 requirement? | Comments |
| --- | --- | --- | --- |
| Clause 6: Transmitter Characteristics |  |  |  |
| UE maximum output power | Yes | Yes | For category M1 and NB1/NB2, 23dBm with +/-2dB tolerance |
| UE maximum output power reduction | Yes | Yes | Different MPR tables for category M1 and NB1/NB2 separately. |
| UE additional maximum output power reduction | No |  |  |
| Spurious emissions for UE co-existence | No |  |  |
| Additional spurious emissions | No |  |  |
| 7: Receiver characteristics |  |  |  |
| Minimum requirements (QPSK) | No |  |  |
| In-band blocking | No | Yes if there is no specific concern | Different band group may have different Different In-band blocking tables for category M1 and NB1/NB2 separately. |
| Out-of-band blocking | No | See comments | Different Out-of-band requirements for b255 and b256; whether to leverage requirement from b255 or b256 needs further discussion. |
| Narrow band blocking | No | Yes if there is no specific concern |  |
| RX spurious emission | No | Yes if there is no specific concern |  |

Conclusion

In the contribution, the potential UE RF requirements are provided.

Observation 1: Regarding the new IoT NTN FDD band (extended L-band) UE RF requirements, to distinguish band-agnostic UE RF requirements from TS 36.102 is provided in Table 2.1-1.

Proposal 1: To agree the proposal in the Table 2.1-1 if there is no specific concern, or at least use them as the starting point for further discussions on the new IoT NTN FDD band (extended L-band) UE RF requirements.

- References

- RP-230790, "New WID on Introduction of the Extended L-band for IoT NTN", Inmarsat