3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-23xxxxx

Online, April 17 – April 26, 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on ATG UE Demodulation requirement - General

Agenda item:	5.14.5.1

Document for:	Approval

- Introduction

In the previous RAN4 Meeting #106, some initial agreements were reached on the General scope of Demodulation Requirements for ATG. In this contribution, we share our view on the open items.

# General

## Channel Models

In RAN4#106 two candidate channel models for the definition of ATG UE Demodulation requirements have been discussed:

- Option 1: Single path AWGN channel

- Option 2: NTN-TDL-C (FFS for the delay spread and K factor)

NTN Channel models are defined in 3GPP TR 38.811 [2], and their reference deployment scenario includes also airborne UAS Unmanned Aircraft Systems with altitude between 8 and 50km (see Deployment D5 in Table 5.1-1 in [2]).

We have identified some differences between the NTN channel models derived for Link Level simulations in [2], with respect to the ATG deployment scenario under analysis:

- The range of ATG UE height is expected to be most of the time below or at the lower end of the range considered by the NTN study for UAS, with typical aircraft altitude between 3 and 12 km;

- NTN Channel model design assumes that the terrestrial (non-airborne) terminal is a UE on the ground and experiences local scattering due to the surrounding environment. For ATG communications, the receiving terminal is expected to be an IMT BS, so the height and surrounding clutter are not expected to match the channel model design;

Observation 1: NTN Channel models defined in 3GPP TR 38.811 are not specifically designed to model ATG UE to ATG BS propagation;

Proposal 1: For ATG UE Demodulation, RAN4 to use Single Path channel without fading;

## Deployment scenarios and impact on Channel modelling

In the last meeting it was discuss whether the following two deployment scenarios for the scope of the Rel-18 ATG demodulation discussion should be discussed:

- Figure 1: Scenario of ATG of TN deployment (Bidirectional)

- Figure 2: Scenario of ATG of ATG deployment (Unidirectional)

RAN4 has agreed to introduce requirements for the Unidirectional deployment scenario (Figure 2), and to keep FFS the Bidirectional scenario (Figure 1).

In our view, in the framework on demodulation requirements, the difference between the two scenarios consists in the different propagation paths that the signal experiences when the UE:

- Propagation delay and delay jump when switching from BS k to BS (k+1);

- Doppler shift and doppler jump when switching from BS k to BS (k+1);

Both of these deployment scenarios are in many aspects similar to the scenarios that R4 has considered previously to be applicable for the FR2 HST channel model, and during the discussions in that WI the same aspects were considered.

Regarding propagation delay, the agreements reached during the FR2 HST work were that propagation delay and delay jump were not modeled in RAN4 Demod channel, and we there aren’t specific concerns to the ATG deployment, we expect the same conclusion to be applicable to this scenario.

Observation 2: RAN4 did not consider propagation delay or propagation delay differences in the FR2 HST Channel model discussion (for bidirectional and unidirectional);

Proposal 3: RAN4 to follow the approach used for FR2 HST Demod channel modeling and not to consider propagation delay or delay jump for ATG UE Demodulation Channel Modeling;

Regarding Doppler shift and doppler jump at BS switch, during the FR2 HST discussions RAN4 agreed to model the doppler jump when switching from RRH to RRH along the track (see 3GPP TR 38.101-4 [3], Annex B.3.4 for reference). In the case of ATG demodulation however, RAN4 has tentatively agreed [1] to consider ATG UE pre-compensation on DL frequency shift as baseline, so it is not straightforward to apply the conclusions from FR2 HST, which did not consider frequency pre-compensation.

If RAN4 agrees on considering ATG UE pre-compensation, it is our view that the large doppler shift experienced due to the high aircraft velocity should not be modeled as part of the channel during Demod performance testing, so the channel modeling should either consider no Doppler shift (assuming ideal UE frequency pre-compensation) or a relatively small Doppler shift (assuming non-ideal UE frequency pre-compensation).

Observation 3: For ATG Demodulation RAN4 has tentatively agreed to consider ATG UE pre-compensation for DL frequency shift, so the ATG UE under test is not expected to experience the high Doppler shift due to the large moving velocity assumed by the Deployment scenario;

Proposal 4: Consider ATG UE pre-compensation for UE demodulation requirements.

Regarding whether or not to consider ideal or non-ideal ATG UE frequency pre-compensation, we are open to considering submitting results including both cases, and then evaluate the final requirement

Proposal 5: RAN4 to consider both ideal and non-ideal ATG UE pre-compensation for the simulation assumptions. Decision on the final test setup will be based on the outcome of the simulation alignment.

Considering the assumptions on time and frequency modeling, it is unclear whether eventual RAN4 requirements with Bidirectional deployment scenario (see figure 1) would result in additional coverage with respect to the Unidirectional deployment already agreed.

Proposal 6: For ATG UE Demodulation requirements, assume the Deployment scenario agreed in RAN4 (unidirectional) and do not consider Bidirectional deployment.

- Conclusions

Observation 1: NTN Channel models defined in 3GPP TR 38.811 are not specifically designed to model ATG UE to ATG BS propagation;

Proposal 1: For ATG UE Demodulation, RAN4 to use Single Path channel without fading;

Observation 2: RAN4 did not consider propagation delay or propagation delay differences in the FR2 HST Channel model discussion (for bidirectional and unidirectional);

Proposal 3: RAN4 to follow the approach used for FR2 HST Demod channel modeling and not to consider propagation delay or delay jump for ATG UE Demodulation Channel Modeling;

Observation 3: For ATG Demodulation RAN4 has tentatively agreed to consider ATG UE pre-compensation for DL frequency shift, so the ATG UE under test is not expected to experience the high Doppler shift due to the large moving velocity assumed by the Deployment scenario;

Proposal 4: Consider ATG UE pre-compensation for UE demodulation requirements.

Proposal 5: RAN4 to consider both ideal and non-ideal ATG UE pre-compensation for the simulation assumptions. Decision on the final test setup will be based on the outcome of the simulation alignment.

Proposal 6: For ATG UE Demodulation requirements, assume the Deployment scenario agreed in RAN4 (unidirectional) and do not consider Bidirectional deployment.

- References

[1]  “R4-2302996 WF for ATG demodulation requirements”, 3GPP TSG-RAN WG4 Meeting # 106

[2] “Study on New Radio (NR) to support non-terrestrial networks (Release 15)”, 3GPP TR 38.811 V15.4.0 (2020-09);

[3] “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17)”, 3GPP TS 38.101-4 V17.7.0 (2022-12)