3GPP TSG-RAN WG1 Meeting #106bis-e	R4-2304545

Electronic meeting, 17th – 26th April 2023

Agenda Item:	5.20.2.2.2

Source:	Ericsson

Title:	TP to TR 38.858 section 10.5 Feasibility of FR2 BS aspects

Document for:	Approval

# 1	Introduction

During RAN4#106, a structure for the RAN4 part of TR 38.858 was agreed. Although several details remain open for the feasibility study and the co-existence simulations need to be carried out, nonetheless initial draft text can be provided for some sections.

Section editors were agreed for each section. It was also agreed that each company should submit text for each of the sections, and then that the editor would merge the texts.

In this TP, we provide some text for the section on FR2 feasibility.

# 3	Text Proposal

## 10.5 Feasibility of FR2 BS aspects

### 10.5.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability and analysis results.

| FR1 (or FR2-1) |  |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | 40 dBm TRP | 30 dBm TRP | 24 dBm TRP |
| BS TX Power dBPTX = ① dBm |  |  |  | 40 dBm | 30 dBm | 24 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used |  | Filtering, CFR |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 80 dBc | 80 dBc |
|  |  | Spatial isolation techniques used |  | A combination of spatial isolation, chokes, absorption, mushroom EBG. |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc | 10 dBc | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | TBC | TBC | TBC |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -78 dBm | -88 dBm | -94 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc | 0 dBc | 0 dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -50 dBm | -60 dBm | -66 dBc |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  |  |  |  |
|  |  | Frequency isolation techniques |  |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -35 dBm | -35 dBm | -35 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -80 dBm | - 110 dBm | -128 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -105dBm | Reciprocal phase noise mixing will add noise at around -110dBm |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -80 dBm | -104 dBm | -110 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dBc | 10 dBc | 10 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 119 dBc | 127dBc | 127 dBc |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm | -93 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 133 dBc | 123 dBc | 117 dBc |
| SBFD configuration |  |  |  | 75-50-75 (See Annex) |  |  |
| Guardband assumption (if exist) |  |  |  | 3 RB (See Annex) |  |  |
| bandwidth over which suppression is achieved |  |  |  | Several GHz |  |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

### 10.5.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

### 10.5.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the typical assumption of RF requirements and the analysis results

### 10.5.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.