3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304564

Online, April 17 – April 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Input to CA_n5-n28 for LBLB and LBLBLB band combinations WI

Agenda Item:	4.30.2.2	CA configuration of CA_n5-n28	[NR_700800900_combo_enh-Core]

Document for:	Approval

# Introduction

In RAN#99, the 7008009000 LBLBLB SI was completed and continued as a WI in order to capture the associated requirement in 38.101-1. In this contribution we provide our requirement proposals for CA_n5-n28 based on our analysis in the SI phase.

# Discussion

## WI objectives

In Last RAN#99 a new Work Item was approved in [1] as the continuation of the completed SI on LBLB and LBLBLB band combinations with the following objectives:

Specify the following requirements based on down-selection of options of the outcome of SI

- Define Tib/Rib for the band combinations.

- Define REFRENS degradation for some specific band combinations if it’s needed.

- Define other necessary requirements for some specific band combinations if they are identified to be needed.

For CA_n5-n8, study the feasibility of non-simultaneous n5 DL + n8 UL with the existing specifications [RAN2]

- Note: RAN2 work will be triggered by LS from RAN4

An overview table of the CA combinations studied in this WI is provided here:

Table 1: Band combinations studied in this WI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |

The channel bandwidths per operating band for each band combination are listed in the following table.

Table 2: Channel bandwidths per operating band for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n28A | CA_n5A-n28A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |

In this contribution, we provide our input on the necessary requirements to be captured in the 38.101-1 specification based on our evaluations performed during the related SI [3] and accounting for the agreed TPs in RAN4#106 [4] that were captured in the TR [2].

## Requirements for CA_n5-n28 1UL/2UL

### Baseline architecture

The baseline architecture should accommodate the support of the full band n28 range with a dual duplexer approach that is the implementation today. As already agreed in the study phase, the requirement should be derived from a two-antenna architecture. To accommodate this, we propose the following two alternatives for a two-antenna architecture to be used to derive the minimum requirements:

- Alternative 1:

  - UL/DL antenna 1: n28lowUL+n28lowDL+n5DL or n28lowUL+n28lowDL+n5DL triplexer to support full band n28 with a split filter approach

  - UL/DL antenna 1: n28fullDL+n5UL+n5UL triplexer.

- Alternative 2:

  - UL/DL antenna 1: n28lowUL+n28fullDL+n5UL+n5DL quadplexer

  - UL/DL antenna 1: n28highUL+n28fullDL+ n5DL triplexer.

- Delta T and Delta R and cross band isolation assumptions should enable full band n28 filters in the future

### Delta T and Delta R

In [4] there are two contribution on Delta T and Delta R values including ours from [3]. The main difference is that our values are targeting to enable two antenna or full band n28 filters while a tree-antenna proposal suggests no Delta T and Delta R. In our view, the requirement should be based on the worst case, lower cost and long-term solution that can be implemented in smaller size UEs. Thus, we re-iterate our proposal.

Proposal on CA_n5-n28 delta T and Delta R values: the values in the Table below are used for the requirement in 38.101-1 Table 6.2A.4.2.3-1 and 7.3A.3.2.1-1 respectively:

Table 2: CA_n5-n28 ΔTIB,c and ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 | 0.2 |
|  | n28 | 0.7 | 0.2 |

### One UL REFSENS exceptions due to cross-band interference

There were two contributions during the study phase that have been captured in [4] and includes our input from [3]. Since the MSD values are very close, we are fine to average the values from the two contributions.

Proposal on CA_n5-n28 1UL REFSENS exceptions: Based on the averaging the two contribution during SI phase including ours, the following MSD Table is used for the 38.101-1 requirement in Table 7.3A.6-1:

Table 3: REFSENS exceptions due to cross-band interference for CA_n5- n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 18.1 | ACLR2 |

### Two UL REFSENS exceptions due to combined cross band interference

Although there is no MSD due to 2UL IMDs in the CA_n5-n28 thanks to the distance between the two UL and the fact that the n28DL is between the two ULs, during the study phase it was shown that because the band n28DL is between n28 and n5UL with small gaps, n28 DL is subject to the combined cross-band interference of the two UL.

In the study phase there were four 2UL cross band MSD proposed:

- Our proposal is based on the combined interference of 20MHz n5 ACLR2 and 20MHz n28 ACLR2, in order to capture a balanced contribution from both bands. The proposed MSD value is on top of the band n28 20MHz REFSENS that already includes some de-sense

- One proposal basically relies on the already specified 1UL n5 crossband MSD in n28 due to n5 20MHz ACLR2 interference for a 5MHz n28DL

- The other two are aligned and use the maximum CBW for both bands: 20MHz n5 ACLR2 and 30MHz n28 ACLR1+ACLR2.

Since we also evaluated the last case in [3] we are fine to follow this path as our understanding is that the proposed MSD is on top of the band n28 30MHz REFSENS without self-de-sense. In that case the -9.9dB in table 5.2.3.3-1 in [4] would translate into a 11.9-9.9=2dB MSD which compares with the proposed 2.9dB and 4.5dB from the other two contributions. If agreeable to all companies, we are fine to average those 3 values.

We think it is important to acknowledge this new 2UL cross band issue in the specification, but also, we think this may need some explanation towards RAN5. The underlying assumptions are that:

- Both UL are at PCmax -3dB and maximum CBW in BCSs

- The MSD is on top of the 5MHz REFSENS scaled with NRB ratio to the largest CBW in the BCSs.

Proposal on CA_n5n28 2UL REFSENS exceptions:

- Based on the average of the two contribution and our data for the same test point in [3], the following MSD Table is used for the 38.101-1 requirement.

- A new Table is created in 38.101-1: 7.3A.6-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 2UL inter-band UL configuration

- The following notes are added at the bottom of the table or in the general text at the top of the section:

  - For Reference sensitivity exceptions due to combined 2UL cross band isolation, each UL is set PUmax-3dB like for the 2UL IMD tests

  - The MSD value is on top of the victim band 5MHz REFSENS scaled to the DL channel bandwidth in the test point.

Table 4: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 3.1 | ACLR2 from n5 UL band and ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

# Conclusions

In this contribution, we make our final proposal on requirements for the proposed LBLB and LBLBLB combinations and make the following proposal for CA_n5-n28 are:

Proposal on CA_n5-n28 delta T and Delta R values: the values in the Table below are used for the requirement in 38.101-1 Table 6.2A.4.2.3-1 and 7.3A.3.2.1-1 respectively:

Table 2: CA_n5-n28 ΔTIB,c and ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 | 0.2 |
|  | n28 | 0.7 | 0.2 |

Proposal on CA_n5-n28 1UL REFSENS exceptions: Based on the averaging the two contribution during SI phase including ours, the following MSD Table is used for the 38.101-1 requirement in Table 7.3A.6-1:

Table 3: REFSENS exceptions due to cross-band interference for CA_n5- n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 18.1 | ACLR2 |

Proposal on CA_n5n28 2UL REFSENS exceptions:

- Based on the average of the two contribution and our data for the same test point in [3], the following MSD Table is used for the 38.101-1 requirement.

- A new Table is created in 38.101-1: 7.3A.6-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 2UL inter-band UL configuration

- The following notes are added at the bottom of the table or in the general text at the top of the section:

  - For Reference sensitivity exceptions due to combined 2UL cross band isolation, each UL is set PUmax-3dB like for the 2UL IMD tests

  - The MSD value is on top of the victim band 5MHz REFSENS scaled to the DL channel bandwidth in the test point.

Table 4: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 3.1 | ACLR2 from n5 UL band and ACLR1+ACLR2 from n28 UL band |

# References

- RP-230764 New WID on enhancement for 700/800/900MHz band combinations for NR, CATT, China Telecom, RAN#99

- RP-230425 TR 38.872 v1.0.0 on Study on enhancement for 700/800/900MHz band combinations, CATT, RAN#99

- R4-2300657 2UL cross band MSDs for CA_n5-n28, Skyworks Solutions, Inc., RAN4#106

- R4-2303545	TP for TR 38.872 to capture MSD evaluation for CA_n5-n28, ZTE Corporation, Skyworks, Huawei, Qualcomm, Murata, Xiaomi