3GPP TSG-RAN4 Meeting #106-bis-e	R4-2304574

# Online, 17th – 26th April 2023

Source:			Ericsson

Title:			Spectrum less than 5 MHz - UE RF requirements

Agenda item:		5.15.3

Document for:		Approval

# Introduction

The Work Item to add NR support for dedicated spectrum less than 5 MHz for FR1 was revised ([1]) in RAN#99. This WI introduces new channel bandwidth(s) narrower than 5 MHz.

Based on the approved WF agreed in RAN4#96 meeting ([2]), this contribution discusses the remaining open issues related to UE RF requirements aspects.

# Discussion

## UE RF Tx requirements – remaining open issue

### Spurious – UE coexistence

In last RAN4#106 meeting, the agreed way forward ([2]) captured the following open issue:

| 6.5.3.2 Spurious UEtoUE co-ex | Option 2: To be further studied, checking if any RB restrictions was specified for 3 MHz channel BW for the considered LTE re-farmed bands. |
| --- | --- |

Looking at the bands listed in the WI, the LTE UE coexistence requirements were specified according to the following table (we only kept the relevant notes in this extract). Only bands 26, 28 and 85 were specified for LTE, and all three are supporting 3 MHz channel bandwidth.

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 54, 65, 66, 70, 71, 73,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 85 | E-UTRA Band 54 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

It should be noted that the restrictions are only for 5 and 10 MHz channel bandwidth and not for 3 MHz channel bandwidth.

As NR 3 MHz channel bandwidth and LTE 3 MHz channel  bandwidth signal would have the same characteristics (15 RBs and similar guard band), no update might not be required for NR as well but still further study would be needed to better understand if 3 MHz channel bandwidth might not have been overlooked.

Observation1: No restriction related to UE coexistence has been specified for LTE 3 MHz channel bandwidth in bands 26 and 28. Further study would still be needed to confirm this.

## UE RF Tx requirements – remaining open issue

### ACS

In last RAN4#106 meeting, the agreed way forward ([2]) captured the following open issue:

| 7.5 Adjacent channel selectivity | Specification impact --> 3 MHz is added into Table 7.5-1 Option 1: 33 dBc can be reused (Apple, Nokia) Option 3: Define 3 MHz ACS and IBB blocking signals similar to LTE (Ericsson, Qualcomm) |
| --- | --- |

To our understanding, both options are not contradictory as the LTE ACS requirement is also 33 dBc limit value.

Observation2: For ACS, 33dBc could be used and the requirement could be specified as done for LTE.

# Conclusion

In this contribution we discussed the impacts on the UE RF parameters in the scope of the new WI on NR support for dedicated spectrum less than 5 MHz. We made the following observations:

Observation1: No restriction related to UE coexistence has been specified for LTE 3 MHz channel bandwidth in bands 26 and 28. Further study would still be needed to confirm this.

Observation2: For ACS, 33dBc could be used and the requirement could be specified as done for LTE.

Our companion draft CR ([3]) captures all agreed changes.

# References

- RP-222645, WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell

- R4-2303655, WF for UE RF requirements for 3 MHz, Nokia

- R4-2304575, Draft CR to TS 38.101-1 - Introduction of 3 MHz channel bandwidth, Ericsson