3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304581

Online, April 17 – April 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Handling of new n8 and n71 CBW for PC2 FDD bands RSD and A-MPR

Agenda Item:	4.22.2	UE RF requirements	[HPUE_NR_FR1_FDD_R18]

Document for:	Approval

# Introduction

In RAN#99, new symmetrical UL/DL 25 and 30MHz CBW are requested for Band n71 and 20/25MHz UL/DL for n8 in [1]. Since these bands are also requested for PC2, the related CBW RSD and A-MPR should be evaluated for 1Tx and 2Tx PC2. In this contribution we suggest some options on how this could be handled.

# Discussion

In RAN#99 a new request was made for the support of symmetrical UL/DL 25 and 30MHz CBW for Band n71 in [1]. The related row of the request table is reproduced below.

| Band | Channel bandwidth and SCS | Justification | Additional information |
| --- | --- | --- | --- |
| n71 | 25 and 30 MHz for uplink for both BS and UE (15 kHz SCS for 25 MHz) | To fully utilize the licensed uplink bandwidth of some operators with n71, 25MHz and 30 MHz channel bandwidth are proposed to be added for the uplink. PC2 support impacts should also be considered in the scope of this request | Only symmetric UL/DL (25+25 and 30+30 MHz) |
| n8 | 25 MHz for both BS and UE (15 kHz SCS for 25 MHz) | To fully utilize the licensed bandwidth of some operators on n8, 25MHz channel bandwidth are proposed to be added. | DL up to 35MHz is optionally supported. UL requires verification |

The following paragraphs discuss the impacted requirement for PC3 and PC2 power classes.

## PC2 RSD needed to complete the 25MHz CBW for n8

### Handling of the work

If indeed the REFSENS value for PC3 was agreed and captured together with the asymmetric UL/DL cases, it does not necessarily cover all the requirements and notably the fact that PC2 is also requested for n8. Since the 20MHz UL is used, all the UL related requirements are already specified in 38.101-1 for PC3 and the current PC2 FDD work will cover A-MPR related aspects; thus, only the 20/25MHz UL/DL RSD evaluation for PC2 is needed.

Since the PC2 RSD and A-MPR for the other n8 CBW have already been evaluated and discussed within the PC2 FDD, we could consider that the PC2 aspect can be covered either in the “Adding channel BW support in existing NR bands” or in the “PC2 FDD” WI. We do not necessarily have a preference, but since we have also covered n71 new CBW in the “Adding channel BW support in existing NR bands” WI, we have submitted this contribution to the same AI.

Proposal on 25MHz CBW addition for n8: The 25MHz addition for n8 cannot be considered complete until the PC2 1Tx and 2Tx RSDs are agreed within the “Adding new channel BWs support to existing NR bands” WI, the WI. An input to RSD is provided in [2] in this meeting.

## Impact for n71

### How to handle PC3 and PC2?

On top of this symmetrical UL/DL 25 and 30MHz CBW support, band n71 is also requested for PC2 FDD introduction. The current 1Tx and 2Tx RSD evaluations provided in last RAN4 meeting. For example, in [4], it only covers the 20MHz UL restricted case for 25 and 30MHz. However, for the Release 18 specification completeness, the PC2 RSD for the symmetrical 25 and 30MHz UL/DL CBW must also be evaluated.

Proposal on symmetrical UL/DL 25 and 30MHz CBW addition for n71:

- For Release 18 completeness the PC2 1Tx and 2Tx RSDs should be specified for the optional symmetrical UL/DL 25 and 30MHz case.

- PC2 FDD should not be considered complete for n71 until PC2 FDD 1Tx and 2Tx RDS is specified for symmetrical UL/DL 25 and 30MHz CBW.

- Symmetrical 25 and 30MHz CBW for n71 should not be considered complete until PC2 FDD 1Tx and 2Tx RDS is specified for symmetrical UL/DL 25 and 30MHz CBW

- We suggest this work is covered in the “new CBW for existing band” as for n8. An input to RSD and A-MPR is provided in [3] in this meeting.

With the new 25MHz and 30MHz UL CBW, the Band n71 specific SEM requirement associated with NS_35, would need to be checked. Currently, this can be met with MPR thus there is no associated A-MPR. The specific SEM is different from the general SEM in that the measurement bandwidth in the first OOB MHz is 30kHz up to 100kHz OOB and then 100KHz MBW above while the general SEM uses a 1% MBW in the first OOB MHz.

Proposal on NS_35:

- NS_35 SEM requirement compliance with MPR for 25MHz and 30MHz needs to be verified for PC3 and PC2; and

- We suggest the PC2 work is covered in the “new CBW for existing band” AI.

# Conclusions

In this contribution, we discussed the impact of the introduction of 25 and 30MHz symmetrical CBW for n71 and the 20/25MHz UL/DL CBW for n8 and make the following proposals.

Proposal on 25MHz CBW addition for n8: The 25MHz addition for n8 cannot be considered complete until the PC2 1Tx and 2Tx RSDs are agreed within the “Adding new channel BWs support to existing NR bands” WI, the WI. An input to RSD is provided in [2] in this meeting.

Proposal on symmetrical UL/DL 25 and 30MHz CBW addition for n71:

- For Release 18 completeness the PC2 1Tx and 2Tx RSDs should be specified for the optional symmetrical UL/DL 25 and 30MHz case.

- PC2 FDD should not be considered complete for n71 until PC2 FDD 1Tx and 2Tx RDS is specified for symmetrical UL/DL 25 and 30MHz CBW.

- Symmetrical 25 and 30MHz CBW for n71 should not be considered complete until PC2 FDD 1Tx and 2Tx RDS is specified for symmetrical UL/DL 25 and 30MHz CBW

- We suggest this work is covered in the “new CBW for existing band” as for n8. An input to RSD and A-MPR is provided in [3] in this meeting.

Proposal on NS_35:

- NS_35 SEM requirement compliance with MPR for 25MHz and 30MHz needs to be verified for PC3 and PC2; and

- We suggest the PC2 work is covered in the “new CBW for existing band”.

# References

- RP-230756 Revised Basket WID: Core part: Adding new channel BWs support to existing NR bands, Ericsson, RAN#99

- R4-2304556 Additional input for 20-25MHz UL-DL CBW for n8, Skyworks Solutions Inc., RAN4#106be

- R4-2304559 Impact of 25 and 30MHz ULDL CBW for n71, Skyworks Solutions Inc., RAN4#106be

- R4-2300652 PC2 FDD bands RSD evaluation and NS requirement, Skyworks Solutions Inc., RAN4#106