3GPP TSG-RAN WG4 Meeting # 106-bis-e 	R4-2304591

E-meeting, April 17th – 26th, 2023

Agenda Item: 5.10.2.3

Source: MediaTek Inc.

Title: Discussion on case 2 requirements (NCSG and concurrent MG)

Document for: Discussion

# Introduction

In existing Rel-15/16 NR, two measurement gaps have been identified, which are per-UE and per-FR measurement gap. Later in Rel-17 NR, three measurement gap enhancement have been considered, which are: (i) pre-configured MG pattern(s) per configured BWP (fast MG configuration), (ii) multiple concurrent and independent MG patterns, and (iii) network controlled small gap (NCSG). Now in Rel-18, further work objective to enhance the existing measurement gap is agreed on, which is ‘Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG’, given in the work item description (WID) [1] as below:

- Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: Gaps that are configured for NTN are precluded in this WI - Note 2: The scenarios that NCSG is considered in Case 1 and that Pre-configured MG is considered in Case 2 are precluded in this WI. - Note 3: Prioritization among other possible combinations of pre-configured MG, concurrent MG and NCSG can be discussed after RAN#100

In the previous RAN4 106 meeting, the issues are captured in the way forward (WF) [2]. The analysis and discussion on the issues from the WF are provided in the next section.

# Discussion

From the WF, the issues are given in five categories, which are: (i) NCSG combination, (ii) collision definition and handling, (iii) other Rel-17 rules, and (iv) requirements.

## Discussion on NCSG combination

The open issues are given as:

## Sub-topic 4-1: Scope and combinations ### Issue 4-1-1: [Case 2] Whether to consider a new capability for NCSG + NCSG in an FR < Wayforward >: - FFS the options: - Option 1: No, without UE capability - Option 1a: E/// - Condition: No, if only one spare RF chain is assumed for NCSG+NCSG. - Option 1b: LGE - New UE capability for overlapping handling can be necessary if two spare RF chains are assumed for NCSG+NCSG. - Option 2: Yes, with UE capability - Option 2a: E/// - Condition: Yes, if two spare RF chains are assumed for NCSG+NCSG. ### Issue 4-1-3: [Case 2] Detail combinations for UE supporting per-FR gap < Agreement >: - Gap combination configuration of case 2 can use gap combination of concurrent gaps defined in TS38.133 table 9.1.8-1 as formatting baseline with the clarification that each configured gap can be NCSG or Type-2 MG. - FFS details and notes.

Issue 4-1-1: In the previous meeting RAN4#105, RAN4 agreed to consider the scenario of NCSG + NCSG in an FR. However, it was also mentioned during the discussion that the necessity to introduce this scenario is not high. This is because NCSG can be seen as a supplement to traditional MG and it is expected to have gap as common case. Therefore, RAN4 should support this with UE capability (i.e. it is left to the UE implementation). However, it is common in RAN4 to introduce a UE capability when there is a complex feature or requirements that normal UE cannot achieve, therefore, it is better for RAN4 to delay this discussion until the requirements for NCSG + NCSG are clear.

- RAN4 to delay the discussion on whether to support a UE capability for the scenario of NCSG + NCSG in an FR until a conclusion is achieved in parallel measurement issue.

Issue 4-1-3: In Rel-17 and Rel-18 concurrent measurement gaps requirements and agreements, RAN4 requirements for concurrent MGs are applied for two gap occasions occurring in the same FR. In addition, FR1 and FR2 use different RF chain and hence there is no collision between the two different FR gaps. Thus, per Rel-17 requirements, one FR can be configured with NCSG, and the other FR can be configured Pre-MG independent of whether the UE supports concurrent measurement gaps. Now in Rel-18, when two concurrent per-FR gaps are configured in one FR and a per-FR gap is configured in the other FR, the latter per-FR gap can be either Type-2 or NCSG independent of what are the two concurrent gaps in the first FR.

The gap combination principles can be captured as below:

- For UE capable of [Concurrent NCSG], one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured and the configuration in the other FR (This is Rel-18 new behaviour).

- For UE incapable of [Concurrent NCSG] but capable of NCSG, NCSG can only be configured if it is the single MG in that FR, but cannot be configured together with other MGs in the same FR, regardless they are per-UE or per-FR configured and the configuration in the other FR (This is Rel-17).

- Measurement gaps in FR1 and FR2 are independent from each other because they operate on two different RF chains.

- For UE capable of [Concurrent NCSG], one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured and the configuration in the other FR (This is Rel-18 new behaviour).

- For UE incapable of [Concurrent NCSG] but capable of NCSG, NCSG can only be configured if it is the single MG in that FR, but cannot be configured together with other MGs in the same FR, regardless they are per-UE or per-FR configured and the configuration in the other FR (This is Rel-17).

Therefore, for NCSG with concurrent MG, the gap combination can be divided into three cases:

- When two concurrent per-FR gaps are configured in an FR, and one more per-FR gap occasion is configured in the other FR, i.e. gap combinations 0 and 1 in Table 1: the single per-FR gap can be either Type-2 or NCSG independent of what are the two concurrent gaps in the first FR.

- When only 1 gap is configured in an FR (either per-UE and/or per-FR): for UE capable of Rel-18 NCSG concurrent gaps, up to 1 gap can be configured as NCSG. This is the case we introduced in Rel-17.

- When only 2 gaps are configured in an FR (either per-UE and/or per-FR): for UE capable of Rel-18 NCSG concurrent gaps, up to 2 gaps can be configured as NCSG, i.e. gap combinations 2 to 7 in Table 1.

- : The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns.

| Gap Combination Configuration Id | The number of simultaneous configured measurement gap patterns |  |  |
| --- | --- | --- | --- |
|  | Per-FR1 measurement gap | Per-FR2 measurement gap | Per-UE measurement gap |
| 0 | 2Note 2 | 1Note 4 | 0 |
| 1 | 1Note 4 | 2 Note 2 | 0 |
| 2 | 0 | 0 | 2 Note 2 |
| 3Note 1 | 1 Note 3 | 0 | 1 Note 3 |
| 4Note 1 | 0 | 1 Note 3 | 1 Note 3 |
| 5Note 1 | 1 Note 3 | 1 Note 3 | 1 Note 3 |
| 6 | 2 Note 2 | 0 | 0 |
| 7 | 0 | 2 Note 2 | 0 |
| Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. If UE is incapable of [Concurrent NCSG], all gaps can only be configured as Type-1/2 MG. Note 2: For UE capable of [Concurrent NCSG], up to 2 gaps can be configured as NCSG. Otherwise, both gaps can only be configured as Type-1/2 MG. Note 3: For UE capable of [Concurrent NCSG], this gap can be configured as NSCG. Otherwise, the gap can only be configured as Type-1/2 MG Note 4: For UE incapable of [Concurrent NCSG] but capable of NCSG, this gap can be configured as NCSG. |  |  |  |

- RAN4 shall define gap combination based on the following cases: (i) When two concurrent per-FR gaps are configured in an FR, and one more per-FR gap occasion is configured in the other FR: the single per-FR gap can be either Type-2 or Pre-MG or NCSG independent of what are the two concurrent gaps in the first FR, (ii) When 1 gap is configured per-UE and/or per-FR: for UE capable of Rel-18 NCSG concurrent gaps, up to 1 gap can be configured as NCSG, and (iii) When 2 gaps are configured per-UE or per-FR: for UE capable of Rel-18 NCSG concurrent gaps, up to 2 gaps can be configured as NCSG.

- RAN4 shall support the gap combination table given in Table 1 in this contribution paper.

## Discussion on collision handling and parallel measurements

The agreements and open issues are given as:

## Sub-topic 4-2: Collision handling ### Issue 4-2-1: [Case 2] Whether to consider equal priority < Agreement >: - No requirements are defined for equal priority in Pre-MG with concurrent gaps. ### Issue 4-2-2: [Case 2] If equal priority is considered then whether to consider gap sharing rule for equal priority < Agreement >: - Based on the outcome of issue 4-2-1, no further discussion is needed. ### Issue 4-2-3: [Case 2] Whether to consider parallel measurements upon gap collision < Wayforward>: - FFS the options: - Option 1: Yes - Option 2: No - Option 2a: No, when the RF chains for the two NCSG patterns are different. - Option 3: Up to UE capability, - For UE supporting this capability, both NCSGs can work when colliding. - For the UE not supporting this capability, R17 priority rules when colliding can be reused. - Option 4: RAN4 to study a general solution to allow both NW and UE to know the parallel measurements combination when UE supports NCSG parallel measurement capability. - Option 5: RAN4 to agree on investigating relevant scenarios with gap collision for the Case 2 scenario. ### Issue 4-2-4: [Case 2] Potential changes to UE behaviour upon gap collision < Wayforward >: - RAN4 to postpone the gap collision rule changes’ discussion until RAN4 has a consensus on parallel measurement scenarios. ### Issue 4-1-2: [Case 2] Whether the same RF chain is assumed for the two NCSG patterns < Wayforward >: - FFS the options: - Option 1: - Yes, the same RF chain is assumed for the two NCSG patterns. - Option 2: - No need to discuss whether the same RF chain is assumed for the two NCSG patterns (not necessary). - Option 2a: - No need to discuss whether the same RF chain is assumed but to consider the assumption on parallel measurement. - Option 3: - At least two spare RF chains are assumed. - Option 4: - UE signals its capability on number of receiver chains per band to the network. ### Issue 4-2-5: [Case 2] Whether to support parallel measurements in the following scenarios for two NCSG < Wayforward >: - FFS the following options Scenario 1: NW only configures deactivated SCells’ measurement - Option 1: - No. - Option 2: - Yes. Scenario 2: NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’ - Option 1: - No. - Option 2: - Yes. Scenario 3: NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands - Option 1: - No. - Option 2: - Yes.

Issue 4-2-3/4-2-4/4-1-2: In general, to achieve parallel measurements the UE needs two available RF chains. However, there is no guarantee that the UE will have two available RF chains. Hence, the UE cannot ensure parallel measurements. Therefore, there is no need for further enhancements on top of Rel-17 NCSG requirements. Besides, although the UE has a vacant RF chain for the NCSG in Rel-17, yet based on Rel-17 requirements, the UE is not required to measure two bands at the same time. This can be observed in the existing requirements for CSSF for NCSG, where CSSF can be 2 when the intra- and inter-frequency MOs are fully overlapping with SMTC. Furthermore, based on requirements for partial overlap between NCSG and SMTC for intra-frequency without MG, the UE is required to measure outside the NCSG with scaling factor kp is used to increase the measurement period.

- Based on Rel-17 requirements, the UE is not required to measure two bands at the same time. This can be observed in intra-frequency without gap requirements and in CSSF design for NCSG.

- RAN4 shall not define requirements for parallel measurement because the UE can never guarantee that there will be two available RF chain to achieve parallel measurements.

Issue 4-2-5: For scenarios 1 and 2: Although the UE can perform parallel transmission and reception for deactivated SCell’s measurement. However, UE has to dedicate one searcher for PCell, leaving only the other one searcher for all SCells. Hence, the UE can not achieve parallel measurements for two deactivated SCell.

- RAN4 shall not define requirements when NW only configures the deactivated SCells’ measurement objects associating with two NCSG patterns.

## Discussion on other Rel-17 rules to be revisited

The open issues are given as:

Issue 4-3-1: [Case 2] Potential changes for de-activated SCell < Wayforward >: - FFS the options - Background: When NW configures a NCSG and a Type-2 MG, the scenario for this deactivated SCell issue is as follow. - The deactivated SCell is measured within NCSG. - After MAC-based SCell activation, the deactivated SCell’s MO needs to be measured within MG if the related SSB is outside the active BWP

Issue 4-3-1: In Rel-17, a deactivated SCell is performed in NCSG. Then, after the SCell activation and the SSB is not in the active BWP the UE can still use NCSG. Yet, we understand if the suggestion is to allow the UE to report either NCSG or MG after the SCell activation. Thus, the motivation for restrict the UE to use MG after the SCell activation is not clear.

- RAN4 shall reuse the Rel-17 association rule, which is using NCSG for deactivated and to continue using NCSG after SCell is activated.

## Discussion on requirements

The open issues are given as:

## Sub-topic 4-4: Requirements ## Issue 4-4-1: [Case 2] Gap interruption < Wayforward >: - FFS: The interruption requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps can be defined as: Tint_ncsg+Tint_other-Toverlap, wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Toverlap is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length. < Agreement >: Update the existing gap interruption requirements for Case 2 as follows. - A slot is considered as interrupted if it is interrupted by an occasion of any of the configured concurrent measurement gaps following the measurement gap interruption requirements in clause 9.1.2, or by VIL occasion of any of the configured NCSG following the NCSG interruption requirements in clause 9.1.9.1, except for a dropped measurement gap or NCSG occasion.

Issue 4-4-1: On the first option, the aim for this option is not justified. Also, it is not clear whether the issue discussed in the first option is covered in collision rule already.

- RAN4 doesn’t need to further discuss the first option provided in issue 4-4-1 for NCSG discussion.

# Summary

In this contribution, discussion on NCSG and concurrent MG is provided, and we have the following proposals:

Proposal 1: RAN4 to delay the discussion on whether to support a UE capability for the scenario of NCSG + NCSG in an FR until a conclusion is achieved in parallel measurement issue.

Proposal 2: RAN4 shall define gap combination based on the following cases: (i) When two concurrent per-FR gaps are configured in an FR, and one more per-FR gap occasion is configured in the other FR: the single per-FR gap can be either Type-2 or Pre-MG or NCSG independent of what are the two concurrent gaps in the first FR, (ii) When 1 gap is configured per-UE and/or per-FR: for UE capable of Rel-18 NCSG concurrent gaps, up to 1 gap can be configured as NCSG, and (iii) When 2 gaps are configured per-UE or per-FR: for UE capable of Rel-18 NCSG concurrent gaps, up to 2 gaps can be configured as NCSG.

Proposal 3: RAN4 shall support the gap combination table given in Table 1 in this contribution paper.

Proposal 4: RAN4 shall not define requirements for parallel measurement because the UE can never guarantee that there will be two available RF chain to achieve parallel measurements.

Proposal 5: RAN4 shall not define requirements when NW only configures the deactivated SCells’ measurement objects associating with two NCSG patterns.

Proposal 6: RAN4 shall reuse the Rel-17 association rule, which is using NCSG for deactivated and to continue using NCSG after SCell is activated.

Proposal 7: RAN4 doesn’t need to further discuss the first option provided in issue 4-4-1 for NCSG discussion

# References

- RP-230755, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, March 2023, Rotterdam Netherlands.

- R4-2303197, “WF on Rel-18 NR MG enhancements”, MediaTek Inc, February-March 2023, Athens Greece.