3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304651

Online, April 17 – April 26, 202

Agenda Item:	6.9.4

Source: 	CMCC

Title: 			 Discussion on RRM core requirements for IOT NTN enhancement

Document for:	Discussion

- Introduction

In RAN4#106 meeting, the RRM core discussion for IOT NTN enhancement was initiated, the WF has been approved in [1]. Due to lack of input from other WGs, there are few agreements in last meeting.

In this contribution, we further discuss the RRM impaction of NR NTN enhancement based on the progress of RAN1 and RAN2.

- Discussion

Issue 2: Requirements which not applicable due to lack of neighbor cell ephemeris

- Conclusion: postpone discussion till the related open issues for NB-IoT/eMTC core & perf. requirements for NTN WI are resolved

Since RAN2 will support neighbour cell information signalling in R18, we think the requirements which not applicable due to lack of neighbour cell ephemeris in R17 could be applied in R18. i.e., the inter-frequency measurement of cell served by NGSO in RRC IDLE and CONNECTED mode. The requirements defined during R17 can be the starting point for further discussion. Revision can be based on the content of neighbour cell assistance information which will be defined by RAN2.

Proposal 1: The requirements which not applicable due to lack of neighbour cell ephemeris in R17 could be applied in R18, the requirements defined during R17 can be the starting point for further discussion.

Issue 3-1: Neighbour cell measurements in connected mode

- The mechanism for defining the neighbour cell measurements in CONNECTED mode for NB-IoT from Rel-17 WI can be reused as baseline for NB-IoT NTN while NTN specific impact needs investigation.

The NTN specific impact includes:

- Measurement capabilities on number of NGSO satellites.

We propose to reuse the measurement capabilities which defined in R17 as follows:

  - The minimum of the UE capability on the total number of the NGSO satellites across the layers is [2].

  - For NB in IDLE and M1 in both IDLE and CONNCTED, the UE shall be capable of monitoring

    - for intra-frequency carrier, the number of target satellites UE needs to monitor is [2] including serving LEO satellite.

    - for inter-frequency carrier, the number of target satellites UE needs to monitor per carrier is [2] if one of the target satellites include the UE serving satellite; the number of target satellites UE needs to monitor is [1] otherwise.

- A scaling factor considering the number of NGSO satellites to be measured.

Since the minimum of the UE capability on the total number of the NGSO satellites across the layers is [2], the delay requirements for NGSO measurement should be scaled up by Ksatellite, which is the number NGSO satellites to be measured.

- Measurement time occasion restriction due to GNSS measurement gap.

For the measurement occasion of inter-frequency detection and measurement, an additional condition should be fulfilled, which is the resources are not colliding with GNSS measurement gap.

Proposal 2: For NB neighbour cell measurement in connected mode, the impaction of NGSO satellites measurement capability should be considered, which is to scale up the delay requirements for NGSO measurement by Ksatellite, Ksatellite is the number NGSO satellites to be measured.

Proposal 3: For NB neighbour cell measurement in connected mode, the impaction of GNSS measurement gap should be considered. The measurement occasion should also fulfill the condition that the resources are not colliding with GNSS measurement gap.

Issue 3-2/3: Neighbour cell measurements in connected mode and triggering before RLF, for quasi-earth fixed cell and earth moving cell

Agreement

- It is up to RAN2 to decide the mechanism of neighbour cell measurements and corresponding measurement triggering before RLF for IoT NTN

- RAN4 to wait for more RAN2 progress on neighbour cell measurement triggered before losing coverage.

For NB-IOT NTN of quasi-earth fixed cell, RAN2 has achieved following conclusions [RAN2#119-121]:

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion) - At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage) - RAN2 will not specify the condition of stopping UE measurement before t-Service - Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not.

For NB-IOT NTN of earth-moving cell, RAN2 has achieved following conclusions [RAN2#119-121]:

- For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS) - For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation - Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris).

RAN2 has agreed to introduce time-based and location-based connected mode measurement initiation for quasi-earth-fixed cell and earth moving cell. In time-based measurement initiation criteria, the exact time of starting measurement can be left to UE implementation based on tentative agreements. In location-based measurement initiation criteria, the exact time of starting measurement will rely on serving cell reference location and a distance threshold/radius, while the specific triggering condition is still under RAN2’s discussion.

RAN4 should introduce time-based and location-based connected mode measurement initiation condition in core spec correspondingly. The details such as exact measurement starting time should wait further progress of RAN2.

Proposal 4: For NB-IOT NTN of quasi-earth fixed cell and earth-moving cell, RAN4 should introduce time-based and location-based connected mode measurement initiation condition in core spec according to RAN2’s design. The details such as exact measurement starting time should wait further progress of RAN2.

Issue 4: For eMTC, CHO requirements

Agreement

- Define RRM requirements for time and [location based] CHO based on further RAN2 progress.

- Define RRM requirements for time and [location based] CHO for both normal and enhanced overage for eMTC.

For eMTC NTN, RAN2 has achieved following conclusions [RAN2#119-121]:

- CHO enhancements for eMTC NTN (i.e. time/timer based solution) are introduced based on the R17 NR NTN solution. - CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.: UE is allowed to perform HO only during T1 to T2. - For eMTC over NTN, for both earth-moving and earth-fixed cell scenarios, we introduce location based CHO triggering events - For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR) - For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR). - FFS whether time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN.

Based on the current progress of RAN2, both time-based CHO and location-based CHO are introduced for eMTC NTN CHO.

Whether time and location-based trigger conditions may be configured independently is still under discussion in RAN2, which will impact the delay requirement in RAN4. Therefore, for the specific delay requirement development, we propose to wait more progress of RAN2.

Proposal 5: Define RRM requirements for time-based and location-based CHO for both normal and enhanced coverage for eMTC. For the specific delay requirement development, wait further progress of RAN2.

- Conclusion

Based on the discussion above, the following and proposals are concluded.

Proposal 1: The requirements which not applicable due to lack of neighbour cell ephemeris in R17 could be applied in R18, the requirements defined during R17 can be the starting point for further discussion.

Proposal 2: For NB neighbour cell measurement in connected mode, the impaction of NGSO satellites measurement capability should be considered, which is to scale up the delay requirements for NGSO measurement by Ksatellite, Ksatellite is the number NGSO satellites to be measured.

Proposal 3: NB For neighbour cell measurement in connected mode, the impaction of GNSS measurement gap should be considered. The measurement occasion should also fulfill the condition that the resources are not colliding with GNSS measurement gap.

Proposal 4: For NB-IOT NTN of quasi-earth fixed cell and earth-moving cell, RAN4 should introduce time-based and location-based connected mode measurement initiation condition in core spec according to RAN2’s design. The details such as exact measurement starting time should wait further progress of RAN2.

Proposal 5: Define RRM requirements for time-based and location-based CHO for both normal and enhanced coverage for eMTC. For the specific delay requirement development, wait further progress of RAN2.

- Reference

[1] R4-2303237, WF on RRM requirements for R18 IOT NTN enhancements, MediaTek.