# 3GPP TSG-RAN WG4 Meeting # 106bis-e                               R4-2304669

# Electronic Meeting, April 17 – April 26, 2023

# Title: 	Discussion on MU-MIMO receiver assumption

# Source: 	ZTE Corporation

# Agenda item:	5.19.1.1

# Document for:	Approval

# 1 Introduction

During the last RAN4 meeting, some conclusions has been reached for advanced receiver for MU-MIMO. In this contribution, we want to share further considerations on reference receiver assumptions .

- 2 Discussion

## 2.1 Reference receiver assumptions

Issue 1-2: Reference receiver assumption for R-ML - Option 1: UE perform RML algorithm for serving and all co-scheduled UEs in the cell - Option 2: UE perform RML algorithm for serving layer(s) + x interference layer(s) - Option 2A: x depends on UE’s capability of modulation order detection and perform E-IRC algorithm for rest interference layers - Option 3: UE can perform R-ML algorithms in the scenario with one additional co-scheduled UE (besides the UE under test) on all the interfering layers at each slot on the same frequency domain resource

At the previous meeting, some companies provided different reference receiver assumptions for R-ML. For above options, the difference between them is whether to perform R-ML algorithm for co-schedule UE or interfering layers. From what we understand, R-ML algorithm can decode the target information and co-schedule information when UE knows target UE and co-schedule UE channel state information. Therefore, if receiver support R-ML algorithm, UE can perform R-M to acquire desired information and don't need to perform other algorithms. On the other hand, from WID[2] it specified subjective is to study candidate receivers which include E-MMSE-IRC and R-ML not E-MMSE-IRC+R-ML. Based on our understanding, we would like to that UE perform RML algorithm for serving and all co-scheduled UEs in the cell.

Proposal 1. UE could perform R-ML algorithm for serving and all co-scheduled UEs in all layers.

- 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for MU-MIMO, The conclusions are:

Proposal 1. UE could perform R-ML algorithm for serving and all co-scheduled UEs in all layers.

# 4	Reference

[1]  R4-2302929 , WF on advanced receiver for MU-MIMO scenario, China Telecom.

[2]  RP-221288, WI: "NR demodulation performance evolution", China Telecom.