3GPP TSG-RAN WG4 Meeting # 106bis-e                               R4-2304800

Online, April 17 – April 26, 2023

Agenda item:	6.5.4

Source: 	ZTE Corporation

Title: 	Discussion on SAN RF requirements for the Extended L-band

Document for:	Approval

Introduction

In RAN#99 meeting, the work item [RP-230790] on introduction of the Extended L-band was approved. Part of the objective is as follows:

- Specify a new E-UTRA NTN FDD band, with initial scope for Region 1, with UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz

  - NOTE: The introduction of this new NTN band through this WID has no impact on existing 3GPP terrestrial band specifications.

- Support UE Categories NB1, NB2, M1;

- Support UE Power class 3 (+23dBm) and Power Class 5 (+20dBm);

- Introduce the corresponding SAN and UE RF core requirements;

- Introduce the corresponding RRM requirements.

- In this contribution, we want to share some initial views on SAN RF requirements for the Extended L-band.

Discussion

## 2.1. Operating bands

Based on the regulation of band number that the satellite bands are numbered in descending order from 256 and the following agreement approved in the last meeting, band number 253 should be applied to the Extended L-band.

Agreement: - n254 is reserved for the band defined for WI NR_NTN_LSband

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [253] | 1668 MHz | – | 1675 MHz | 1518 MHz | – | 1525 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

Proposal 1: The Extended L-band should be numbered as Table 2.1-1.

## 2.2 Channel bandwidth and transmission bandwidth configuration

The channel bandwidth and transmission bandwidth configuration of band 256 and 255 can be reused to the Extended L-band.

### 2.2.1 Channel bandwidth for category M1

The channel bandwidth and transmission bandwidth configuration for category M1 can be defined as Table 2.2-1.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

Proposal 2: For the Extended L-band, channel bandwidth and transmission bandwidth configuration for eMTC NTN operation should be defined as Table 2.2-1.

### 2.2.2 Channel bandwidth for category NB1 and NB2

The channel bandwidth and transmission bandwidth configuration for category NB1 and NB2 can be defined as Table 2.2-2.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Proposal 3: For the Extended L-band, channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should be defined as Table 2.2-2.

## 2.3 Channel raster, carrier frequency and EARFCN

The granularity of the global frequency raster and the relation between EARFCN and the carrier frequency of band 256/255 can be reused to the Extended L-band. Therefore, the channel raster, carrier frequency and EARFCN can be defined as Table 2.3-1.

Table 2.3-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [253] | 100 | 1518 | 228501 | 228501-<1>-228570 | 1668 | 261269 | 261269-<1>-261338 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Proposal 4: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.3-1.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: The Extended L-band should be numbered as Table 2.1-1.

Proposal 2: For the Extended L-band, channel bandwidth and transmission bandwidth configuration for eMTC NTN operation should be defined as Table 2.2-1.

Proposal 3: For the Extended L-band, channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should be defined as Table 2.2-2.

Proposal 4: For the Extended L-band, the channel raster, carrier frequency and EARFCN can be defined as Table 2.3-1.

References

- RP-230790, “New WID on Introduction of the Extended L-band (UL 1668-1675, DL 1518-1525) for IoT NTN”, Inmarsat