3GPP TSG-RAN WG4 Meeting # 106bis-e                               R4-2304804

Online, April 17 – April 26, 2023

Agenda item:	6.6.4

Source: 	ZTE Corporation

Title: 	Discussion on SAN RF requirements for FDD band (L+S band) for IoT NTN operation

Document for:	Approval

Introduction

In RAN#99 meeting, the work item [RP-230792] on FDD band (L+S band) for IoT NTN operation was approved. Part of the objective is as follows:

- Specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - 1610 – 1626.5 MHz in UL, and 2483.5 – 2500 MHz in DL

- Support UE power class 3 (+23dBm) and power class 5 (+20dBm)

- In this contribution, we want to share some initial views on SAN RF requirements for FDD band (L+S band) for IoT NTN operation.

Discussion

## 2.1. Operating bands

Based on the regulation of band number that the satellite bands are numbered in descending order from 256, band number 254 should be applied to the new FDD frequency bands for NB-IoT/eMTC NTN operation.

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 254 | 1610 MHz | – | 1626.5 MHz | 2483.5 MHz | – | 2500 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

Proposal 1: The new FDD frequency bands for NB-IoT/eMTC NTN operation should be numbered as Table 2.1-1.

## 2.2 Channel bandwidth and transmission bandwidth configuration

The channel bandwidth and transmission bandwidth configuration of band 256 and 255 can be reused to the new FDD frequency bands for NB-IoT/eMTC NTN operation.

### 2.2.1 Channel bandwidth for category M1

The channel bandwidth and transmission bandwidth configuration for category M1 can be defined as Table 2.2-1.

Table 2.2-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

Proposal 2: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, channel bandwidth and transmission bandwidth configuration for eMTC NTN operation should be defined as Table 2.2-1.

### 2.2.2 Channel bandwidth for category NB1 and NB2

The channel bandwidth and transmission bandwidth configuration for category NB1 and NB2 can be defined as Table 2.2-2.

Table 2.2-2: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Proposal 3: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should be defined as Table 2.2-2.

## 2.3 Channel raster, carrier frequency and EARFCN

The granularity of the global frequency raster and the relation between EARFCN and the carrier frequency of band 256/255 can be reused to the new FDD frequency bands for NB-IoT/eMTC NTN operation. Therefore, the channel raster, carrier frequency and EARFCN can be defined as Table 2.3-1.

Table 2.3-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 254 | 100 | 2483.5 | 228571 | 228571-<1>-228735 | 1610 | 261339 | 261339-<1>-261503 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Proposal 4: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the channel raster, carrier frequency and EARFCN can be defined as Table 2.3-1.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: The new FDD frequency bands for NB-IoT/eMTC NTN operation should be numbered as Table 2.1-1.

Proposal 2: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, channel bandwidth and transmission bandwidth configuration for eMTC NTN operation should be defined as Table 2.2-1.

Proposal 3: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, channel bandwidth and transmission bandwidth configuration for NB-IoT NTN operation should be defined as Table 2.2-2.

Proposal 4: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the channel raster, carrier frequency and EARFCN can be defined as Table 2.3-1.

References

- RP-230792, “New WID: FDD band (L+S band) for IoT NTN operation”, MediaTek Inc. Apple, Globalstar, Kepler Communications, Qualcomm, ZTE