3GPP TSG-RAN WG4 Meeting # 106bis-e													R4-2304835 Electronic Meeting, April 17-26, 2023

Source:	Ericsson, Nokia

Title:	MRTD and MTTD for UE architecture Type 4.

Agenda item:	5.12.3

Document for:	Approval

# 1	Introduction

In this contribution we discuss MRTD and MTTD requirements for UE architecture type 4.

# 2	Discussion

The different UE architectures discussed in this WI are listed in [1]:

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |

Table 1: UE architectures

In RAN4#105 Toulouse we have agreed MRTD and MTTD for EN-DC and NR-CA, for Type UE [2]:

… Issue 1-2-1: MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE - Agreements - MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE - MRTD=33us - MTTD=34.6us Issue 1-2-2: MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE - Agreements - MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE - MRTD=33us - MTTD=35.21us

The technical reason for the large MRTD and MTTD values of 33 µs and 34.6/35.21 µs is the fact that Type 2 UE does not share LNA. The drawback with the shared LNA type 3 UE is stated in [3]:

“…For the architecture Type 3a/3b, it is agreed that the each receiver branch will have an LNA shared by the two CCs. The power imbalance between the CCs could lead to signal distortion, especially on the weaker CC as the gain of the LNA will have to be aligned to the stronger carrier. Large MRTD will create issues in aligning the LNA gain switch timing. Since the gain switch can only be aligned to the slot boundary of one of the CCs, the gain change will happen during the useful symbol on the other CC distorting the received signal…”.

and

“…The UE should be able to handle a power imbalance larger than 6dB, however, a RTD > CP cannot be handled by the same LNA. In order to enable 4Rx on each CC, the actual RTD should be within the CP such that the LNA gain switching does not distort the useful part of the signal. If the switch happens in the useful part of the signal, the SNR on one of the carriers will be impacted. The performance degradation is rather difficult to assess as it depends on several factors such as actual RTD, which channels are affected (reference signals, control channel or data channel), the actual SNR of the affected signals, etc. This analysis will be very time consuming and results are likely to vary among companies…”.

The fact that Type 2 UE does not share LNA allow control of LNA switching gain of each carrier separately and we could agree on large MRTD and MTTD in RRM session (as well as large 25 dB Power Imbalance).

Observation 1: 	The technical reason for the large MRTD and MTTD values of 33 µs and 34.6/35.21 µs for UE Type 2 is the fact that UE Type 2 does not share LNA.

From Table 1 above, it is evident that also UE architecture 4 has separate, non-shared, LNA. The table entry is copied below [4]:

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | NRCA/ ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | ENDC | 25dB full range | Requires 6 antennas and LNA => FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | NRCA ENDC | 25dB full range | Requires 8 antennas and LNA => FWA only |

Observation 2: 	UE architecture Type 4 has separate, non-shared, LNA, just like UE Type 2.

The fact that UE architecture Type 4 has separate, non-shared, LNA means that we can apply the same technical reasoning when it comes to UE Type 4 as we did for UE Type 2 and tolerate the same large MRTD and MTTD values as UE architecture Type 2 (as well as large 25 dB Power Imbalance).

Observation 3: 	The UE architecture Type 4 with separate, non-shared, LNA tolerates the same MRTD and MTTD values as already agreed for Type 2.

Proposal 1:
MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 4 UE

- MRTD = 33 µs

- MTTD = 34.6 µs

MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 4 UE

- MRTD = 33 µs

- MTTD = 35.21 µs

# 3 	Summary

Observation 1: 	The technical reason for the large MRTD and MTTD values of 33 µs and 34.6/35.21 µs for UE Type 2 is the fact that UE Type 2 does not share LNA.

Observation 2: 	UE architecture Type 4 has separate, non-shared, LNA, just like UE Type 2.

Observation 3: 	The UE architecture Type 4 with separate, non-shared, LNA tolerates the same MRTD and MTTD values as already agreed for Type 2.

Proposal 1:
MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 4 UE

- MRTD = 33 µs

- MTTD = 34.6 µs

MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 4 UE

- MRTD = 33 µs

- MTTD = 35.21 µs

# References

[1] 		R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI, RAN4#104bis-e

[2] 		R4-2220398, WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA, Huawei, HiSilicon, Samsung, RAN4#105

[3] 		R4-2218770, Issues for Non-collocated Deployments with 4Layers per CC, Qualcomm

[4] 		R4-2303696, WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106