3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304892

Electronic Meeting, Apr 17- Apr 26, 2023

Agenda Item: 	5.14.4.5

Source:	Ericsson

Title:	Discussions on ATG measurement requirements

Document for:	Discussion

# 1	Introduction

In RAN4#106 meeting, RAN4 continued to discuss the measurement requirements impact for ATG. Some agreements were reached [1].

Issue 5-1-1: Measurement gap Agreement: - Reuse the legacy FR1 MG requirements (per UE gap and per FR1 gap) for R18 ATG. - Rel-16 MGs are considered as baseline. Whether other MG features are considered depends on further discussion. Issue 5-1-2: CSSF Agreement: - Consider inter-frequency measurement without GAP in R18 ATG. - For CSSF value - For the case of UE monitoring of multiple layers outside gaps: - For intra-frequency and inter-frequency measurement - Current requirements for CSSF in clause 9.1.5.1.2 are re-used for ATG. - For the case of UE monitoring of multiple layers within gaps: - Legacy definitions can be reused for ATG Issue 5-1-4: Intra-frequency measurements requirement Agreement: - Reuse the legacy NR intra-frequency measurements requirement with following updates: - The assumption that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 is not valid in ATG scenario. Issue 5-1-5: Inter-frequency measurements requirement Agreement: - Reuse the legacy NR inter-frequency measurements requirement as the starting point.

In this contribution, we will continue to discuss and provide our view on measurement related requirement.

# 2	Measurements

Measurement mechanism

One of the remaining issues for ATG is whether to consider UE assistant information to perform measurement. Generally, A2G UE is embedded in the aircraft. The A2G UE’s measurement behaviour shall start after the aircraft flies in the air. A2G UE shall determine the flight mode based on the info., such as the altitude. Thus, we support option 1 below.

Issue 5-1-3: Measurement mechanism - Option 1: - For ATG UE in ILDE/Inactive mode - If ATG UE altitude higher than certain threshold, ATG UE perform measurement - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement - For ATG UE in connected mode - If ATG UE altitude higher than certain threshold, ATG UE perform measurement and measurement report - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement and/or does not perform measurement report - Other Options are not precluded

Proposal 1. ATG UE shall determine the flight mode based on the flight data information as follows:

- If ATG UE altitude is higher than certain threshold, ATG UE performs measurement and reports the  measurement.

- If ATG UE altitude is lower than certain threshold, ATG UE shall not perform measurement and/or is not required to report the measurement.

- The altitude threshold can be indicated by NW.

Intra-frequency measurement

In last meeting, it was agreed to reuse the legacy intra-frequency measurement requirement for ATG intra-frequency measurement. RAN4 can further identify the impact due to ATG system.

Issue 5-1-4: Intra-frequency measurements requirement Agreement: - Reuse the legacy NR intra-frequency measurements requirement with following updates: - The assumption that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 is not valid in ATG scenario. Issue 5-1-6: Scheduling restriction - Option 1: The legacy scheduling restriction requirement can be reused for ATG. - Current scheduling restriction is depending on deriveSSB-IndexFromCell. However, there is no need to extend the scheduling restriction since it was agreed that legacy TN requirement can be reused for deriveSSB-IndexFromCell. This means it is up to NW to enable to disable deriveSSB-IndexFromCell, but in either case the scheduling restriction would be same as today. - When it is enabled, the restriction is on SSB symbols and 1 data symbol before and after. - When it is disabled, the restriction is on all symbols in SMTC. - Option 2: RAN4 to study the impact of larger propagation delay for scheduling restriction - Due to large time difference between the neighbour cells, the legacy scheduling restriction for 1 symbol before after SMTCs may not be suitable for ATG system. - Other Options are not precluded.

In last meeting, it was agreed that NW will decide whether to enable the deriveSSB-IndexFromCell. However, due to large cell in ATG deployment, the SSB timing offset will be larger than for normal cell deployment. In this case, it will be too conservative if NW always disable the deriveSSB-IndexFromCell. In other words, the performance will be degraded since the scheduling restriction will be applied to the whole SMTC. Thus, we propose to further introduce an adjust window which can be larger than 1 symbol. When NW enables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SSB symbols to be measured. NW can indicate the offset symbol value based on UE location and height information.

- Proposal 2. When NW enables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SSB symbols to be measured. The dedicated symbol value can be indicated by NW.

At the same time, we also noticed that the scheduling restriction also needs to be updated due to large cell impact even if deriveSSB-IndexFromCell is disabled. Considering the cell radius of 200km, the timing difference between the neighbour cell and the serving cell can be [-0.67ms, 0.67ms]. The maximum timing difference between two neighbour cells will be larger than 1ms. Especially, as we analyzed in our Tdoc[2], the real deployed cell radius may be larger than 200km. We think such timing difference will be even larger. Thus, the current 5ms SMTC window may be not enough. Thus, an additional offset is also needed before and after SMTC even if deriveSSB-IndexFromCell is disabled.

- Proposal 3. When NW disables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SMTC to be measured. The dedicated symbol value can be indicated by NW.

Pre-MG

One of the KPIs for ATG network is the throughput. If the MG can be dynamically disabled, it will help to improve the total system throughput. In Rel-17, Pre-MG is introduced mainly for dynamically switching between intra-frequency measurement with gap and intra-frequency measurement without gap. In ATG system, the ATG UE will spend a long time flying among the cell center area which means the inter-frequency measurement is unnecessary. At the same time, normally, the intra-frequency measurement for A2G won’t need MG since ATG UE will have a very large active BWP. In last meeting, some companies mentioned both NW and UE may know UE’s location information without further UAI. Thus, the Pre-MG can be disabled autonomously when UE enters into the cell center.

Therefore, we propose to consider also Pre-MG for ATG network, especially when the UE is moving in the cell center without inter-frequency measurement.

Issue 5-1-8: Pre-configured measurement gap - FFS on whether to introduce pre-configured measurement gap in ATG network

Proposal 4. Pre-MG shall be introduced in ATG network to improve the total system performance.

Proposal 5. Pre-MG can be activated/deactivated autonomously based on UE’s location information.

# Conclusion

In this contribution we have discussed and provided our view on the measurement issues identified in the way forward document at last meeting. Based on the discussions, we have made following proposals and observations:

Proposal 1. ATG UE shall determine the flight mode based on the flight data information as follows:

- If ATG UE altitude is higher than certain threshold, ATG UE performs measurement and reports the  measurement.

- If ATG UE altitude is lower than certain threshold, ATG UE shall not perform measurement and/or is not required to report the measurement.

- The altitude threshold can be indicated by NW.

Proposal 2. When NW enables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SSB symbols to be measured. The dedicated symbol value can be indicated by NW.

Proposal 3. When NW disables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SMTC to be measured. The dedicated symbol value can be indicated by NW.

Proposal 4. Pre-MG shall be introduced in ATG network to improve the total system performance.

Proposal 5. Pre-MG can be activated/deactivated autonomously based on UE’s location information.

# References

R4-2220361, "WF on NR ATG RRM core requirements", CMCC

R4-2301900, “On large TDD cells and other timing requirements”, Ericsson