3GPP TSG-RAN WG4 Meeting #106bis-e                                           	R4-2304901

Online, April 17 – April 26, 2023

Source:	Samsung

Title:	discussion and simulation results on 8Rx SDR requirements

Agenda item:	5.5.4.1.2

Document for:	Discussion

- Introduction

In last RAN4 #106 meeting, general test scope, simulation assumptions for FR1 8Rx SDR requirements have some agreements in WF [1]. In this contribution, we share our views on open issues of FR1 SDR requirements.

- Discussion

  - Upper bound MCS

Using the method mentioned in [2] section 2.1, we could calculate the upper bound MCS for 8 layers cases and 1024QAM cases based on following assumptions:

PDSCH Length (L) = 13

Number of additional DMRS = 1

Single or double symbol DM-RS: single for Rank <= 4; double for Rank > 4

Number of PDSCH DMRS CDM group(s) without data: 1 for 2 layers CCs; 2 for others

Number of PRBs in CORESET: 102 (40MHz)

MCS Index for maximum modulation format 2, 4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214; MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214; MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214.

The calculated upper bound MCS for 8 Rx as Table 2-1.

Table 2-1. Upper bound MCS for 8 Rx SDR requirements

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | upper bound MCS | requirement MCS |
| --- | --- | --- | --- | --- |
| 8 | 8 | 1 | 27 | TBD |
| 8 | 8 | 0.8 | 27 | TBD |
| 8 | 8 | 0.75 | 26 | TBD |
| 8 | 8 | 0.4 | 15 | TBD |
| 8 | 6 | 1 | 28 | TBD |
| 8 | 6 | 0.8 | 28 | TBD |
| 8 | 6 | 0.75 | 27 | TBD |
| 8 | 6 | 0.4 | 18 | TBD |
| 8 | 4 | 1 | 16 | TBD |
| 8 | 4 | 0.8 | 16 | TBD |
| 8 | 4 | 0.75 | 16 | TBD |
| 8 | 4 | 0.4 | 13 | TBD |
| 8 | 2 | 1 | 9 | TBD |
| 8 | 2 | 0.8 | 9 | TBD |
| 8 | 2 | 0.75 | 9 | TBD |
| 8 | 2 | 0.4 | 6 | TBD |
| 4 | 10 | 1 | 26 | TBD |
| 4 | 10 | 0.8 | 23 | TBD |
| 4 | 10 | 0.75 | 22 | TBD |
| 4 | 10 | 0.4 | 10 | TBD |
| 2 | 10 | 1 | 26 | TBD |
| 2 | 10 | 0.8 | 23 | TBD |
| 2 | 10 | 0.75 | 22 | TBD |
| 2 | 10 | 0.4 | 10 | TBD |

  - Simulation assumptions

Agreements for simulation assumptions in WF [1] is as below Table2-2. Simulations in this document are based on this simulation assumptions.

Table 2-2: Simulation assumptions

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 for Rank <= 4 2 for Rank > 4 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1005} for 6 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2, 4 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |

  - Feasibility of 64QAM

About the 8Rx SDR feasibility of 64QAM, below agreement is achieved in RAN4#106.

Issue 3-2: Feasibility of 64QAM - It is feasible to define SDR test for 8 MIMO layer for 64QAM - Companies can bring the simulation results in next meeting for the maximum achievable MCS

Simulation results for 64QAM 8 MIMO layers SDR test are as below. Considering the maximum achievable SNR for 64QAM is 24.4dB, and the RF impairment margin, it is more suitable to define MCS26 as the maximum achievable MCS for 64QAM 8 MIMO layers.

Proposal 1: define MCS26 (scaling factor =1) as the maximum achievable MCS for 64QAM, 8 layers, 8Rx.

  - Feasibility of 256QAM

About the 8Rx SDR feasibility of 256QAM, below agreement is achieved in RAN4#106.

Issue 3-3: Feasibility of 256QAM - Evaluate SDR test cases for 2, 4 and 8 MIMO layers for 8Rx; FFS 6 MIMO layers - 2 and 4 MIMO layers: Use the existing MCS value defined in Table 5.5A-5 of TS 38.101-4 - 8 MIMO layers: FFS the max achievable MCS

Simulation results for 256QAM 8 MIMO layers SDR test are as below. Considering the maximum achievable SNR for 64QAM is 30.4dB, and the RF impairment margin, it is more suitable to define MCS25 as the maximum achievable MCS for 256QAM 8 MIMO layers.

Proposal 2: define MCS25 (scaling factor =1) as the maximum achievable MCS for 256QAM, 8 layers, 8Rx.

For 2 and 4 MIMO layers of 256QAM 8Rx cases, the SNR working points @85% maximum throughput are as below.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS | SNR working points @85% maximum throughput |
| --- | --- | --- | --- | --- |
| 2 | 8 | 1 | 26 | 19.7 |
| 2 | 8 | 0.8 | 21 | 13.71 |
| 2 | 8 | 0.75 | 20 | 13.46 |
| 2 | 8 | 0.4 | 11 | 4.33 |
| 4 | 8 | 1 | 26 | 22.8 |
| 4 | 8 | 0.8 | 23 | 19.71 |
| 4 | 8 | 0.75 | 22 | 18.71 |
| 4 | 8 | 0.4 | 12 | 8.71 |

Thus it is acceptable to use the 2 and 4 layer requirements in Table 5.5A-5 of TS38.101-4 for 2 and 4 layer of 256QAM 8Rx SDR requirements.

Proposal 3: for 256QAM 2 and 4 layer of 8Rx SDR requirements, reuse the 256QAM 2 and 4 layer SDR requirements in Table 5.5A-5 of TS38.101-4.

  - Feasibility of 1024QAM

About the 8Rx SDR feasibility of 1024QAM, below agreement is achieved in RAN4#106.

Issue 3-4: Feasibility of 1024QAM - FFS whether it is feasible to consider 1024QAM - If it is feasible to introduce requirements for 1024QAM - FFS the maximum achievable MIMO layers: Rank 2 or Rank 4 - FFS the maximum achievable MCS

Simulation results for 1024QAM 2 and 4 MIMO layers SDR test are as below. Considering the maximum achievable SNR for 1024QAM is 32dB, and the RF impairment margin, it is more suitable to define MCS24 as the maximum achievable MCS for 1024QAM 2 MIMO layers, and define MCS23 as the maximum achievable MCS for 1024QAM 4 MIMO layers.

Proposal 4: propose the maximum achievable MIMO layers is layer 4 for 1024QAM 8Rx.

Proposal 5: define MCS24 (scaling factor =1) as the maximum achievable MCS for 1024QAM, 2 layers, 8Rx.

Proposal 6: define MCS23 (scaling factor =1) as the maximum achievable MCS for 1024QAM, 4 layers, 8Rx.

  - MCS indexes for 8 MIMO layers

According the simulation results in section 2.3 and 2.4, and considering the theoretical upper bound MCS in table 2-1 and Table 5.5A-5 in 38.101-4, we propose below MCS indexes for 8 MIMO layers.

Proposal 7: define MCS indexes for 8 MIMO layers as

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 25 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

  - MCS indexes for 1024QAM 8Rx

According the simulation results in section 2.5, and considering the theoretical upper bound MCS in table 2-1 and Table 5.5A-6 in 38.101-4, we propose below MCS indexes for 8 MIMO layers.

Proposal 8: define MCS indexes for 1024QAM 8Rx as

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 22 |
|  | 2 | 10 | 0.75 | 20 |
|  | 2 | 10 | 0.4 | 10 |
|  | 4 | 10 | 1 | 23 |
|  | 4 | 10 | 0.8 | 21 |
|  | 4 | 10 | 0.75 | 19 |
|  | 4 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

- Conclusion

In this contribution, we provide analysis and views on 8Rx SDR requirements for remaining open issues.

Proposal 1: define MCS26 (scaling factor =1) as the maximum achievable MCS for 64QAM, 8 layers, 8Rx.

Proposal 2: define MCS25 (scaling factor =1) as the maximum achievable MCS for 256QAM, 8 layers, 8Rx.

Proposal 3: for 256QAM 2 and 4 layer of 8Rx SDR requirements, reuse the 256QAM 2 and 4 layer SDR requirements in Table 5.5A-5 of TS38.101-4.

Proposal 4: propose the maximum achievable MIMO layers is layer 4 for 1024QAM 8Rx.

Proposal 5: define MCS24 (scaling factor =1) as the maximum achievable MCS for 1024QAM, 2 layers, 8Rx.

Proposal 6: define MCS23 (scaling factor =1) as the maximum achievable MCS for 1024QAM, 4 layers, 8Rx.

Proposal 7: define MCS indexes for 8 MIMO layers as

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 25 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Proposal 8: define MCS indexes for 1024QAM 8Rx as

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 22 |
|  | 2 | 10 | 0.75 | 20 |
|  | 2 | 10 | 0.4 | 10 |
|  | 4 | 10 | 1 | 23 |
|  | 4 | 10 | 0.8 | 21 |
|  | 4 | 10 | 0.75 | 19 |
|  | 4 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

- Reference

[1] R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, 3GPP RAN4 #106 meeting

[2] R4-1812165, “NR SDR requirements”, Intel Corporation, 3GPP RAN4 #88bis meeting

- Appendix

SDR related simulation results

For 2 and 4 MIMO layers of 256QAM 8Rx cases, the simulation results are as below.