3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304910

# 2023 Online, April 17 – April 26, 2023

Source:	LG Electronics

Title:	Discussion on enhanced cell reselection requirements in NTN

Agenda item:	7.1.2

Document for:	Discussion

Introduction

In the last RAN2#121 meeting, there is a following incoming LS [1].

1. Overall Description: RAN2 would like to thank RAN4 for their LS on enhanced cell reselection requirements in NTN. RAN2 has discussed how the feature is defined in TS 38.306 and decided to agree a corresponding Change Request in R2-2300470 (attached). This CR corrects the description of the behavior applied when the NTN UE does not support enhanced RRM requirements for measurements in RRC IDLE/RRC_INACTIVE. RAN2 would like to point out that section 4.2C.2 in TS 38.133 describes only IDLE mode, so it needs to be clarified if the same requirements apply to INACTIVE as well. In addition, RAN2 believes the changes to RAN4 feature list (i.e. in 25-4) are needed to align with the agreed CR (R2-2300470). 2. Actions: To RAN WG4 group. ACTION: RAN2 respectfully asks RAN4 to check the recently agreed CR (in R2-2300470) and let RAN2 know if the feature definition for Enhanced RRM requirements for measurements in IDLE and INACTIVE still contains erroneous parts. In particular, please inform if INACTIVE requires separate description. RAN2 also asks RAN4 to pursue corresponding changes in RAN4 feature list.

In this contribution, we provide our view on inactive mode requirements in NTN and Reply LS for [1].

Discussion

We think that the IDLE mode requirements can apply to INACTIVE mode requirements as like other IDLE/Inactive mode requirements in TS 38.133. But, as pointed out by RAN2, INACTIVE mode requirements are not defined yet in TS 38.133 i.e., there is no clause 5.1C Cell Re-selection for NR UE for Satellite Access. So, we think before replying the LS R2-2301966 from RAN2, the CR for inactive mode requirements (e.g., 5.1C in TS 38.133) need to be agreed first. However, in this meeting RAN4 106-bis-e, since there is no maintenance agenda, the CR for inactive mode will be approved in next RAN4#107 meeting.

- Proposal 1: The IDLE mode requirements can apply to INACTIVE mode requirements in NTN. But, the reply LS can be sent in the next RAN4 #107 meeting after CR for inactive mode requirements (e.g., 5.1C in TS 38.133) is agreed.

Conclusion

In this contribution, we provide a proposal 1.

- Proposal 1: The IDLE mode requirements can apply to INACTIVE mode requirements in NTN. But, the reply LS can be sent in the next RAN4 #107 meeting after CR for inactive mode requirements (e.g., 5.1C in TS 38.133) is agreed.

Reference

[1] R2-2301966, “On enhanced cell reselection requirements in NTN,” TSG RAN WG2