3GPP TSG-RAN WG4 # 106bis-e	R4-2304914

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.5.4.1.1

Source:	MediaTek Inc.

Title:	Discussion on PDSCH requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#106 meeting, a WF related to 8Rx UE PDSCH requirements was agreed [1]. In this contribution, we provide simulation results based on [1] and discuss outstanding issues.

- Discussion

Based on the WF in RAN4#106, several MCS are considered to define the PDSCH requirements for 8Rx UE.

- The following MCS can be used starting point; Rank 2: - up to 256QAM: [MCS26] (table 2); (Low correlation) (Only forTDLA30-10) - MCS16 (Medium B); - MCS19(Medium B) - MCS23(Low) Rank 4: - up to 64QAM: MCS26; - MCS17; Rank 8: - up to 64QAM: MCS20; - MCS13;

The simulation results in Table 1 are based on the assumptions from Table 2 in Appendix. It is noted the MCS in Table 1 are all from MCS Table 1 except MCS26 for Rank 2.

Table 1. SNR point of 70% maximum throughput for rank 2, 4, 8

| Rank | MCS | Channel Correlation | Propagation Condition | SNR (dB) |
| --- | --- | --- | --- | --- |
| 2 | 26 (256 QAM, MCS Table 2) | ULA Low | TDLA 30-10 | 22.8 |
|  | 23 (64 QAM) |  |  | 11.6 |
|  | 19 (64 QAM) | ULA Medium B |  | 9.6 |
|  | 16 (16 QAM) |  |  | 7.3 |
|  | 23 (64 QAM) | ULA Low | TDLC 300-100 | 12.5 |
|  | 19 (64 QAM) | ULA Medium B |  | 10.1 |
|  | 16 (16 QAM) |  |  | 8.7 |
| 4 | 26 (64 QAM) | ULA Low | TDLA 30-10 | 20.9 |
|  | 17 (64 QAM) |  |  | 10.3 |
| 8 | 20 (64 QAM) | ULA Low | TDLA 30-10 | 25.3 |
|  | 13 (16 QAM) |  |  | 14.8 |

From simulation results, it can be shown that the SNR values to achieve 70% of maximum throughput for considered MCS are within a reasonable range. However, the required SNR for the case of rank 2 (MCS26), rank4 (MCS26) and rank8 (MCS20) are very high, which is over 20dB. To reduce test effort, we propose to pick one MCS for each rank configuration to define requirements for 8Rx UE.

Proposal 1: Introduce PDSCH requirements for 8Rx UE with the following MCS:

- MCS19 (rank2, ULA Medium B, TDLC 300-100)

- MCS17 (rank4, ULA low, TDLA 30-10)

- MCS13 (rank8, ULA low, TDLA 30-10)

- Conclusion

In this contribution, we provide simulation results for different MCS based on agreed simulation assumptions in the last meeting. According to simulation results, we have following proposals for MCS selection to define 8Rx UE PDSCH requirements.

Proposal 1: Introduce PDSCH requirements for 8Rx UE with the following MCS:

- MCS19 (rank2, ULA Medium B, TDLC 300-100)

- MCS17 (rank4, ULA low, TDLA 30-10)

- MCS13 (rank8, ULA low, TDLA 30-10)

- Reference

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon

- Appendix

Table 1. Simulation assumptions for PDSCH requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U (FR1.30-1) |
| SCS(kHz)/Bandwidth (MHz) |  |  | 30/40 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank 2 and 4 (@symbols#2,11) 2 for rank 8 (@symbols#2, 3, 10, 11) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | for rank 2 and 4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 for rank 8 l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  | CSI-RS periodicity | Slots | 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4. |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Row index |  | 3 for rank 2 5 for rank 4 6 for rank 8 |
|  | First subcarrier index in the PRB used for CSI-RS |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  | Same as number of transmit antenna |
|  | CDM Type |  | 'FD-CDM2' |
|  | Density (ρ) |  | 1 |
|  | CSI-RS periodicity | Slots | 40 |
|  | CSI-RS offset |  | 0 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |