3GPP TSG-RAN WG4 # 106bis-e	R4-2304916

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.5.4.1.3

Source:	MediaTek Inc.

Title:	Discussion on CQI requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#106 meeting, a WF related to 8Rx UE CQI requirements was agreed [1]. In this contribution, we provide simulation results based on [1] and discuss about the codebook restriction for 8Rx CQI requirements.

- Discussion

CodebookSubsetRestriction

For the legacy RAN4 CQI requirements, at most 2Tx is considered and 6 bits CodebookSubsetRestriction is used to restrict UE for reporting some specific PMI. For the newly introduced 8Rx UE CQI requirements, RAN4 agreed to use 4Tx. Therefore, according to TS38.214, both i1 and i2 need to be indicated for UE to restrict PMI reporting and RAN4 achieved the below agreements in RAN4#106.

- Antenna configuration: XP 4T8R - (N1,N2)=(2,1); (O1,O2)=(4,1) - ReportQuantity: CRI/Ri/i1/CQI - Two-one-TypeI-SinglePanel-Restriction: 0b00000001 - TypeI-SinglePanel-codebookSubsetRestriction-i2: 0x0000000000000001

However, according to TS38.306, UE is not mandatory to support the report quantity 'CRI/RI/i1/CQI '.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| semiOpenLoopCSI Indicates whether UE supports CSI reporting with report quantity set to 'CRI/RI/i1/CQI ' as defined in clause 5.2.1.4 of TS 38.214 [12]. | UE | No | No | Yes |

Observation 1: UE is not mandatory to support the report quantity 'CRI/RI/i1/CQI '.

Proposal 1: If RAN4 agreed to define 8Rx UE CQI requirements with report quantity 'CRI/RI/i1/CQI '. It is necessary to introduce related UE capability for 8Rx UE CQI requirements.

Simulation results

Our simulation result in Figure 1 is based on parameters in Table 1.

Table 1. Simulation assumptions for 8Rx UE CQI requirement

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-i1/CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0, i1.3=0 and i2=0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

Figure 1. Simulation results for median CQI

According to results in Figure 1, RAN4 can set lower SNR [4,5] dB for 16QAM and higher SNR [11,12] dB for 64QAM for 8Rx CQI testing.

Observation 2: If it is agreed to define 8Rx UE CQI requirements with report quantity 'CRI/RI/i1/CQI ', RAN4 can set lower SNR [4,5] dB for 16QAM and higher SNR [10,11] dB for 64QAM.

- Conclusion

In this contribution, we provide simulation results based on [1] and discuss about the codebook restriction for 8Rx CQI requirements.

Observation 1: UE is not mandatory to support the report quantity 'CRI/RI/i1/CQI '.

Proposal 1: If RAN4 agreed to define 8Rx UE CQI requirements with report quantity 'CRI/RI/i1/CQI '. It is necessary to introduce related UE capability for 8Rx UE CQI requirements.

Observation 2: If it is agreed to define 8Rx UE CQI requirements with report quantity 'CRI/RI/i1/CQI ', RAN4 can set lower SNR [4,5] dB for 16QAM and higher SNR [10,11] dB for 64QAM.

- Reference

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon