3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304917

# Electronic Meeting, April 17 – April 26, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | draft | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS36.102, PDSCH requirements for NB-IoT over NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Demodulation performance requirements for NB-IoT over NTN needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The demodulation performance requirements for NB-IoT over NTN are introduced in sections 8 and the propogation channels are defined in Annex E. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation performance requirements for NB-IoT over NTN are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Add Sections 8.3 and Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE 1

- 8.3	Demodulation performance requirements for UE category NB1 and NB2

8.3.1   Half-duplex FDD

8.3.1.1	NPDSCH demodulation requirements

The parameters specified in Table 8.3.1.1-1 and Table 8.3.1.1-2 are valid for all half-duplex FDD tests unless otherwise stated.

Table 8.3.1.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes per component carrier | Processes | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | N/A |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB |  | OCNG |
| OCNG pattern (Note 1) |  | NB.OP.1 |
| Note 1: The OCNG pattern is defined in Annex A.5.3 of TS36.101 [7]. |  |  |

Table 8.3.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH (Note 1) |  | R.NB.3 FDD |
| (npdcch-Offset-USS-r13) |  | 0 |
| K_offset | Subframes | 8 |
| Note 1: The Reference channel for NPDCCH is defined in Table A.3.1.1-1T of TS36.101 [7]. |  |  |

8.3.1.1.1	 Minimum Requirements for Standalone

The requirements are specified in Table 8.3.1.1.1-1, with the addition of the parameters in Table 8.3.1.1.1-1 and the downlink physical channel setup according to Annex C. The purpose of these tests is to verify the performance.

Table 8.3.1.1.1-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 512 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | [TBD] | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.6-1 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | [TBD] | NB1, NB2 |

END OF CHANGE 1

START OF CHANGE 2

Annex E (normative):
Propagation conditions

# E.1	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

## E.1.1	Delay profiles

The delay profiles are derived from the TR 38.811 [x] NTN-TDL models for the desired delay spread and tap resolution. After scaling the normalized delay spread values for each tap by the desired RMS delay spread, the tap delays are quantized to a delay resolution of 5ns by rounding to the nearest multiple of the delay resolution.

Table E.1.1-1: Delay profiles for IoT NTN channel models

| Type | Model | Delay spread (r.m.s.) | Delay resolution |
| --- | --- | --- | --- |
| NLOS | NTN-TDLA100 | 100 ns | 5 ns |
| LOS | NTN-TDLC5 | 5 ns | 5 ns |

Table E.1.1-2: NTN-TDLA100 (DS = 100 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Table E.1.1-3 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

## E.1.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler>, or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table E.1.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for NLOS and LOS propagation conditions.

Table E.1.2-1: Channel model parameters for NTN

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-10 | NTN-TDLA100 | 10 Hz |
| NTN-TDLA100-200 | NTN-TDLA100 | 200 Hz |
| NTN-TDLC5-30 | NTN-TDLC5 | 30 Hz |
| NTN-TDLC5-200 | NTN-TDLC5 | 200 Hz |

END OF CHANGE 2