3GPP TSG-RAN WG4 Meeting # 106bis	R4-2304932

Electronic Meeting, April 17 – April 26, 2023

Agenda Item: 5.33.2

Source: MediaTek Inc.

Title: Discussion on RRM impacts for R18 NR sidelink relay enhancements

Document for: Discussion

- Introduction

The discussion on R18 NR sidelink relay enhancements [1] in RAN4 started last meeting. This contribution provides further analysis on the requirements to define from RRM perspective.

- Discussion

According to WID [1], RAN4 is supposed to “Specify RRM core requirements for relay discovery and (re)selection in UE-to-UE relay”. To our knowledge, relay discovery and (re)selection procedure in U2U (UE-to-UE) relay is basically the same as U2N (UE-to-Network) relay. So the legacy requirements of selection / reselection of relay UE in U2N relay can be used as a baseline for U2U relay. As some details are still in discussion in RAN2, we can wait for RAN2 outcome and further discuss the impact on RRM requirements.

Proposal 1: Use the legacy requirements of selection/reselection of relay UE in U2N relay as a baseline for U2U relay.

Proposal 2: Wait for RAN2 outcome and further discuss the impact on RRM requirements.

In last meeting, there were also some discussions on whether to define (re)selection requirements of relay UE and interruption requirements caused by the source/remote UE on the direct path when the source/remote UE performs the selection/reselection of the indirect path for multi-path case. In our understanding, RAN4 should focus on UE-to-UE relay as required in WID (Specify RRM core requirements for relay discovery and (re)selection in UE-to-UE relay) [1].

Proposal 3: Not define requirements for multi-path relay scenario.

- Summary

In this paper, we have some discussion on R18 sidelink relay enhancements. We have the following proposals:

Proposal 1: Use the legacy requirements of selection/reselection of relay UE in U2N relay as a baseline for U2U relay.

Proposal 2: Wait for RAN2 outcome and further discuss the impact on RRM requirements.

Proposal 3: Not define requirements for multi-path relay scenario.

4 Reference

- RP-223501, “Revised WID on NR sidelink relay enhancements”, LG Electronics, RANP#98 e-meeting, Dec. 2022.