3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304954

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE Rx requirements for sidelink in unlicensed spectrum

Agenda item:	5.31.2.1.3

Document for:	Approval

Introduction

The updated WID [1] of NR sidelink evolution from 3GPP TSG RAN Meeting #99-e includes an objective related to the support of operation of sidelink in unlicensed spectrum and also has an objective related to support of new introduced features (e.g. SL unlicensed) and frequency bands. Relevant objectives are copied below.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel. - UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4] - The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI. - The frequency band combinations is limited to the ITS-dedicated spectrum only (Band n47) for contiguous carrier aggregation. - Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements. - Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

In this paper we discuss potential RAN4 impact on UE Rx requirements for SL operating in unlicensed spectrum.

Discussion

Currently, sidelink requirements in RAN4 RF specifications are defined at least on the following specific sections for V2X communications:

- 5.2E	Operating bands for V2X

- 5.3E	Channel bandwidth for V2X

- 5.4E	Channel arrangement for V2X

- 6.2E	Transmit power for V2X

- 6.3E	Output power dynamics for V2X

- 6.4E	Transmit signal quality for V2X

- 6.5E	Output RF spectrum emissions for V2X

- 7.3E	Reference sensitivity for V2X

- 7.4E	Maximum input level for V2X

- 7.5E	Adjacent channel selectivity for V2X

- 7.6E	Blocking characteristics for V2X

- 7.7E	Spurious response for V2X

- 7.8E	Intermodulation characteristics for V2X

In NR-U, a number of RF requirements were introduced in RAN4 specifications in Rel-16. As an example, the following list shows impacted or introduced sections (refer to first large RAN4 CR from NR-U Rel-16, RP-202117):

- 5.2	Operating bands

- 5.2D	Operating bands for UL MIMO

- 6.2F	Transmit power for shared spectrum channel access

- 6.3F	Output power dynamics for shared spectrum channel access

- 6.4F	Transmit signal quality for shared spectrum channel access

- 6.5F	Output RF spectrum emissions for shared spectrum channel access

- 7.3F	Reference sensitivity for shared spectrum channel access

- 7.5F	Adjacent channel selectivity for shared spectrum channel access

- 7.6F	Blocking characteristics for shared spectrum channel access

- 7.7F	Spurious response for shared spectrum channel access

- 7.8F	Intermodulation characteristics for shared spectrum channel access

Since SL-U is being specified in Rel-18 to operate in same bands as supported by NR-U in FR-1, i.e., including Band n46 (5 GHz) and Band n96/n102 (6 GHz), it is expected that SL-U should also follow similar RF requirements introduced by NR-U in RAN4 specs or utilize NR-U requirements as baseline.

SL-U is being specified to operate in same bands as NR-U in FR1, thus SL-U may reuse NR-U RF requirements in large extent.

As a starting point, RAN4 should consider if the receiver requirements currently defined for shared spectrum channel access can be reused for operation of sidelink in unlicensed spectrum.

RAN4 to consider the receiver requirements currently defined for shared spectrum channel access for operation of sidelink in unlicensed spectrum.

Since ITS band n47 which is targeted for V2X is overlapped with shared spectrum access band n46 as shown in TS 38.101-1 - Table 5.2-1. It may be needed for RAN4 to assess if any additional Rx requirements are needed.

ITS band n47 is overlapping shared spectrum access band n46

TS 38.101-1 - Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD13 |
| n4711 | 5855 MHz – 5925 MHz | 5855 MHz – 5925 MHz | TDD |
| n9614 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD13 |
| n10214 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD13 |
| NOTE 11: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. NOTE 13: This band is restricted to operation with shared spectrum channel access as defined in 37.213. NOTE 14: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. |  |  |  |

RAN4 shall assess if any additional requirements are needed to address the overlapping of band n47 and n46.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: SL-U is being specified to operate in same bands as NR-U in FR1, thus SL-U may reuse NR-U RF requirements in large extent.

Proposal 1: RAN4 to consider the receiver requirements currently defined for shared spectrum channel access for operation of sidelink in unlicensed spectrum.

Observation 2: ITS band n47 is overlapping shared spectrum access band n46

Proposal 2: RAN4 shall assess if any additional requirements are needed to address the overlapping of band n47 and n46.

- References

- RP-230077, WID revision: NR sidelink evolution, OPPO.