3GPP TSG-RAN WG4 Meeting #106bis	R4-2305001

Electronic Meeting, 17 – 26 Apr, 2023

Source:	ZTE Corporation

Title:	Discussion on Timing requirements for UL multi-DCI multi-TRP with two TAs

Agenda Item:	5.30.3.2

Document for:	Approval

- Introduction

In RAN4 104 meeting, one LS[1] from RAN1 for this WID was received.

Conclusion: For multi-DCI multi-TRP operation with two TAs, the decision on the maximum uplink timing difference is left up to RAN4. - send an LS to RAN4 asking them the maximum uplink timing difference RAN1 can assume between the two TAs for multi-DCI multi-TRP operation. RAN1 would kindly like to ask RAN4 to provide feedback on what maximum uplink timing difference that RAN1 can assume between the two TAs for multi-DCI multi-TRP operation.

RAN4 has discussed the related issues for several meetings, some consensus were achieved while still some related issues are suspending. From last meeting, the discussion around these issues was held in this WID. The following agreements were achieved in [2].

Agreement: - Specify new MTTD/MRTD requirements for multi-DCI multi-TRP operation with 2 TAs, capture all the agreements related. Agreement: - For UE not supporting RTD>CP MRTD = CP

In this document, we will provide some further views on the remaining timing related issues for UL multi-DCI multi-TRP with two TAs.

- Discussion

- MTTD

Regarding to the MTTD for multi-DCI multi-TRP operation with 2 TAs, until right now, the following agreements were achieved:

Agreements: - For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP+M1 µs for FR1. Where M1 is FFS. - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered - For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 33/34.6 µs. Agreements: - For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP + M2 µs for FR2. Where M2 is FFS. - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered - For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 8/8.5 µs.

So it can be concluded that for the MTTD requirements, two categories can be identified:

- For UE not supporting RTD>CP, MTTD can be CP+M1 µs for FR1. MTTD can be CP + M2 µs for FR2.

- FFS for M1, M2.

- For UE supporting RTD>CP

- MTTD is 34.6 µs for FR1, i.e. reuse the inter-band requirements.

- MTTD is 8.5 µs for FR2, i.e. reuse the inter-band requirements.

The remaining is to identify the value of M1, M2. Based on the discussion in last meeting, the following options were suggested:

- Option 1: M1=M2=0 (Apple, MediaTek, vivo) - Option 1a: in both FR1 and FR2, for both intra-cell and inter-cell multi-TRP, the MTTD between multiple TRPs can be assumed within a CP length as baseline. (Apple) - Option 2: The MTTD between multiple TRPs can be defined as (CP + 1.6µs) for FR1 and (CP + 0.5µs) for FR2, e.g. M1=1.6us and M2=0.5 us. (Nokia, Xiaomi, Samsung, Huawei, Ericsson, Qualcomm)

Based on RAN4 understanding, MTTD is mainly defined by MRTD and UE transmission timing margin. For the case of multi-DCI multi-TRP, all TRPs operate at the same bandwidth, share the same frequency and same SCS. Regarding to the UE transmission timing margin, based on common understanding, three components were included, i.e. Te, TA adjustment accuracy and TA resolution error. We can estimate the value of the three components.

For Te, it is specified in Table 7.1.2-1 of [3] as follows:

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | [1.58]*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | [1.35]*64*Tc |
|  |  | 960 | [0.90]*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | [1.13]*64*Tc |
|  |  | 960 | [0.86]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

For TA adjustment accuracy, it is specified in Table 7.3.2.2-1 of [3] as follows:

Table 7.3.2.2-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | ±10 Tc | ±6 Tc |

For TA resolution error, based on the formula of TA given by [4], the TA resolution error is 16∙64∙Tc2μ. So 16*64Tc for FR1 and 256Tc for FR2-1.

Based on the above three components, we can derive the value of M1, M2. For FR1, M1 is about 1.6 us, and for FR2, M2 is about 0.5 us.

- Observation 1: MTTD is mainly defined by MRTD and UE transmission timing margin. Such margin includes three components: Te, TA adjustment accuracy and TA resolution error. Based on the spec, the sum of such three components is: 1.6 us for FR1 and 0.5 us for FR2.

Compared the current MRTD/MTTD, in fact the difference between MRTD and MTTD is 1.6 us for FR1 and 0.5 us for FR2, same as the above estimation. That is no matter whether the UE is capable of RTD > CP or not, the difference between MRTD and MTTD should be aligned, such difference would not change along with the UE capability. So obviously we prefer Option 2.

Proposal 1: No matter whether the UE is capable of RTD > CP or not, the difference between MRTD and MTTD should be aligned, i.e. M1 is 1.6us for FR1 and M2 is 0.5us for FR2.

- Reference timing

- Regarding to the reference timing, the following options were proposed in last meeting:

- P1: Clause 7.1: some clarification may be needed in the Introduction section regarding reference for UL Tx timing (Apple) - P2: The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission. (Nokia) - P3: Single reference timing is feasible. (Samsung) - P4: RAN4 need to study how to select the DL reference timing for each TAG on a CC and RAN1’s inputs on TAG association are needed. (Huawei) - P5: RAN4 should discuss whether single reference timing shall be considered or not and if it is considered. (Ericsson) - P6: FFS, more RAN1 inputs are needed.

- In uplink transmission timing, the reference timing is derived from the reception of the first detected path of the corresponding downlink frame. If the downlink signals are from m-TRP, UE applies the downlink signal detection for both of two TRPs. The reference timings will be different. So define independent reference timing for each UL Tx timing respectively is a direct solution.

- Furthermore, if UE can identify the offset between the two reference timing(i.e. the first detected path of DL), it seems that applying a single reference timing for the two UL Tx timing is feasible. Here referring to the offset between the two reference timing, the propagation delay difference between the m-TRPs should be considered.

Proposal 2: Regarding to the reference timing of UL timing, a direct solution is to define independent reference timing for each UL Tx timing respectively. While if UE can identify the offset between the two reference timing, applying a single reference timing for the two UL Tx is feasible.

- Whether to consider two UL transmissions associated with two TAs are partially overlapped

- With respect to this issue, the following options were proposed in last meeting:

- Option 1: RAN4 need to study the following two aspects: - Whether or when partially overlapped UL transmissions with different TAs is allowed. - How to handle overlapping part between two UL transmissions associated with two TAs if not allowed. - Option 2: From RAN4 RRM perspective, for TDM based multi-DCI uplink transmission in one component carrier, considering the worst case, the minimal separation between the two UL transmissions associated with two TAs should not be less than the transient period specified in RF specs. Overlapping between UL transmission is not allowed.

- In our view, this issue is closely related with the assumption of the PA architecture. Once only a single PA is assumed for the two UL transmissions associated with two TAs, it seems that the minimal separation between the two UL transmission should not be less than the transient period define in RF session.

- Proposal 3: Whether two partial overlapped UL transmissions with two TAs are allowed, which depends on the PA architecture.

- Whether to consider transient period between 2 UL signals with 2 different TAs for MRTD and MTTD requirements

- Regarding to this issue, the following option was proposed in last meeting:

- Option 1: Not to consider the transient period for MRTD and MTTD requirement. - Option 1a: There is no need to define the transient period for uplink timing adjustment operation. For UE not capable of supporting RTD>CP, the performance degradation is expected for the first symbol of the first slot after the switching between two UL signals with different TAs, when the timing difference between the two UL signals exceeds the CP length of UL SCS for data.

- In our opinion, the transient period is try to define the switching period between Tx ON/OFF, it seems that it is not related with MRTD at all. While regarding to MTTD, since referring to two Tx transmission, so it seems that the transient period would impact the MTTD. But from the other side, UE performs the switching between two UL transmissions associated with 2 different TAs more like different UL timing adjustments. For the UL timing adjustment definition, no need to consider the transient period. So similar logic can be reused, here referring to the requirement of MTTD, no need to consider the transient period between two UL signals.

- Proposal 4: For both requirements of MRTD and MTTD, no need to consider the transient period between two UL signals.

- Whether to study on TAG management for multi-TRP with 2 TAs

- During last meeting, some company proposed that the TAG management for 2TAs should be considered, we believe it makes sense. In fact there are two types of UE: 1）support RTD > CP; 2）not support RTD > CP. Such two types of UEs are both possible under the 2 TAGs mTRP case. For the UE supporting RTD >CP, the UE can always work. While for the UE not supporting RTD >CP, since the actual RTD can change through the UE movement or rotation, so the actual RTD can less than or larger than CP. If the actual RTD is larger than CP(i.e. a single FFT is assumed), such UE can not work under the assumption of 2 TAGs. If not introducing any UE reporting mechanism, the NW is unknown of the actual RTD at UE side, then when the UE performance would be degraded since of actual RTD >CP for the UE not support RTD >CP, the NW is also unknown. So we need to discuss the TAG management for the UE not supporting RTD >CP under the m-TRP with 2 TAs.

- Proposal 5: It is necessary to discuss the TAG management for the UE not supporting RTD >CP under the m-TRP with 2 TAs.

- Conclusion

In this contribution, we have the following observations and proposals for the Timing issues for UL multi-DCI multi-TRP with two TAs:

- Observation 1: MTTD is mainly defined by MRTD and UE transmission timing margin. Such margin includes three components: Te, TA adjustment accuracy and TA resolution error. Based on the spec, the sum of such three components is: 1.6 us for FR1 and 0.5 us for FR2.

Proposal 1: No matter whether the UE is capable of RTD > CP or not, the difference between MRTD and MTTD should be aligned, i.e. M1 is 1.6us for FR1 and M2 is 0.5us for FR2.

Proposal 2: Regarding to the reference timing of UL timing, a direct solution is to define independent reference timing for each UL Tx timing respectively. While if UE can identify the offset between the two reference timing, applying a single reference timing for the two UL Tx is feasible.

- Proposal 3: Whether two partial overlapped UL transmissions with two TAs are allowed, which depends on the PA architecture.

- Proposal 4: For both requirements of MRTD and MTTD, no need to consider the transient period between two UL signals.

- Proposal 5: It is necessary to discuss the TAG management for the UE not supporting RTD >CP under the m-TRP with 2 TAs.

- Reference

- R4-2211512	LS on maximum uplink timing difference for Multi-DCI Multi-TRP with two TAs, Ericsson.  RAN4 #104e

- R4-2303258 WF on R18 NR MIMO RRM requirements, Samsung. RAN4#106

- TS 38.133-h80.

- TS 38.213