3GPP TSG RAN WG4 Meeting #106bis-e  	                                                               R4-2305106

Online, April 17 – April 26, 2023

Title:	Reply LS to GSMA TSGAP on NR Bandwidth for OTA TRS testing

Release:	Rel-18

Work Item:	NR_FR1_TRP_TRS_enh

Source:	3GPP RAN4

To:	GSMA TSGAP

Cc:	RAN5

Contact person:	Ruixin Wang

ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

3GPP RAN4 would like to thank GSMA TSGAP for the LS (R4-2304023/TSGAP76_004) on NR Bandwidth for OTA TRS testing.

For NR TRP/TRS OTA testing, the test configurations for each band is using the same test parameters as those defined in RAN5 for conducted MOP (Maximum output power) and REFSENS (Reference sensitivity) testing.

RAN4 agreed that the default channel bandwidth for TRP/TRS OTA testing is the Mid test channel bandwidth for RF defined in TS 38.508-1 Table 4.3.1.0A-1. But some special configurations for band n28 (20MHz) and n41/n77/n78/n79 (100MHz) are adopted based on requests from operators.

Given the n41 and n78 TRP/TRS requirements have been defined in Rel-17 based on the measurement results of UE configured with 100MHz, RAN4 decided not to change the 100MHz channel bandwidth in TS 38.161.

Meanwhile, RAN4 is discussing whether an additional set of configurations for NR bands (e.g. 20 MHz CBW in high and mid bands, and 10 MHz in low bands) could be added in the TR 38.870 as informative examples.

3GPP RAN4 looks forward to further cooperation with GSMA TSGAP on NR FR1 TRP TRS work.

2	Actions

To GSMA TSGAP:

ACTION: 	3GPP RAN4 respectfully asks GSMA TSGAP to take the above feedback into account.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #107                           	 22-26 May, 2023    		     Incheon, KR

TSG-RAN WG4 Meeting #108                           	 21-25 August, 2023    		Toulouse, FR

Reference

- 3GPP TS 38.508-1 v17.7.0, User Equipment (UE) conformance specification; Part 1: Common test environment.

- 3GPP TR 38.870 v0.3.0, Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS).