# 3GPP TSG-RAN WG4 Meeting # 106bis-e                             R4-2305132

# Online, April 17 – April 26, 2023

# Source: 	ZTE Corporation

# Title: 	Views on RF requirements for CA band combination of n5-n28

# Agenda item:	4.30.2.2

# Document for:	Approval

# 1 Introduction

In last RAN4 #106 meeting, the SID of 700/800/900MHz was completed, all of the results for the band combination were captured in the TR38.872 [1], in which the requirements include different RF architecture without further converged.

Later, the SID was converted to WID [2] in last RAN #99 meeting. RAN4 need to make decisions on how to define the RF requirements during the WID phase, the objectives are.

Specify the following requirements based on down-selection of options of the outcome of SI

- Define Tib/Rib for the band combinations.

- Define REFRENS degradation for some specific band combinations if it’s needed.

- Define other necessary requirements for some specific band combinations if they are identified to be needed.

In this contribution, we give some discussion on how to define the new requirements concluded in the R18 SID.

# 2	Discussion

For CA_n5-n28, it is a bit special since the requirements for 1UL/2DL CA have already been defined/included in the Rel-17 specification. However, in Rel-18 SID phase, the discussions for both 1UL/2DL CA and 2UL/2DL CA were included based on different RF architectures, i.e. 2-antennas implementation and 3-antennas implementation.

In [3], we gave some backgrounds about the 1UL CA n5-n28 RF requirements (TIB,c and RIB,c) in the specification. In terms of the backgrounds, we can deduce that the RF requirement of NR CA_n5-n28 defined in the R17 spec were derived from quadplexer, i.e. 2 antenna implementation (total number of antennas).

Observation 1. The RF requirement of NR CA_n5-n28 defined in the R17 spec were derived from quadplexer, i.e. 2 antenna implementation.

When we compare the requirements included in the SID with the requirements defined in TS38.101-1, we can see that the requirements included in the SID are a bit different with the existing requirements defined in the spec.

# 1. ∆TIB,c and ∆RIB,c values

The requirements for ∆TIB,c and ∆RIB,c values in the SID TR38.872 are:5.2.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n28 with 2 antenna implementation, the TIB,c and RIB,c values has been specified in TS 38.101-1.

Referring to contribution R4-2300657, it is suggest to reuse the DC_28_n5 TIB,c and RIB,c values with an additional 0.2dB to all to enable the two-antenna n-plexers in the future and including the support of full band n28, which are shown below:

## Table 5.2.2.4-0: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 |
|  | n28 | 0.7 |

## Table 5.2.2.4-0a: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.2 |
|  | n28 | 0.2 |

For CA_n5-n28 with 3 antenna implementation, the TIB,c and RIB,c values are shown below.

# Table 5.2.2.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

# Table 5.2.2.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

The TIB,c values defined in TS 38.101-1 are 0.5dB for either n5 or n28, and RIB,c of 0dB values are for either n5 or n28, however, additional 0.2dB are also proposed, which means more stringent requirements than the existing requirements are included.

Observation 2. The TIB,c and RIB,c requirements, more stringent requirements than the existing requirements are included in the SID TR for 2 antenna implementation.

2. 1UL cross-band isolation MSD

The requirements for 1UL cross-band isolation MSD in the SID TR38.872 are:

Based on the agreed CR R4-2206134, one MSD test configuration (n28 30MHz) can be a reference considering the impacts from band n5 UL.

Table 5.2.2.5-1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

For CA_n5-n28 with 3 antennas implementation, referring to contribution R4-2301266, the n28 MSD evaluation on the same MSD test configuration (n28 30MHz) as 2 antennas implementation is shown as below.

Table 5.2.2.5-2 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 18.6 | ACLR2 |

It can be seen that the requirements of 3 antennas implementation is more stringent than the existing requirements derived from 2 antennas implementation.

Observation 3. The 1UL/2DL cross-band isolation MSD requirements, more stringent requirements than the existing requirements are included in the SID TR.

3. 2UL cross-band isolation MSD

2UL cross-band isolation MSD is a new issue, and it seems CA_n5-n28 is the first band combination to define/discuss the 2UL cross-band isolation MSD.

The requirements for 2UL cross-band isolation MSD in the SID TR38.872 are:

Table 5.2.3.3-2: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 793 | 20 | [3.6] | ACLR2 |
| n28 |  | 738 | 20 | 15 | 25 (Rbstart=81) |  |  |  | ACLR2 |

Table 5.2.3.3-3: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 2.9 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

Table 5.2.3.3-4: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 800.5 | 5 | 17.5 | n5 ACLR2 |
| n28 |  | 5 | 5 | 15 | 25 (Rbstart=0) |  |  |  |  |

Table 5.2.3.3-5: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 4.5 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

It can be seen the requirements for different implementations from different companies are different, even for the UL/DL configurations, ignoring one typo for n28 UL Fc in table 5.2.3.3-4, quite different for UL/DL configurations are seen. In addition, the cross-band interference source are also not the same among companies’s proposed results.

Regarding the 2UL cross-band isolation MSD, except the results in table 5.2.3.3-4 which was reused from 1UL, interference source in the other three results include:

1. ACLR2 from n5 UL band (aggressor band) falls into n28 DL band(DL Fc) (victim band)

2. ACLR1+ACLR2 from n28 UL band (aggressor band) falls into n28 DL band(DL Fc) (victim band)

In our understanding, the interference falls into band n28 DL band for 2UL cross-band isolation are the total interference from both 1UL n5 ACLR2 and 1UL n8 (ACLR1+ACLR2). However, for the interference from 1UL n5 ACLR falls into band n28 DL, it seems it was already included in the 1UL cross-band isolation MSD requirements.

Observation 4. The 2UL cross-band isolation MSD requirements, it includes the total interference from both 1UL n5 ACLR2 and 1UL n8 (ACLR1+ACLR2). In our understanding, for the interference from 1UL n5 ACLR falls into band n28 DL, it seems it was already included in the 1UL cross-band isolation MSD requirements.

Observation 5. The UL/DL configurations are different for the values proposed by companies.

Comparing 2UL cross-band isolation MSD and 1UL cross-band isolation MSD, we see interesting thing happen, i.e. the 2UL cross-band isolation MSD values proposed by majority companies are much smaller than 1UL cross-band isolation MSD value defined in the spec. Therefore, we think more discussions are needed, including whether to introduce 2UL  cross-band isolation MSD in the spec, and if 1UL cross-band isolation MSD could cover the 2UL cross-band isolation MSD, etc.

Proposal 1. More discussions are needed for the necessity for 2UL cross-band isolation MSD.

For a certain band combination, in our understanding, in general there exist different implementations in practical. Usually, 2-antennas implementation is adopted due to it will cause most stringent requirements among the implementations since some common RF components would be shared by the two bands. Also, there may exist some exceptions for some specific band combination in which requirements are more stringent in the implementation other than 2-antennas implementation.

Regarding the different requirements derived from different architecture, the most stringent requirements should be defined in the spec, which means the requirements defined in the specifications should be implementation agnostic

Proposal 2. The requirements for the band combination defined in the specifications should be implementation agnostic

Since the requirements for 1UL/2DL CA_n5-n28 have already been defined in Rel-17 TS38.101-1 spec, there may exist two methods to handle with the new requirements in Rel-18 WID:

1). Update the existing requirements by replacing the existing requirement with the new requirement.

2). Keep the existing requirement and define new requirement additionally.

The first one is a straightforward way since worse case requirements are defined. Meanwhile, in order to keep specification alignment, CR to R17 is also needed during the WID although it is for R18 WID.

The second one may not a promising method, it seems new capability related to architecture would be introduced, which needs more discussion during the WID. But it this way, only R18 CR is enough.

Proposal 3. How to treat the stringent requirement included in SID TR and to be included in the spec should be discussed.

# 3 Conclusion

In this contribution, we give some discussion on how to define the new requirements concluded in the R18 SID. The conclusions are summarized below:

Observation 1. The RF requirement of NR CA_n5-n28 defined in the R17 spec were derived from quadplexer, i.e. 2 antenna implementation.

Observation 2. The TIB,c and RIB,c requirements, more stringent requirements than the existing requirements are included in the SID TR for 2 antenna implementation.

Observation 3. The 1UL/2DL cross-band isolation MSD requirements, more stringent requirements than the existing requirements are included in the SID TR.

Observation 4. The 2UL cross-band isolation MSD requirements, it includes the total interference from both 1UL n5 ACLR2 and 1UL n8 (ACLR1+ACLR2). In our understanding, for the interference from 1UL n5 ACLR falls into band n28 DL, it seems it was already included in the 1UL cross-band isolation MSD requirements.

Observation 5. The UL/DL configurations are different for the values proposed by companies.

Proposal 1. More discussions are needed for the necessity for 2UL cross-band isolation MSD.

Proposal 2. The requirements for the band combination defined in the specifications should be implementation agnostic.

Proposal 3. How to treat the stringent requirement included in SID TR and to be included in the spec should be discussed.

# 4	Reference

[1] R4-2300561, TR 38.872 v0.4.0, CATT.

[2] RP-230764, New WID on enhancement for 700/800/900MHz band combinations, CATT

[3] R4-2219206, TP to TR38.872: TIB,c and RIB,c values for n5-n28, ZTE

[4] R4-2301265, On CA band combination of n5-n28, ZTE