# 3GPP TSG-RAN WG4 Meeting # 106bis-e                                                       R4-2305140

# Online, April 17 – April 26, 2023

Source:	ZTE Corporation

Title:	TP for TR 38.894: Harmonic mixing MSD for simultaneous RxTx CA_n39-n41

Agenda item:			4.27.2

Document for:	Approval

1. Introduction

For PC3 simultaneous RxTx CA_n39-n41, the MSD for harmonic mixing issue (3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL) are still FFS.

The RF architecture for simultaneous RxTx CA_n39-n41 can be found in [1]. The harmonic MSD is evaluated as:

Table 1. Harmonic mixing MSD for n41->n39

| n39 REFSEN@5MHz | 5MHz |  | 40MHz |  |
| --- | --- | --- | --- | --- |
| n39 REFSEN@5MHz | -100 |  | -90.6 |  |
|  | Main path | Div path | Main path | Div path |
| Total interference noise referred to antenna port (dBm) (Main path) | -91.6 | -91.8 | -91.6 | -91.8 |
| MSD (Mian) | 4.4 | 4.2 | 0.8 | 0.7 |
| MSD(after MRC) | 4.3 |  | 0.8 |  |

Therefore, the  harmonic MSD for harmonic mixing issue (3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL) are defined as:

Table 2. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 4.3 | NOTE x | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 0.8 | NOTE x | UL3/DL4 |
| NOTE x: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

A text proposal for TR 38.894 [2] to include the harmonic mixing MSD for simultaneous Rx/Tx CA_n39-n41.

2. Reference

[1 R4-2215899,  RF requirements on CA_n39-n41 supporting simultaneous Rx/Tx, ZTE

# [2] TR 38.894, V0.1.0

2. Text Proposal

---Start of changes---

# 5.2	CA_n39A-n41A

5.2.1	Status of the band combination

5.2.1.1	Operating bands for CA

The operating bands for CA_n39-n41 are specified in Table 5.2.1.1-1.

Table 5.2.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.x-1) |
| --- | --- |
| CA_n39-n41 | n39, n41 |

For CA_n39-n41, the current spec only considers scenarios without simultaneous Rx/Tx.

5.2.1.2 Power class of the BC

The UE Power Class for uplink CA_n39A-n41A are specified in Table 5.2.1.2-1.

Table 5.2.1.2-1 UE Power Class for uplink CA_n39A-n41A

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n39A-n41A | 23 | +2/-3 |

5.2.2	MSD analysis for simultaneous Rx/Tx

Table 5.2.2-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n39-n41.

Table 5.2.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there is no harmonic issue for the band combination of n39 and n41.

Table 5.2.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there are UL3/DL4 order harmonic mixing issue for the band combination of n39 and n41, i.e.  3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL.

5.2.3 Requirements for simultaneous Rx/Tx

For CA_n39-n41 supporting simultaneous Rx/Tx, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 5.2.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n39-n41 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.2.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n39-n41 | 0.2 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

The cross band isolation MSD for CA_n39-n41 supporting simultaneous Rx/Tx are defined in table 5.2.3-3.

Table 5.2.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 1.4 | >ACLR2 |

The harmonic mixing MSD requirements for CA_n39-n41 supporting simultaneous Rx/Tx are defined in table 5.2.3-4.

Table 5.2.3-4. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 4.3 | NOTE x | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 0.8 | NOTE x | UL3/DL4 |
| NOTE x: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

---End of changes---