3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305170

Electric Meeting, 17th April – 26th April 2023

Agenda item:	5.29.1.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on NCR class declaration on NCR type 1-H and 1-O.

Document for:	Approval

- Introduction

In the RAN4 meeting#104bis-e, the discussion of NCR RF requirements was conducted. The baseline assumption that same NCR type/class applied for NCR-fwd and NCR-MT was agreed [1]. In last meeting RAN4#106, NCR type was agreed [2], which can be categorized into 1-C/1-H/1-O/2-O, but the RF class of NCR type-1-H and 1-O are still open. In this contribution, we provide our views on NCR class and its RF requirements.

- Discussion

In RAN4#104bis-e, the categorization for NCR type and NCR class were agreed [1] as below.

RAN4#104bis-e [1] Issue 2-2 NCR type and NCR class: - Agreements: - For NCR-fwd link in Rel-18, at least reuse the Rel-17 repeater type/Class i.e., - Repeater Type: 1-C and 2-O; - Repeater class: wide-area and local area (DL, UL), medium range for DL - FFS for repeater type 1-H and 1-O for FR1 repeater given the beamforming capability assumption; - The baseline assumption that same type/class applied for NCR-fwd link and NCR-MT

As state above agreement, for NCR-fwd link in Rel-18, the Rel-17 repeater type and class are reused because repeater type 1-C and 2-O in Rel-17 had been specified and the RF configuration was the same as the NCR-fwd.

In last meeting RAN4#106, the NCR type was agreed [2] as below.

RAN4#106 [2] Issue 1-3: NCR types - Agreement: - For NCR: 1-C, 1-H and 1- O and 2-O need to be specified. - FFS whether separate declaration needed for MT, NW side and UE side at fwd. link for FR1 NCR

As state above agreement, NCR type is categorized into 1-C, 1-H, 1-O and 2-O. The type 1-H and 1-O were not addressed in Rel-17, therefore, NCR class needs to be stated for NCR type 1-H and 1-O in Rel-18.

Observation 1: NCR class for NCR type 1-H/1-O needs to be clearly stated separately from NR repeater in Rel-17.

For NCR class 1-H, the deployment scenario is the same as NCR class 1-C and the difference with NCR class 1-C is mainly reference points, which is not relevant to classification. Thus, the classification method can be the same for UL/DL for 1-C/1-H in the NCR as it is the same for BS for 1-C and 1-H.

Proposal 1: NCR class for NCR type 1-H: wide-area and local area (DL, UL), medium range for DL.

For NCR class 1-O, the deployment scenario is the same as NCR class 2-O and the difference with NCR class 2-O is mainly frequency range, which is not relevant to classification. Thus, the classification method can be the same for UL/DL for 1-O/2-O in the NCR as it is the same for BS for 1-O and 2-O.

Proposal 2: NCR class for NCR type 1-O: wide-area and local area (DL, UL), medium range for DL.

- Conclusion

In this contribution, we provide views on NCR class declaration for NCR-Fwd type 1-H and 1-O. The following an observation and a proposal are obtained.

Observation 1: NCR class for NCR type 1-H/1-O needs to be clearly stated separately from NR repeater in Rel-17.

Proposal 1: NCR class for NCR type 1-H: wide-area and local area (DL, UL), medium range for DL.

Proposal 2: NCR class for NCR type 1-O: wide-area and local area (DL, UL), medium range for DL.

References

- R4-2217471, “WF on NCR RF requirements”, ZTE, RAN4#104bis-e, October 2022

- R4-2302903, “WF for NCR system parameters and RF requirements”, ZTE, RAN4#106, March 2023