3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305173

Electronic meeting, 17 April – 26 April 2023

Agenda Item:	5.19.2

Source:	Ericsson

Title:	Simulation results of absolute physical layer throughput requirements

Document for:	Information

# 1	Introduction

RAN4#106 agreed with the way forward on physical layer throughput requirements [1]. This contribution provides our updated simulation results according to the simulation assumption.

# 2	Simulation results

## 2.1	Simulation parameters

Table 1 shows the simulation parameters. As we discussed in RAN4#106, we added options to enable HARQ retransmission during the tests.

Table 1	Simulation assumption for alignment (based on Table 5.10-3-1 in [2]).

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2-1 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0 to 20dB with 2dB step | 0 to 20dB with 2dB step | 0 to 16dB with 2dB step |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission (Note 4) |  |  | Option 1: 1 Option 2: 4 | Option 1: 1 Option 2: 4 | Option 1: 1 Option 2: 4 |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). Note 3: The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank 2. Note 4: For the maximum number of HARQ transmission with 4, configure RV {0,2,3,1} with same MCS and rank as initial transmission; for precoder, follow the latest UE reported PMI whose rank is same as the initial transmission. Note 5: TxEVM=6%. |  |  |  |  |  |

## 2.2	Simulation results (Maximum number of HARQ transmission is 1)

According to WF [1], we provide throughput vs. SNR, and statistics of BLER, median CQI and median RI. In this simulation results we have not enabled HARQ retransmission.

### 2.2.1	FR1 FDD 15kHz

Table 2	Simulation result of FDD 15kHz with 2Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (64.54 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 4 | 0.06 | 0.09 | 3 | 1 |
| 2 | 5.8 | 0.09 | 0.09 | 4 | 1 |
| 4 | 7.6 | 0.12 | 0.11 | 5 | 1 |
| 6 | 9.6 | 0.15 | 0.11 | 6 | 1 |
| 8 | 11.7 | 0.18 | 0.12 | 7 | 1 |
| 10 | 14 | 0.22 | 0.13 | 8 | 1 |
| 12 | 16.7 | 0.26 | 0.13 | 9 | 1 |
| 14 | 19.5 | 0.3 | 0.13 | 9 | 1 |
| 16 | 22.7 | 0.35 | 0.13 | 9 | 2 |
| 18 | 25.7 | 0.4 | 0.13 | 9 | 2 |
| 20 | 28.1 | 0.44 | 0.17 | 10 | 2 |

Table 3	Simulation result of FDD 15kHz with 4Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (64.54 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 5.2 | 0.08 | 0.41 | 5 | 1 |
| 2 | 7.4 | 0.11 | 0.37 | 5 | 1 |
| 4 | 10.4 | 0.16 | 0.3 | 5 | 2 |
| 6 | 13.6 | 0.21 | 0.27 | 6 | 2 |
| 8 | 17.7 | 0.27 | 0.21 | 7 | 2 |
| 10 | 21 | 0.33 | 0.21 | 8 | 2 |
| 12 | 24.4 | 0.38 | 0.22 | 9 | 2 |
| 14 | 28.5 | 0.44 | 0.22 | 10 | 2 |
| 16 | 31.8 | 0.49 | 0.23 | 10 | 2 |
| 18 | 35.9 | 0.56 | 0.23 | 12 | 2 |
| 20 | 41 | 0.64 | 0.21 | 13 | 2 |

### 2.2.2	FR1 TDD 30kHz

Table 4	Simulation result of TDD 30kHz with 2Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (207.4336 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 11.3 | 0.05 | 0.12 | 3 | 1 |
| 2 | 16.2 | 0.08 | 0.12 | 4 | 1 |
| 4 | 21.4 | 0.1 | 0.16 | 5 | 1 |
| 6 | 27 | 0.13 | 0.17 | 6 | 1 |
| 8 | 33.1 | 0.16 | 0.18 | 7 | 1 |
| 10 | 40.2 | 0.19 | 0.17 | 8 | 1 |
| 12 | 44.5 | 0.21 | 0.22 | 9 | 1 |
| 14 | 49.4 | 0.24 | 0.26 | 9 | 1 |
| 16 | 61.6 | 0.3 | 0.21 | 9 | 2 |
| 18 | 70.5 | 0.34 | 0.22 | 9 | 2 |
| 20 | 75 | 0.36 | 0.27 | 9 | 2 |

Table 5	Simulation result of TDD 30kHz with 4Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (207.4336 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 20.9 | 0.1 | 0.19 | 5 | 1 |
| 2 | 27.1 | 0.13 | 0.19 | 6 | 1 |
| 4 | 38.1 | 0.18 | 0.11 | 5 | 2 |
| 6 | 48.1 | 0.23 | 0.12 | 5 | 2 |
| 8 | 59.7 | 0.29 | 0.11 | 6 | 2 |
| 10 | 74.7 | 0.36 | 0.05 | 7 | 2 |
| 12 | 88.3 | 0.43 | 0.06 | 8 | 2 |
| 14 | 99.4 | 0.48 | 0.09 | 9 | 2 |
| 16 | 112.2 | 0.54 | 0.1 | 10 | 2 |
| 18 | 116.6 | 0.56 | 0.17 | 11 | 2 |
| 20 | 128.3 | 0.62 | 0.19 | 12 | 2 |

### 2.2.3	FR2-1 TDD 120kHz

Table 6	Simulation result of TDD 120kHz with 2Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (243.5304 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 18.5 | 0.08 | 0.18 | 5 | 1 |
| 2 | 25.9 | 0.11 | 0.15 | 7 | 1 |
| 4 | 33.7 | 0.14 | 0.15 | 8 | 1 |
| 6 | 43.4 | 0.18 | 0.13 | 9 | 1 |
| 8 | 52.9 | 0.22 | 0.13 | 10 | 1 |
| 10 | 62 | 0.25 | 0.17 | 11 | 1 |
| 12 | 76.5 | 0.31 | 0.14 | 12 | 1 |
| 14 | 92.8 | 0.38 | 0.1 | 10 | 2 |
| 16 | 107.2 | 0.44 | 0.1 | 10 | 2 |

## 2.3	Simulation results (Maximum number of HARQ transmission is 4)

According to WF [1], we provide throughput vs. SNR, and statistics of BLER, median CQI and median RI. In this simulation results we have enabled HARQ retransmission.

### 2.2.1	FR1 FDD 15kHz

Table 2	Simulation result of FDD 15kHz with 2Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (64.54 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 4.2 | 0.06 | 0.07 | 3 | 1 |
| 2 | 6 | 0.09 | 0.08 | 4 | 1 |
| 4 | 8 | 0.12 | 0.07 | 5 | 1 |
| 6 | 10 | 0.15 | 0.08 | 6 | 1 |
| 8 | 12.4 | 0.19 | 0.08 | 7 | 1 |
| 10 | 14.8 | 0.23 | 0.09 | 8 | 1 |
| 12 | 17.5 | 0.27 | 0.09 | 9 | 1 |
| 14 | 20.4 | 0.32 | 0.09 | 9 | 1 |
| 16 | 23.5 | 0.36 | 0.1 | 9 | 2 |
| 18 | 27.2 | 0.42 | 0.09 | 9 | 2 |
| 20 | 30.1 | 0.47 | 0.12 | 10 | 2 |

Table 3	Simulation result of FDD 15kHz with 4Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (64.54 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 6.7 | 0.1 | 0.28 | 5 | 1 |
| 2 | 9.3 | 0.14 | 0.22 | 5 | 1 |
| 4 | 12.1 | 0.19 | 0.2 | 5 | 2 |
| 6 | 15.6 | 0.24 | 0.17 | 6 | 2 |
| 8 | 19.4 | 0.3 | 0.14 | 7 | 2 |
| 10 | 23.1 | 0.36 | 0.14 | 8 | 2 |
| 12 | 27.2 | 0.42 | 0.14 | 9 | 2 |
| 14 | 31.4 | 0.49 | 0.15 | 10 | 2 |
| 16 | 35.4 | 0.55 | 0.15 | 10 | 2 |
| 18 | 39.9 | 0.62 | 0.16 | 12 | 2 |
| 20 | 45.6 | 0.71 | 0.13 | 13 | 2 |

### 2.2.2	FR1 TDD 30kHz

Table 4	Simulation result of TDD 30kHz with 2Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (207.4336 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 12.1 | 0.06 | 0.08 | 3 | 1 |
| 2 | 17.2 | 0.08 | 0.09 | 4 | 1 |
| 4 | 22.8 | 0.11 | 0.11 | 5 | 1 |
| 6 | 29.2 | 0.14 | 0.12 | 6 | 1 |
| 8 | 36.1 | 0.17 | 0.11 | 7 | 1 |
| 10 | 43.4 | 0.21 | 0.11 | 8 | 1 |
| 12 | 50 | 0.24 | 0.14 | 9 | 1 |
| 14 | 56.8 | 0.27 | 0.16 | 9 | 1 |
| 16 | 67.7 | 0.33 | 0.14 | 9 | 2 |
| 18 | 78.9 | 0.38 | 0.14 | 9 | 2 |
| 20 | 87.9 | 0.42 | 0.15 | 9 | 2 |

Table 5	Simulation result of TDD 30kHz with 4Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (207.4336 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 22.7 | 0.11 | 0.14 | 5 | 1 |
| 2 | 29.7 | 0.14 | 0.12 | 6 | 1 |
| 4 | 40.2 | 0.19 | 0.06 | 4 | 2 |
| 6 | 49.9 | 0.24 | 0.08 | 5 | 2 |
| 8 | 62.5 | 0.3 | 0.07 | 6 | 2 |
| 10 | 76.4 | 0.37 | 0.04 | 7 | 2 |
| 12 | 90.4 | 0.44 | 0.05 | 8 | 2 |
| 14 | 103.6 | 0.5 | 0.06 | 9 | 2 |
| 16 | 117 | 0.56 | 0.07 | 10 | 2 |
| 18 | 126.7 | 0.61 | 0.11 | 11 | 2 |
| 20 | 141.7 | 0.68 | 0.11 | 12 | 2 |

### 2.2.3	FR2-1 TDD 120kHz

Table 6	Simulation result of TDD 120kHz with 2Rx

| SNR | Absolute throughput (Mbps) | Relative to the maximum throughput (243.5304 Mbps) | BLER | Median CQI | Median RI |
| --- | --- | --- | --- | --- | --- |
| 0 | 19.9 | 0.08 | 0.13 | 5 | 1 |
| 2 | 27.6 | 0.11 | 0.11 | 7 | 1 |
| 4 | 35.9 | 0.15 | 0.12 | 8 | 1 |
| 6 | 44.6 | 0.18 | 0.12 | 9 | 1 |
| 8 | 55.5 | 0.23 | 0.1 | 10 | 1 |
| 10 | 66.2 | 0.27 | 0.12 | 11 | 1 |
| 12 | 80.1 | 0.33 | 0.1 | 12 | 1 |
| 14 | 96.5 | 0.4 | 0.07 | 10 | 2 |
| 16 | 111 | 0.46 | 0.08 | 10 | 2 |

# References

- R4-2302930, “WF for Absolute physical layer throughput requirements”, Moderator (Intel).

- TR37.901-5, “Study on 5G NR User Equipment (UE) application layer data throughput performance”, 3GPP.

# Appendix

## A.1	Mapping of CQI Index to Information Bit payload

### A.1.1	CQI Table 1 (64QAM) (TS38.101-4 Table A.4-1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 |
| --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 6 | 6 |
| Available RE-s |  |  |  | 7590 | 7590 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR | N/A | N/A |
| 1 | 0.2344 | 0 | QPSK | 1800 | 3624 |
| 2 | 0.2344 | 0 |  | 1800 | 3624 |
| 3 | 0.3770 | 2 |  | 2856 | 5640 |
| 4 | 0.6016 | 4 |  | 4480 | 8968 |
| 5 | 0.8770 | 6 |  | 6528 | 13064 |
| 6 | 1.1758 | 8 |  | 8712 | 17928 |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 |
| 8 | 1.9141 | 13 |  | 14344 | 28680 |
| 9 | 2.4063 | 15 |  | 17928 | 35856 |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 |
| 11 | 3.3223 | 20 |  | 25104 | 50184 |
| 12 | 3.9023 | 22 |  | 29192 | 58384 |
| 13 | 4.5234 | 24 |  | 33816 | 67584 |
| 14 | 5.1152 | 26 |  | 38936 | 77896 |
| 15 | 5.5547 | 28 |  | 42016 | 83976 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-1 of TS 38.214 [12] |  |  |  |  |  |

### A.1.2	CQI Table 2 (256QAM) (TS38.101-4 Table A.4-2)

| TBS Scheme |  |  |  | TBS.2-1 | TBS.2-2 | TBS.2-3 | TBS.2-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | QPSK | 1480 | 2976 | 2976 | 5896 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 | 4744 | 9480 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 | 11016 | 22536 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18432 | 18960 | 37896 |
| 5 | 1.9141 | 7 |  | 12040 | 24072 | 24576 | 49176 |
| 6 | 2.4063 | 9 |  | 15112 | 30216 | 30728 | 61480 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 33816 | 34816 | 69672 |
| 8 | 3.3223 | 13 |  | 20496 | 40976 | 42016 | 83976 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 | 49176 | 98376 |
| 10 | 4.5234 | 17 |  | 28168 | 56368 | 57376 | 114776 |
| 11 | 5.1152 | 19 |  | 31752 | 63528 | 65576 | 131176 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 | 69672 | 139376 |
| 13 | 6.2266 | 23 |  | 38936 | 77896 | 79896 | 159880 |
| 14 | 6.9141 | 25 |  | 43032 | 86040 | 88064 | 176208 |
| 15 | 7.4063 | 27 |  | 46104 | 92200 | 94248 | 188576 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-2 of TS 38.214 [12] |  |  |  |  |  |  |  |

## A.2	Theoretical maximum throughput

### A.2.1	FDD SCS=15kHz

Figure 1 illustrates the DL scheduling for physical layer throughput test for FDD SCS=15kHz. Since 14 of 20 slots are available every 20ms for PDSCH and the maximum TBS for TBS.2-2 is 92200 bits, the theoretical maximum physical layer throughput (over 20ms) is 64.54 Mbps.

Figure 1	DL scheduling for FDD SCS=15kHz

### A.2.2	TDD SCS=30kHz (7D1S2U)

Figure 2 illustrates the UL/DL scheduling for physical layer throughput test for TDD SCS=30kHz with 7D1S2U. Since 22 of 40 slots are available every 20ms for PDSCH and the maximum TBS for TBS.2-4 is 188576 bits, the theoretical maximum physical layer throughput (over 20ms) is 207.4336 Mbps.

Figure 2	UL/DL scheduling for TDD SCS=30kHz with 7D1S2U.

### A.2.3	TDD SCS=120kHz (2D1S1U)

Figure 3 illustrates the UL/DL scheduling for physical layer throughput test for TDD SCS=120kHz with 2D1S1U. Since 58 of 160 slots are available every 20ms for PDSCH and the maximum TBS for TBS.1-2 is 83976 bits, the theoretical maximum physical layer throughput (over 20ms) is 243.5304 Mbps.

Figure 3	UL/DL scheduling for TDD SCS=120kHz with 2D1S1U.