3GPP TSG RAN WG4 Meeting #106bis-e	R4-2305195

Electronic Meeting, April 17th –  April 26th , 2023

Source: 	Ericsson

Title:  	RRM requirements for NR NTN enhancement

Agenda Item:	5.27.5

Document for:	Discussion

# Introduction

WID on NR NTN (Non-Terrestrial Networks) enhancements [1] is introduced in Rel-18 release. In this contribution, we present our viewpoints and proposals on the RRM requirements for this feature.

# Discussion

## Topic #2: Coverage enhancement

The objective is focused on the applicability of the solutions developed by general NR coverage enhancement to NTN.

The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

The ongoing discussions in RAN1/RAN2 haven’t triggered or initiate any issues in RRM requirements yet. We can wait for further outcome from them.

Observation 1: Coverage enhancement has no impact on RRM.

## Topic #3: NR-NTN deployment in above 10 GHz bands

Proposal 1: For above 10 GHz bands, mobility between 10 GHz bands ( e.g., intra-frequency and inter-frequency) is supported. Mobility between FR1 and 10 GHz bands in NTN isn’t covered by the WI.

Proposal 2: Table 1 below provides high level overview of existing RRM requirements which are possibly impacted due to the NTN operation in the 10 GHz bands. Note: the reference for comparison is existing NTN requirements in Rel-17.

Table 1 Summary of RRM requirements which are possibly impacted by NTN operation in the NTN-Ka bands

| Requirements |  | Potential impact due to NR-NTN deployment in Ka bands (i.e. above 10 GHz bands according to the WID) |
| --- | --- | --- |
| Cell reselection | UE measurement capability | Different capability in NTN-Ka bands |
|  | Measurements on intra/inter-frequency NR cells | Impacted due to new frequency band category, possible side condition change. |
|  | ﻿Maximum interruption in paging reception | No impact |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion | Impacted due to new frequency band category, possible side condition change. |
| Handover | Handover | Impacted due to new frequency band category |
|  | NR conditional handover | Impacted due to new frequency band category |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment | No impact |
| Timing | UE transmit timing | Impacted due to new SCS and frequency band category |
|  | UE timer accuracy | Impacted due to new SCS and frequency band category |
|  | Timing advance | Impacted due to new SCS and frequency band category |
| Signaling | RLM | Impacted due to new frequency band category |
|  | LR | Impacted due to new frequency band category |
|  | BWP switch | No impact |
|  | TCI state switch | No impact |
| Measurement procedure | UE Measurement capability | Different capability in NTN-Ka bands |
|  | Intra/inter-frequency measurements | Impacted due to new frequency band category, possible side condition change. |
|  | L1-RSRP measurements | Impacted due to new frequency band category |

### Subcarrier spacing for different UL/DL signals/channels and RRM UL timing requirements

The basic uncertainty budget for UE UL can be written as:

CP - Dispersion - 2(Te + ∆TA_command + ∆TA_adjust) > 0

Where:

- CP is Cyclic Prefix

- Dispersion is radio channel delay dispersion (time spanned of delayed signals).

- Te is UE UL initial timing error (TS 38.133, 7.1.2), the +/- difference of UE TX actual transmission time compared to the nominal transmission time given by 
DL time reference and TA command.

- ∆TA_command is TA command quantization error (82μ) Ts units

- ∆TA_adjust is UE TA adjustment error (TS 38.133, 7.3.2).

The factor of 2 originates from the fact that one UE can be off +Te and another UE in the same cell can be off -Te when received at BS RX. The same is true for the other error factors.

### 3GPP release-17

This means that the Dispersion term is bounded by:

CP - 2(Te + ∆TA_command + ∆TA_adjust) > Dispersion 			(1)

For Te 3GPP RAN4 has specified for FR1:

Plugging in into (1) we get:

In 3GPP release 17 we assumed 50 meters position error for the UE and 30 meters of position error for the satellite.  The actual error is doubled, i.e., 2*(50 + 30) m = 160 m, since all UE UL timing has to transmit early to compensate for Round Trip Time (RTT) => we can have +160 m RTT or  –160 m RTT added or deleted to the path, due to UE and satellite position errors, worst case.  In release 17 a UE GNSS position accuracy of 50 m was assumed together with a 30 m position error for the satellite. This led to the exclusion of SCS = 60 kHz for FR1.

Observation 2: In release 17 a UE GNSS position accuracy of 50 m was assumed together with a 30 m position error for the satellite. This led to the exclusion of SCS = 60 kHz for FR1.

For example, in legacy SCS = 15 kHz for data and SSB has Te = 12 Ts => Te_NTN = 12 + 16.4 Ts  = 28.4 Ts (29 Ts in 3GPP due to some additional margins), where 16.4 Ts = (160 m)/c.

### 3GPP release-18

If we consider equation (1) for SCS = 120 kHz in FR2 we have:

- SCS = 120 kHz, SSB DL = 120 kHz.

- CP is Cyclic Prefix = 18 Ts ≈ 0,585 µs

- Dispersion ≈ 0.26 µs (say)

- ∆TA_command is TA command quantization error (82μ) Ts units = 1 Ts ≈ 0.032 µs

- ∆TA_adjust is UE TA adjustment error (TS 38.133, 7.3.2). = 0.5 Ts≈ 0.0163 µs

CP -Dispersion - 2(∆_(TA_command) + ∆_(TA_adjust))2>Te

We get 0.114 µs ≈ 3.5 Ts > Te

This is Te = 3.5 Ts for terrestrial today => This is impossible with GNSS margin on top.

Observation 3: With current release-17 GNSS position inaccuracies for UE and satellite (50 m and 30 m, respectively), then SCS = 120 kHz would result in zero margin for GNSS error.

If we focus on SCS = 60 kHz UE UL, we have different options:

- Stricter GNSS positioning error in RAN4, for SCS = 60 kHz and normal CP

- Extended Cyclic prefix, ECP, for SCS = 60 kHz.

For SCS = 60 kHz, UL data, SSB DL = 120 kHz,  normal CP we have:

- CP is Cyclic Prefix = 36 Ts ≈ 1.17 µs

- Dispersion ≈ 0.26 µs (say)

- ∆TA_command is TA command quantization error (82μ) Ts units = 2 Ts ≈ 0.0651 µs

- ∆TA_adjust is UE TA adjustment error (TS 38.133, 7.3.2). = 1 Ts ≈ 0.032 µs

CP -Dispersion - 2(∆_(TA_command) + ∆_(TA_adjust))2>Te

0.358 µs > Te  11 Ts ,

Te = 3.5 Ts for terrestial today => 7.5 Ts margin => RTT error of 7.5 Ts * c = 73 m = pos error of 37 meters in total = 23 meters UE GNSS pos error + 14 meters satellite pos error

Observation 4: A total positioning error of 37 meters (23 meters UE GNSS and 14 meters of satellite position error) would suffice for UE UL SCS = 60 kHz, corresponding to Te_NTN = 11 Ts.

For SCS = 60 kHz, UL data, SSB DL = 120 kHz, Extended CP (12 symbols per slot instead of 14), we get:

- ECP is optional in TS 38.306 UE capability spec.

- CP is Cyclic Prefix = 144 Ts ≈ 4.69 µs

- Dispersion ≈ 0.26 µs (say)

- ∆TA_command is TA command quantization error (82μ) Ts units = 2 Ts ≈ 0.0651 µs

- ∆TA_adjust is UE TA adjustment error (TS 38.133, 7.3.2). = 1 Ts ≈ 0.032 µs

CP -Dispersion - 2(∆_(TA_command) + ∆_(TA_adjust))2>Te

2.25 µs > Te  69 Ts ,

Te = 3.5 Ts for terrestial today => 65.5 Ts margin => RTT error of 65.5 Ts * c = 640 m = pos error of 320 meters in total allowed => Existing 50 m UE position error and 30 m satellite position error fit nicely.

Observation 5: ECP and UL SCS = 60 kHz work with existing 50 m UE position error and 30 m satellite position error.

## Topic #4: Network verified UE location

As per update in WID [1], RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view. Legacy Rel. 17 Multi-RTT based positioning technique relies on positioning measurement performed on PRS resources transmitted from multiple/different TRPs. In contrast to the legacy method, network verified UE positioning in NTN is expected to exploit multiple RTT measurements performed on PRS resource(s) transmitted from the same satellite over different time occasions. In this regard, existing RRM requirement on UE Rx-Tx time difference measurements may be influenced by multi-RTT solution in NTN.

Observation 6:  Network verified UE positioning in NTN is expected to exploit multiple RTT measurements performed on PRS resource(s) transmitted from the same satellite over different time occasions.

Another aspect of multi-RTT in NTN different from terrestrial system is satellite(s) for positioning may face frequent handover resulting from LEO satellite moving characteristics. It is evitable and common that the measurement period may happen at any time instance before/during/after HO procedure. It shall be checked whether cell change, especially satellite switch, impacts UE Rx-Tx time difference measurements and corresponding requirements.

Proposal 3: RAN4 shall study RRM requirements on UE Rx-Tx time difference measurements specified by single satellite based multi-RTT solution in NTN, e.g. measurement period requirements, report mapping.

Observation 7: Frequently encountering cell changes in NTN (LEO deployment) in multi-RTT measurement procedure isn’t corner case.

Proposal 4: Study multi-RTT measurement requirements in NTN compatible with cell change, e.g. HO.

## Topic #5: NTN-TN and NTN-NTN mobility and service continuity enhancements

### Issue 5-2: NTN-TN Cell reselection enhancements

Regarding NTN-TN mobility, the issue differs depending upon the type of network where UE is camping, a UE camping in TN with NTN neighbor cells or a UE. Ranking, measurements for cell reselection both could be affected. An agreed understanding in RAN2 is an NTN/TN capable UE isn’t required to measure at TN frequencies if it is located far from the TN coverage. On the contrary, it’s also may be true that an NTN/TN capable UE isn’t required to measure at NTN frequencies if it is situated in TN coverage.

In NR NTN Rel-17, it was agreed that assistance information of neighbor cells, e.g. ephemeris data, frequency layer and PCI shall be mandatory, otherwise UE is not required to perform neighbor cell measurements if the corresponding assistance information is not present. Back to NTN-TN scenario, for a UE camping in TN with NTN neighbor cells(especially for LEO deployment), the agreement demands TN cell always broadcasts NTN satellite information which may be renewed dynamically, it  causes extremely numerous signaling overhead and brings difficulties to network configurations.

From signaling overhead perspective, a TN cell isn’t feasible to always broadcast the updating assistance information of neighbor NTN cells. If not acquiring this information from SI, UE has to acquire ephemeris data from neighbor NTN cells, it will extend the measurement delay.

Proposal 5: RAN4 shall revisit the applicability of mandatory assistance information of neighbor NTN cells ( e.g. ephemeris data, frequency layer and PCI) to a serving NTN cell in NTN-TN scenario. The information is beneficial but shall not be mandatory to mobility from TN to NTN.

Proposal 6: RAN4 study shall comply with the following essential guidelines in NTN-TN scenario and use them as baseline to study corresponding RRM requirements.

- UE isn’t required to measure at TN frequencies if it is outside the TN coverage.

- UE shall prioritize TN frequencies if it is located in the both NTN/TN coverage.

Proposal 7: Priority (explicit or implicit) on TN cell in  NTN/TN mobility requests differentiation between cell reselection from NTN to TN and cell reselection from TN to NTN. How to prioritize TN cell in the two cases shall be studied.

The UE may have to change to the TN cell upon acquiring information about the TN coverage signaled by TN/NTN network node. However, the TN coverage information can’t guarantee that in practice the UE always detect a TN cell e.g. due to complex deployment and environment scenario, e.g. macro cell, micro cell, urban scenario, rural scenario. Depending on coarse/fine accuracy of TN coverage, the UE may perform unnecessary mobility operations, e.g. unnecessary measurements/missing measurements/ping pong between TN/NTN, which isn’t expected.

Proposal 8:  Definition of assistance information indicating TN cell coverage is up to RAN2. But in practice the assistance information may not always correctly reflect the TN cell coverage. Consequently, measurement optimization/robustness to deal with inaccuracy of coverage information should be studied further in RAN4. At the least, two kinds of approaches should be studied, e.g. for the case of UE mobility from NTN to TN:

- UE can try measurement when UE’s close to TN cell coverage indicated by network and stop measurement for a while if no TN cell is detected, or

- UE can perform relaxed measurement when UE’s close to TN cell coverage indicated by network and switch to normal measurement if TN cell is detected.

### Issue 5-3: NTN-NTN handover (to reduce the signalling overhead)

Regarding NTN-NTN enhancements, we observed some ongoing activities in RAN2, e.g. reduction of signaling, RACH-less HO in connected mode and  reference location in Earth moving cells, feeder link switch scenario in idle mode. These new features and aspects in mobility shall be studies from RRM perspective in our view.

Proposal 9: RAN4 work depends on the progress and outcome of RAN2 on NTN-NTN  mobility enhancements. Some of the following procedures may require new RRM requirements or may affect the existing RRM requirements depending on the RAN2 agreements.

- HO chain

- RACH-less HO

- Reselection for Earth moving cells

- feeder link switch scenario

When UE measures downlink signals in the new cell taking over as the cell serving the area, and during measurement gaps, no data scheduling in the serving cell is allowed. A UE may be configured to perform such measurements and report to the serving gNB so that the serving gNB can trigger a handover to the new cell when appropriate. More preferably, however, the UE is configured with a CHO anticipating the switch from an old to a new NTN cell (i.e. the candidate target cell of the CHO configuration is the new cell), and then the UE has to measure on the downlink signals in the new cell in order to monitor the execution condition of the CHO configuration. However, these downlink signals in the new cell are available only when the cell has been “turned on or appeared”, i.e., in practice during the period of coexistence of the old and the new cell, and thus the UE cannot measure (at least not successfully) on these downlink signals during at other times than during the coexistence period of the old and the new cell.

Proposal 10: RAN4 shall further study how to avoid redundant/pre-mature measurements in connected mode with support of satellite assistance information.

New issue： NTN-NTN reselection

In NTN Rel-17, below agreements indicate UE shall start at least before Ttrigger, otherwise, no intra-frequency or inter-frequency detection, measurement or evaluation requirements shall be followed by UE.

If [serving cell service time information] is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location is met [or the legacy Srxlev/Squal condition are met], and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger. Ttrigger = max(Tdetect,NR_Intra, Kcarrier* Tdetect,NR_Inter), where - Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell, - Tdetect,NR_Intra refers to HST intra-frequency cell detection delay in IDLE/INACTIVE mode defined Table Table 4.2C.2.3-2, - Tdetect,NR_Inter refers to HST inter-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2C.2.4-2.

From practical reselection perspective, if UE can detect a qualified neighbor cell before Tservice, then UE shall be able to execute reselection procedure. The total time period of Ttrigger is tremendous specially for long DRX cycle configuration or more than one inter-frequency layer which shall be measured, one of consequences is even a neighbor cell is measured, the signal strength of the neighbor cell may change after Ttrigger. It further causes increment of failure rate of reselection.

How to shorten the necessary Ttrigger shall be studied with respect to the above issue. There are two essential directions to alleviate Ttrigger: one is shortening all Tdetect , e.g. using SMTC even DRX is configured; the other one is adding conditions with respect to the known(e.g. successfully detected) cells.

Observation 8: Long Ttrigger may cause increment of failure rate of reselection, how to avoid no any measurement requirement under the framework of Ttrigger shall be studied.

Proposal 11: To improve efficiency of measurements for reselection, additional associated conditions can be added under the framework of Ttrigger:

- If time span from broadcast time of T-Service to the time instant of T-Service is longer than Ttrigger with Kcarrier=[1 or the number of [known] cell], and there are one or more than one inter-frequency cell is [known], then the measurement requirement to inter-frequency is valid.

- If time span from broadcast time of T-Service to the time instant of T-Service is longer than Ttrigger with Kcarrier=0, and at least one intra-frequency cell is [known], then the measurement requirement to intra-frequency is valid.

# Conclusion

Observation 1: Coverage enhancement has no impact on RRM.

Observation 2: In release 17 a UE GNSS position accuracy of 50 m was assumed together with a 30 m position error for the satellite. This led to the exclusion of SCS = 60 kHz for FR1.

Observation 3: With current release-17 GNSS position inaccuracies for UE and satellite (50 m and 30 m, respectively), then SCS = 120 kHz would result in zero margin for GNSS error.

Observation 4: A total positioning error of 37 meters (23 meters UE GNSS and 14 meters of satellite position error) would suffice for UE UL SCS = 60 kHz, corresponding to Te_NTN = 11 Ts.

Observation 5: ECP and UL SCS = 60 kHz work with existing 50 m UE position error and 30 m satellite position error.

Observation 6:  Network verified UE positioning in NTN is expected to exploit multiple RTT measurements performed on PRS resource(s) transmitted from the same satellite over different time occasions.

Observation 7: Frequently encountering cell changes in NTN (LEO deployment) in multi-RTT measurement procedure isn’t corner case.

Observation 8: Long Ttrigger may cause increment of failure rate of reselection, how to avoid no any measurement requirement under the framework of Ttrigger shall be studied.

Proposal 1: For above 10 GHz bands, mobility between 10 GHz bands ( e.g., intra-frequency and inter-frequency) is supported. Mobility between FR1 and 10 GHz bands in NTN isn’t covered by the WI.

Proposal 2: Table 1 below provides high level overview of existing RRM requirements which are possibly impacted due to the NTN operation in the 10 GHz bands. Note: the reference for comparison is existing NTN requirements in Rel-17.

Proposal 3: RAN4 shall study RRM requirements on UE Rx-Tx time difference measurements specified by single satellite based multi-RTT solution in NTN, e.g. measurement period requirements, report mapping.

Proposal 4: Study multi-RTT measurement requirements in NTN compatible with cell change, e.g. HO.

Proposal 5: RAN4 shall revisit the applicability of mandatory assistance information of neighbor NTN cells ( e.g. ephemeris data, frequency layer and PCI) to a serving NTN cell in NTN-TN scenario. The information is beneficial but shall not be mandatory to mobility from TN to NTN.

Proposal 6: RAN4 study shall comply with the following essential guidelines in NTN-TN scenario and use them as baseline to study corresponding RRM requirements.

- UE isn’t required to measure at TN frequencies if it is outside the TN coverage.

- UE shall prioritize TN frequencies if it is located in the both NTN/TN coverage.

Proposal 7: Priority (explicit or implicit) on TN cell in  NTN/TN mobility requests differentiation between cell reselection from NTN to TN and cell reselection from TN to NTN. How to prioritize TN cell in the two cases shall be studied.

Proposal 8:  Definition of assistance information indicating TN cell coverage is up to RAN2. But in practice the assistance information may not always correctly reflect the TN cell coverage. Consequently, measurement optimization/robustness to deal with inaccuracy of coverage information should be studied further in RAN4. At the least, two kinds of approaches should be studied, e.g. for the case of UE mobility from NTN to TN:

- UE can try measurement when UE’s close to TN cell coverage indicated by network and stop measurement for a while if no TN cell is detected, or

- UE can perform relaxed measurement when UE’s close to TN cell coverage indicated by network and switch to normal measurement if TN cell is detected.

Proposal 9: RAN4 work depends on the progress and outcome of RAN2 on NTN-NTN  mobility enhancements. Some of the following procedures may require new RRM requirements or may affect the existing RRM requirements depending on the RAN2 agreements.

- HO chain

- RACH-less HO

- Reselection for Earth moving cells

- feeder link switch scenario

Proposal 10: RAN4 shall further study how to avoid redundant/pre-mature measurements in connected mode with support of satellite assistance information.

Proposal 11: To improve efficiency of measurements for reselection, additional associated conditions can be added under the framework of Ttrigger:

- If time span from broadcast time of T-Service to the time instant of T-Service is longer than Ttrigger with Kcarrier=[1 or the number of [known] cell], and there are one or more than one inter-frequency cell is [known], then the measurement requirement to inter-frequency is valid.

- If time span from broadcast time of T-Service to the time instant of T-Service is longer than Ttrigger with Kcarrier=0, and at least one intra-frequency cell is [known], then the measurement requirement to intra-frequency is valid.

# References

- RP-223534 ‘Revised WID: NR NTN (Non-Terrestrial Networks) enhancements’ Thales

- R1-2213001 ‘Reply LS on RACH-less handover in NTN’, (R2-2211017), Oppo