3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305309

# Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	UE UL 256QAM

Agenda Item:	5.7.2

Document for:	Approval

- Introduction

The support of FR2 UL 256QAM was discussed in previous meetings, and latest WF was approved in [1].

In this contribution, we provide some discussions on the open issues.

- Discussion

2.2 minimum EIRP requirements for EVM test

Using QPSK as reference, the UE RIRP for 16 QAM is relaxed by 3 dB  and 7 dB for 64 QAM. Since we had the agreement on requirement for 256 QAM, the difference in dB can be found in Table 2.3-1.

Table 2.3-1

| Parameter | EVM level (%) | EVM level (dB) | Delta (dB) |
| --- | --- | --- | --- |
| QPSK | 17.5 | -15.1 | 0 |
| 16QAM | 12.5 | -18.1 | -3 |
| 64QAM | 8 | -21.9 | -6.8 |
| 256QAM | 3.5 | -29.1 | -14 |

Hence, the minimum EIRP for UL 256 QAM for EVM test could be relaxed by 14 dB based on the minimum output power for different PCs. The parameters for PC 1 in FR2-1 is shown in table 2.3-2 as an example.

Table 2.3-2: Parameters for Error Vector Magnitude for power class 1 in FR2-1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | 4 |
| UE EIRP for UL 16 QAM | dBm | 7 |
| UE EIRP for UL 64 QAM | dBm | 11 |
| UE EIRP for UL 256 QAM | dBm | 18.2 |
| Operating conditions |  | Normal conditions |

At last meeting, some companies proposed to consider also a correction factor addtion to“-1dB/dB” relation, as shown in table below.

| Parameter | Unit | PC1 | PC2 | PC5 |
| --- | --- | --- | --- | --- |
| UE EIRP | dBm | 4 | -13 | -6 |
| UE EIRP for UL 256 QAM | dBm | 19.5 | 2.5 | 9.5 |

As discussed in [3], to consider the thermal noise rise and phase noise in FR2 TX, a correction factor of 1.3 dB is assumed.

Proposal 1: the minimum EIRP for UL 256 QAM for EVM test is relaxed by 8.5 dB compared to 64QAM, based on the minimum output power for different PCs

| Parameter | Unit | PC1 | PC2 | PC5 |
| --- | --- | --- | --- | --- |
| UE EIRP | dBm | 4 | -13 | -6 |
| UE EIRP for UL 256 QAM | dBm | 19.5 | 2.5 | 9.5 |

- Conclusion

In this contribution, we provide some consideration on FR2 UL 256QAM.

Proposal 1: the minimum EIRP for UL 256 QAM for EVM test is relaxed by 8.5 dB compared to 64QAM, based on the minimum output power for different PCs

| Parameter | Unit | PC1 | PC2 | PC5 |
| --- | --- | --- | --- | --- |
| UE EIRP | dBm | 4 | -13 | -6 |
| UE EIRP for UL 256 QAM | dBm | 19.5 | 2.5 | 9.5 |

- Reference

[1] R4-2303709, “WF on FR2-1 UL 256QAM”, Xiaomi

[2] R4-2301433, “Discussion on UE UL 256QAM”, Huawei, HiSilicon

[3] R4-2212370, “Discussion on minimum UE EIRP for UL 256QAM”, Apple.