3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2305387

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon, Spark NZ

Title: 	Discussion on RF architecture and potential solution for CA_n5-n105

Agenda Item:	5.4.2.1

Document for:	Approval

Introduction

A new SID [1] for enhancement for sub-1GHz band combinations was approved in RAN #99 meeting. One of these outstanding low band combinations is CA_n5A-n105. In this paper, we discuss the potential RF architecture and identify some RF impacts, e.g. MSD.

Discussion on RF architecture

The frequency ranges of band n5 and n105 are shown below.

Table 1 frequency ranges of CA_n5-n105

| Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Referring to the outcomes of TR 38.872 for other low band combinations, the candidate UE RF architectures are with two antennas and three antennas. For CA_n5-n105, we can use the similar methods.

For UE RF architecture with two antennas shown in figure 1 and 2, quadplexer or triplexer which may cover both 600MHz and 850MHz frequency ranges are needed.

Figure 1 Potential UE RF architecture for CA_n5-n105 with 2 antennas (a)

Figure 2 Potential UE RF architecture for CA_n5-n105 with 2 antennas (b)

In addition, there are two potential implementation challenges with 2-antenna architectures. The first challenge is that the antenna design needs to cover both 600MHz and 850MHz simultaneously as shown in Figure 1 for either of the variants. Two close resonant points in low band antenna may cause some difficulties for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination is expected to be compromised.

The second challenge is on the feasibility of a quadplexer design for variant (a) to aggregate four closely spaced spectrum ranges, not only with sufficient filter isolation between self-band and cross-band but also with acceptable insertion loss. The variant (b) though avoids the use of quadplexer, the design of the intended triplexer is still relatively challenging considering the closely spaced spectrum ranges.

Observation 1: UE RF architectures for CA_n5-n105 with two antennas may face some difficulties, such as wideband low frequency antenna, complex quadplexer or triplexer design. However further investigations may be needed.

Alternatively, UE RF architecture with three antennas for CA_n5-n105 can be considered as below.

Figure 3 Potential UE RF architecture for CA_n5-n105 with 3 antennas

For two main path antennas, only one low band need to be covered and there is no need to develop new multiplexer components. Thus, this kind of implementation can be considered as baseline.

Proposal 1: Pending further investigation of the two antennas case, for CA_n5-n105, UE RF architecture with three antennas can be assumed as baseline for further evaluation.

Discussion on MSD

1) For one UL case:

Since the frequency ranges of band n5 and n105 are very close, there is no MSD issue due to harmonic or harmonic mixing interference.

For MSD due to cross band isolation, the performance of band n5 and n105 duplexers need to be checked and UE RF architecture with three antennas is assumed.

Referring to the contribution [2], the band n105 duplexer can provide 35~40 dB attenuation in Rx frequency range of band n5. In addition, 10dB antenna isolation can be assumed. There is no MSD due to cross band isolation (n105 Tx => n5Rx).

The band n5 duplexer can provide 25~35 dB attenuation in Rx frequency range of band n105. In addition to 10dB antenna isolation, FFS the MSD due to cross band isolation (n5 Tx => n105Rx) based on the attenuation assumption.

Observation 2: For CA_n5-n105, there is no MSD issue due to harmonic or harmonic mixing interference. There is no MSD due to cross band isolation (n105 Tx => n5Rx). FFS the MSD due to cross band isolation (n5 Tx => n105Rx) based on the attenuation assumption.

2) For two UL case:

The Band n5 and Band n105 UL IMD products are shown as below.

Table 2 Band n5 and Band n105 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 121 | 186 | 1487 | 1552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 477 | 582 | 945 | 1035 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2150 | 2255 | 2311 | 2401 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1140 | 1285 | 1769 | 1884 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2813 | 2958 | 3135 | 3250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 372 | 242 | 2974 | 3104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2733 | 2593 | 1988 | 1803 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1066 | 461 | 291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3959 | 4099 | 3476 | 3661 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3798 | 3953 | 3637 | 3807 |

Based on Table 2, there is no MSD issue due to IMD interference when both Band n5 and Band n105 transmit the UL signals for CA_n5-n105.

Observation 3: there is no MSD issue due to IMD interference when both Band n5 and Band n105 transmit the UL signals for CA_n5-n105.

- Summary

Observation 1: UE RF architectures for CA_n5-n105 with two antennas may face some difficulties, such as wideband low frequency antenna, complex quadplexer or triplexer design. However further investigations may be needed.

Proposal 1: Pending further investigation of the two antennas case, for CA_n5-n105, UE RF architecture with three antennas can be assumed as baseline for further evaluation.

Observation 2: For CA_n5-n105, there is no MSD issue due to harmonic or harmonic mixing interference. There is no MSD due to cross band isolation (n105 Tx => n5Rx). FFS the MSD due to cross band isolation (n5 Tx => n105Rx) based on the attenuation assumption.

Observation 3: there is no MSD issue due to IMD interference when both Band n5 and Band n105 transmit the UL signals for CA_n5-n105.

- References

[1] RP-230789, Study on enhancement for sub-1GHz NR band combinations, Spark NZ, Huawei, HiSilicon

[2] R4-2111045, “TP to TR 38.860: B1 full band filter feasibility analysis”, Huawei