3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2305389

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF architecture and potential solution for CA_n26-n28

Agenda Item:	5.4.2.3

Document for:	Approval

Introduction

New SID [1] for enhancement for sub-1GHz band combinations was approved in RAN #99 meeting. One of these outstanding low band combinations is CA_n26-n28. In addition, DL_n26A-n28A_UL_n28A has been completed in RAN4#106 meeting referring to [2]. In this paper, we’d like to discuss the potential RF architecture and identify some RF impacts, e.g. MSD, for other UL configurations.

Discussion on RF architecture

The frequency ranges of band n26 and n28 are shown below.

Table 1 frequency ranges of CA_n26-n28

| Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

Referring to the outcomes of TR 38.872 for other low band combinations, the candidate UE RF architectures are with two antennas and three antennas. For CA_n26-n28, we can use the similar methods.

For UE RF architecture with two antennas shown in figure 1 and 2, quadplexer or triplexer which may cover both 700MHz and 850MHz frequency ranges are needed. Currently, dual-duplexer RF architecture is assumed for band n28, i.e. n28A and n28B duplexers. Logically, vendors need to implement dual-quadplexer or dual-triplexer to support both n28A and n28B.

Figure 1 Potential UE RF architecture for CA_n26-n28 with 2 antennas (a)

Figure 2 Potential UE RF architecture for CA_n26-n28 with 2 antennas (b)

In addition, there are two potential implementation challenges with 2-antenna architectures. The first challenge is that the antenna design needs to cover the entire spectrum range of almost 200 MHz simultaneously as shown in Figure 1 for either of the variants, which would far exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination is expected to be compromised.

The second challenge is on the feasibility of a quadplexer design for variant (a) to aggregate four closely spaced spectrum ranges, not only with sufficient filter isolation between self-band and cross-band but also with acceptable insertion loss. The variant (b) though avoids the use of quadplexer, the design of the intended triplexer is still relatively challenging considering the closely spaced spectrum ranges.

Observation 1: UE RF architectures for CA_n26-n28 with two antennas may face some difficulties, such as wideband low frequency antenna, complex quadplexer or triplexer design.

Alternatively, UE RF architecture with three antennas for CA_n26-n28 can be considered as below.

Figure 3 Potential UE RF architecture for CA_n26-n28 with 3 antennas

For two main path antennas, only one low band need to be covered and there is no need to develop new multiplexer components. Thus, this kind of implementation can be considered as baseline.

Proposal 1: For CA_n26-n28, UE RF architecture with three antennas can be assumed as baseline for further evaluation.

Discussion on MSD

1) For one UL case:

Since the frequency ranges of band n26 and n28 are very close, there is no MSD issue due to harmonic or harmonic mixing interference.

For MSD due to cross band isolation, it’s concluded that there is no MSD due to cross band isolation (n28 Tx => n26 Rx), but band n28 Rx may suffer from severe interference from band n26 UL transmitter.

Observation 2: For CA_n26-n28, there is no MSD issue due to harmonic or harmonic mixing interference. There is no MSD due to cross band isolation (n28 Tx => n26Rx).

After checking datasheets related to band n26 and n28 duplexer, band n28B Rx filter can only provide about 12dB attenuation at Tx frequency range of band n26, so Rx path of band n28 may suffer from the blocking interference. On the other hand, DC_18_n28 has been specified in current spec, and to further check whether this similar MSD values can be reused for CA_n26-n28.

Proposal 2: to further check whether the MSD specified for DC_18_n28 can be reused for CA_n26-n28 (n26 Tx => n28Rx).

2) For two UL case:

The Band n26 and Band n28 UL IMD products are shown as below.

Table 2 Band n26 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 146 | 66 | 1517 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 880 | 995 | 557 | 682 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2331 | 2446 | 2220 | 2345 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1694 | 1844 | 1260 | 1430 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3145 | 3295 | 2923 | 3093 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 132 | 292 | 3034 | 3194 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2178 | 1963 | 2693 | 2508 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 616 | 411 | 1141 | 946 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3626 | 3841 | 3959 | 4144 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3737 | 3942 | 3848 | 4043 |

Based on Table 2, 3rd order IMD may also fall into Rx frequencies of bands n26 when both Band n26 and Band n28 transmit the UL signals. Further MSD evaluation is needed based on the assumed UE RF architecture for CA_n26-n28.

Observation 3: 3rd order IMD may fall into Rx frequencies of bands n26 when both Band n26 and Band n28 transmit the UL signals. Further MSD evaluation is needed based on the assumed UE RF architecture for CA_n26-n28.

- Summary

Observation 1: UE RF architectures for CA_n26-n28 with two antennas may face some difficulties, such as wideband low frequency antenna, complex quadplexer or triplexer design.

Proposal 1: For CA_n26-n28, UE RF architecture with three antennas can be assumed as baseline for further evaluation.

Observation 2: For CA_n26-n28, there is no MSD issue due to harmonic or harmonic mixing interference. There is no MSD due to cross band isolation (n28 Tx => n26Rx).

Proposal 2: to further check whether the MSD specified for DC_18_n28 can be reused for CA_n26-n28 (n26 Tx => n28Rx).

Observation 3: 3rd order IMD may fall into Rx frequencies of bands n26 when both Band n26 and Band n28 transmit the UL signals. Further MSD evaluation is needed based on the assumed UE RF architecture for CA_n26-n28.

- References

[1] RP-230789, Study on enhancement for sub-1GHz NR band combinations, Spark NZ, Huawei, HiSilicon

[2] R4-2303576, “TP for TR 38.718-02-01: CA_n26-n28”, MediaTek Inc., Telstra