3GPP TSG-RAN WG4 Meeting#106bis                             R4-2305408

E-meeting, April 17 – April 26,2023

Source:	ZTE Corporation

Title:	TP for TS 36.181: Clause 7.4 and 10.5

Agenda Item:	6.8.3.5

Document for:	Approval

- Introduction

- This contribution provides a text proposal to TS36.181 [1] on sub-clause 7.4 and 10.5.

- Reference

- R4-2302451, TS 36.181 SAN conformance testing skeleton and work-split arrangement, Huawei, approved.

- Text Proposal

----- Start of TP -----

7.4	In-band sensitivity and blocking

7.4.1	Adjacent Channel Selectivity (ACS)

7.4.1.1	Definition and applicability

Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at TAB connector for SAN type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

7.4.1.2	Minimum requirement

The minimum requirement for SAN type 1-H is in TS 36.108 [2], clause 7.4.1.2.

7.4.1.3	Test purpose

The test purpose is to verify the ability of the SAN receiver filter to suppress interfering signals in the channels adjacent to the wanted channel.

7.4.1.4	Method of test

7.4.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see clause 4.9.1.

SAN RF Bandwidth positions to be tested for multi-carrier (MC):

-	MRFBW for single-band connector(s), see clause 4.9.1

7.4.1.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For SAN type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.3 for SAN type 1-H.

2)	For FDD operation, set the SAN to transmit:

-	For single carrier operation set the connector under test to transmit at manufacturers declared rated carrier output power (Prated,c,TABC, D.34).

-	For a connector under test declared to be capable of multi-carrier  operation (D.39) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2

3)	Set the signal generator for the wanted signal to transmit as specified in table 7.4.1.5-1 for SAN supporting E-UTRA, table 7.4.1.5-2 for SAN supporting standalone NB-IoT.

4)	Set the signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.4.1.5-1 for SAN supporting E-UTRA, table 7.4.1.5-2 for SAN supporting standalone NB-IoT.

7.4.1.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SAN supporting E-UTRA, the wanted and the interfering signal coupled to the SAN type 1-H  TAB connector are specified in table 7.4.1.5-1. The reference measurement channel for the wanted signal is identified in table 7.2.2-1 for each SAN channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex C.

For SAN supporting standalone NB-IoT operation, the wanted and the interfering signal coupled to the SAN type 1-H  TAB connector are specified in table 7.4.1.5-2. The reference measurement channel for the wanted signal is identified in table 7.2.2-1 for each SAN channel bandwidth in any operating band and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex C.

The ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Minimum conducted requirement is defined at the TAB connector for SAN type 1-H.

Table 7.4.1.5-1: ACS requirement of SAN supporting E-UTRA

| SAN channel bandwidth of the lowesthighest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 11dB (Note) | GEO SAN class: -57.6 LEO SAN class: -60.7 | ±0.7025 | 1.4MHz E-UTRA signal |
| Note: PREFSENS depends on the channel bandwidth as specified in Table 7.2.2-1 and Table 7.2.2-2. |  |  |  |  |

Table 7.4.1.5-2: ACS requirement of SAN supporting standalone NB-IoT operation

| SAN channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB (Note) | GEO SAN class: -56.6 LEO SAN class: -59.7 | ±100 | 180 kHz NB-IoT signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-3 and Table 7.2.2-4. |  |  |  |  |

7.4.2	In-band blocking

The requirement is not applicable in this version of the specification.

----- Next of TP -----

10.5	OTA in-band selectivity and blocking

10.5.1	OTA adjacent channel selectivity

10.5.1.1	Definition and applicability

OTA Adjacent channel selectivity (ACS) is a measure of the receiver's ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

10.5.1.2	Minimum requirement

For SAN type 1-O, the minimum requirement is in TS 36.108 [2], clause 10.5.1.2.

10.5.1.3	Test purpose

The test purpose is to verify the ability of the SAN receiver filter to suppress interfering signals in the channels adjacent to the wanted channel.

10.5.1.4	Method of test

10.5.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-	M; see clause 4.9.1.

SAN RF Bandwidth edge position to be tested for multi-carrier :

-	MRFBW in single-band operation, see clause 4.9.1;

Directions to be tested:

-	For SAN type 1-O, receiver target reference direction (D.26),

10.5.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.4.2.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Align the SAN with the test antenna in the declared direction to be tested.

4)	Align the SAN so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	For FDD operation, set the SAN to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in clauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the SAN Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 10.5.1.5-1 for SAN supporting E-UTRA and  table 10.5.1.5-2 for SAN supporting standalone NB-IoT.

b)	Set the signal generator for the interfering signal at the adjacent channel frequency of the wanted signal to transmit as specified in table 10.5.1.5-1 for SAN supporting E-UTRA and  table 10.5.1.5-2 for SAN supporting standalone NB-IoT.

8)	Measure throughput according to annex A.1 for each supported polarization, for multi-carrier operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clauses 4.7.2 and 4.8.

10.5.1.5	Test requirement

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SAN supporting E-UTRA, the OTA wanted signal and the interfering signal are specified in table 10.5.1.5-1 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristic of the interfering signal is further specified in annex C.

For SAN supporting standalone NB-IoT operation, the OTA wanted signal and the interfering signal are specified in table 10.5.1.5-2 for OTA ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.1. The characteristic of the interfering signal is further specified in annex C.

The OTA ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Table 10.5.1.5-1: ACS requirement of SAN supporting E-UTRA

| SAN channel bandwidth of the lowesthighest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | PREFSENS + 11dB- ΔOTAREFSENS (Note) | GEO SAN class: -57.6- ΔOTAREFSENS LEO SAN class: -60.7- ΔOTAREFSENS | ±0.7025 | 1.4MHz E-UTRA signal |
| Note: PREFSENS depends on the channel bandwidth as specified in Table 7.2.2-1 and Table 7.2.2-2. |  |  |  |  |

Table 10.5.1.5-2: ACS requirement of SAN supporting standalone NB-IoT operation

| SAN channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB - ΔOTAREFSENS(Note) | GEO SAN class: -56.6- ΔOTAREFSENS LEO SAN class: -59.7- ΔOTAREFSENS | ±100 | 180 kHz NB-IoT signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-3 and Table 7.2.2-4. |  |  |  |  |

10.5.2	OTA in-band blocking

The requirement is not applicable in this version of the specification.

----- End of TP -----