3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305425
Online, April 17 – April 26, 2023

Agenda item:	5.31.2.1.2

Source:	OPPO

Title:	R18 SL-U single CC Tx requirements

Document for:	Approval

- Introduction

Rel-18 NR SL evolution WI was approved with latest WID in [1] and the work in RAN4 starts from this meeting. In this paper, the RF impacts will be discussed.

- Discussion

2.1 MOP and MIMO/TxD

It was agreed PC5 (20dBm) is the default power class and FFS on other power classes (PC1/2/3) in last meeting. Reason behind is considering the power class supported in NR-U, reusing WIFI PAs to reduce the UE implementation costs, and reducing the potential high standardization complexity of the coexistence analysis between high power SL-U and NR-U/WIFI. Therefore, RAN4 should focus on completing PC5 before the introduction of other power classes.

Proposal 1:         RAN4 focus on completing PC5 before the introduction of other power classes.

UE can meet PC5 easily with single WIFI PA, in this case, there is no need to further consider TxD for SL-U in Rel-18.

Then for SL-MIMO it can be supported to increase Tx throughputs.

Proposal 2:         TxD is not considered in Rel-18 SL-U, and SL-MIMO can be considered.

Besides, there are also transmit power density requirements in NR-U as shown in figure 1 which are from regulations. SL-U UE should also follow such transmit power density restrictions.

Figure 1 NR-U transmit power density requirements

Proposal 3:         Transmit power density requirements defined in NR-U shall be followed by SL-U UE.

2.2 Pcmax

Pcmax for NR-U is same as legacy NR, and there is no difference in Pcmax definition for SL-U and current V2X especially PRegulatory,c is additionally considered [16] comparing to NR single CC. The Pcmax of current V2X can be reused.

Proposal 4:         Reuse current Pcmax for V2X in clause 6.2E.4.1 for SL-U.

2.3 Min output power

The min power is defined differently from NR single CC requirement, take 20MHz CBW as example, the NR V2X min power is defined as -30dBm (figure 2), but NR uu is -40dBm (figure 3).

Figure 2 NR V2X min power requirements

Figure 3 NR uu min power requirements

This difference is copied from LTE V2X which is triggered by RAN5 LS [2] where it asked RAN4 how to test LTE V2X IBE requirement in low power range with large absolute power tolerance. And then in RAN4 reply LS [3] it changed not only the LTE V2X absolute power tolerance but also changed the LTE V2X minimum output power from original -40dBm to -30dBm. The reason for this change was explained in [4] as shown in figure 4, i.e. to make IBE requirement testable.

Figure 4 Reason of LTE V2X min power change from -40dBm to -30dBm

Then after several meetings in RAN4#90, another CR [5] was agreed to change it back from -30dBm to -40dBm, however, the minimum output power was not changed back. This makes the LTE V2X minimum output power is -30dBm in the LTE spec, and then reused in NR spec.

Observation 1:   The NR V2X minimum output power was reused from LTE V2X. And in LTE V2X, absolute power tolerance was changed smaller to make RAN5 IBE testing feasible when Tx power is low, and the minimum output power was also changed from -40dBm to -30dBm as a side change. Then, in following meetings, the absolute power tolerance was changed back, however, minimum output power was left not changed back.

From above history tracks it can be noticed that the minimum output power difference was just an side change of absolute power tolerance but when the absolute power tolerance was changed back, the minimum output power was overlooked. Then it is suggested to correct this requirement and align with NR uu.

Proposal 5:         Correct NR V2X minimum output power to align with NR uu, i.e. for CBW ≤ 20MHz change it from -30dBm to -40dBm and scale it for higher CBWs (reasons see observation 1).

Considering NR-U also reuses the NR uu minimum output power, then SL-U minimum output power should be defined same as NR.

Proposal 6:         Reuse NR single CC (same as NR-U) minimum output power for SL-U, i.e. -40dBm for CBW ≤ 20MHz and scale it for higher CBWs.

2.4 OFF power

The OFF power for NR V2X is as seen in figure 5, and NR-U reuse the NR single CC requirement shown in figure 6. Basically, the are same. SL-U can reuse the NR single CC requirement as in figure 6.

Figure 5 NR V2X OFF power

Figure 6 OFF power for NR single CC (reused by NR-U)

Proposal 7:         -50dBm OFF power is defined for SL-U which is same as NR single CC, NR-U and NR V2X.

2.5 ON/OFF time mask

Two ON/OFF time masks were defined for NR V2X, i.e. general PSSCH and PSCCH time mask, and S-SSB time mask as shown in figure 7. For SL-U, these two ON/OFF time masks are also needed.

Figure 7 ON/OFF time mask for NR V2X

Comparing to NR V2X, the NR-U considered the case of COT sharing, and defined CP-E to fit for the 16us or 25us transmit gap length requirements in COT sharing. To give more time for UE LBT measurements, the existing10us transient period in the front and 5us transient period in the back were pushed inside the transmit symbols. And another 5us transient period was defined in the front to allow UE ramp-up earlier [6][7][8]. This can be seen in figure 8.

Figure 8 ON/OFF time mask for NR-U

Observation 2:   NR-U ON/OFF time mask considered the case of COT sharing, and defined CP-E to fit for the 16us or 25us transmit gap length requirements in COT sharing. The ON/OFF time mask of NR-U is different from V2X in transient period location and length.

To combine them together, bellow figure 9 and 10 ON/OFF time masks for SL-U are proposed. For the leading transient period, it is defined as 15us including 5us before CP-E and 10us within the CP-E, and for the ending transient period, it is proposed to keep the 10us within the guard period symbol instead of pushing inside the useful symbols for SL-U.

Proposal 8:         It is proposed to adopt figure 9 and 10 ON/OFF time mask for SL-U with transient period same as NR-U in the front, and same as V2X in the end.

Figure 9 General ON/OFF time mask for SL-U PSSCH and PSCCH

Figure 10 ON/OFF time mask for SL-U S-SSB

2.6 IBE

The IBE requirement for NR V2X is reusing the NR single CC requirements as shown in figure 11, and the NR-U IBE requirement is different from NR single CC in the general requirement as shown in figure 12. The NR-U IBE requirement is reusing the eLAA IBE requirement according to [9][10] considering the interlaced RB allocations. For SL-U, the general IBE requirement for NR-U can be reused.

Figure 11 NR single CC and V2X IBE

Figure 12 NR-U IBE

Observation 3:   NR-U reuse eLAA general IBE requirement considering the interlaced RB allocations in unlicensed spectrums.

Proposal 9:         It is proposed to reuse the general IBE requirement of NR-U for SL-U. And the IQ image/Carrier leakage of NR-U can also be reused since it is same as NR single CC and NR V2X.

2.7 SEM/ASEM

The SEM definition is different for NR V2X or NR single CC to NR-U as can be seen in below figure 13 and figure 14. The difference coming from NR-U protection of WIFI and LTE-LAA existing systems in unlicensed spectrums and regulations in EU [11][12][13]. For SL-U, it also needs to follow these protection requirements in unlicensed spectrum.

Figure 13 NR V2X and NR single CC SEM

Figure 14 NR-U SEM

Proposal 10:       Reuse the NR-U SEM requirements for SL-U due to same protection requirements to existing systems like WIFI and LTE-LAA.

For ASEM, there is no specific requirements defined for NR-U in current spec for bands n46/n96/n102, therefore no specific ASEM is requirement for SL-U either.

Proposal 11:       No ASEM is needed for SL-U in bands n46/n96/n102 which is same as current NR-U.

2.8 ACLR

NR-U ACLR was defined for power class 5 with 27dB [14], and ACLR2 was defined for n46 in Japan [15]. These can also be reused for SL-U.

Proposal 12:       NR-U ACLR can be reused for SL-U with power class 5 in bands n46/n96/n102.

2.9 UE coexistence

For the UE coexistence, there is no requirement defined for the UE works in unlicensed bands in standalone mode. This is following the general handling of LAA and WIFI as described in [6] copied below:

“The NR-U bands are unlicensed they do not necessarily follow the same conventions as licensed bands.  Licensed bands, for example, are not required to provide incoming coexistence protection to LAA Band 46.  Moreover, other technologies such as WiFi using the unlicensed bands do not provide any outgoing coexistence protection to other 3GPP bands.  It is therefore proposed that UE coexistence requirements, either incoming to protect the NR-U bands or outgoing to protect other 3GPP bands, are not required with respect to the NR-U standalone bands.  Instead, the general spurious emission requirements apply.  Carrier aggregation or dual connectivity combinations including a licensed band would be obligated to protect other 3GPP band in the same way as eLAA with respect to the licensed band.”

Therefore, for the SL-U the UE coexistence requirements also doesn’t need to be defined for the standalone mode. For the uu+SL concurrent transmission, it only needs to follow the NR uu coexistence protection.

Proposal 13:       UE coexistence requirement is not defined for SL-U standalone working mode following the same principle of NR-U, i.e. it doesn’t need to provide protection to other bands or receiving protection from them.

2.10 ASE

There are NR-U ASE requirements defined, for n46 the NS_28, NS_29, NS_30 and NS_31 apply, for n96 the NS_53/54/60 and NS_61 apply, for n102 the NS_58 apply. There requirements may also need to be considered in SL-U.

Observation 4:   ASE may need to be defined for SL-U.

2.11 MPR/AMPR

MPR and AMPR requirements need to be discussed further after the relevant general requirements are determined.

Observation 5:   MPR and AMPR will be discussed after the relevant general requirements are determined.

2.12 other requirements with no difference

Power control, Freq Error, EVM, Carrier leakage, CBW, General SE and Tx intermodulation, they are all same among NR single CC, NR V2X and NR-U. Same requirements can also be applied to SL-U.

Proposal 14:       The NR single CC requirements of Power control, Freq Error, EVM, Carrier leakage, CBW, General SE and Tx intermodulation can be reused for SL-U considering there is no difference among NR V2X and NR-U.

- Conclusions

In this contribution, the Tx requirements for Rel-18 NR SL evolution are analysed and got the following observations and proposals.

2.1 MOP and MIMO/TxD

Proposal 1:         RAN4 focus on completing PC5 before the introduction of other power classes.

Proposal 2:         TxD is not considered in Rel-18 SL-U, and SL-MIMO can be considered.

Proposal 3:         Transmit power density requirements defined in NR-U shall be followed by SL-U UE.

2.2 Pcmax

Proposal 4:         Reuse current Pcmax for V2X in clause 6.2E.4.1 for SL-U.

2.3 Min output power

Observation 1:   The NR V2X minimum output power was reused from LTE V2X. And in LTE V2X, absolute power tolerance was changed smaller to make RAN5 IBE testing feasible when Tx power is low, and the minimum output power was also changed from -40dBm to -30dBm as a side change. Then, in following meetings, the absolute power tolerance was changed back, however, minimum output power was left not changed back.

Proposal 5:         Correct NR V2X minimum output power to align with NR uu, i.e. for CBW ≤ 20MHz change it from -30dBm to -40dBm and scale it for higher CBWs (reasons see observation 1).

Proposal 6:         Reuse NR single CC (same as NR-U) minimum output power for SL-U, i.e. -40dBm for CBW ≤ 20MHz and scale it for higher CBWs.

2.4 OFF power

Proposal 7:         -50dBm OFF power is defined for SL-U which is same as NR single CC, NR-U and NR V2X.

2.5 ON/OFF time mask

Observation 2:   NR-U ON/OFF time mask considered the case of COT sharing, and defined CP-E to fit for the 16us or 25us transmit gap length requirements in COT sharing. The ON/OFF time mask of NR-U is different from V2X in transient period location and length.

Proposal 8:         It is proposed to adopt figure 9 and 10 ON/OFF time mask for SL-U with transient period same as NR-U in the front, and same as V2X in the end.

2.6 IBE

Observation 3:   NR-U reuse eLAA general IBE requirement considering the interlaced RB allocations in unlicensed spectrums.

Proposal 9:         It is proposed to reuse the general IBE requirement of NR-U for SL-U. And the IQ image/Carrier leakage of NR-U can also be reused since it is same as NR single CC and NR V2X.

2.7 SEM/ASEM

Proposal 10:       Reuse the NR-U SEM requirements for SL-U due to same protection requirements to existing systems like WIFI and LTE-LAA.

Proposal 11:       No ASEM is needed for SL-U in bands n46/n96/n102 which is same as current NR-U.

2.8 ACLR

Proposal 12:       NR-U ACLR can be reused for SL-U with power class 5 in bands n46/n96/n102.

2.9 UE coexistence

Proposal 13:       UE coexistence requirement is not defined for SL-U standalone working mode following the same principle of NR-U, i.e. it doesn’t need to provide protection to other bands or receiving protection from them.

2.10 ASE

Observation 4:   ASE may need to be defined for SL-U.

2.11 MPR/AMPR

Observation 5:   MPR and AMPR will be discussed after the relevant general requirements are determined.

2.12 other requirements with no difference

Proposal 14:       The NR single CC requirements of Power control, Freq Error, EVM, Carrier leakage, CBW, General SE and Tx intermodulation can be reused for SL-U considering there is no difference among NR V2X and NR-U.

- References

- RP-222806, WID revision: NR sidelink evolution, OPPO, RAN#98-e meeting, Dec 2022.

- R5-181694, LS on RF requirements for V2X UEs

- R4-1805741, LS reply on RF requirements for V2X UEs, RAN4->RAN5, RAN4#86bis, Apr 2018

- R4-1805742, CR on absolute power tolerance for V2X, RAN4#86bis, Huawei, Apr 2018

- R4-1902207, CR on absolut power tolerance for V2X, Huawei, RAN4#90, Feb 2019

- R4-2011345, Remaining UE RF requirements for stand-alone single carrier NR-U, QC, RAN4#96, Aug 2020

- R4-2010344, Additional TX requirements for NR-U operation, Ericsson, RAN4#96, Aug 2020

- R4-2011847, Email discussion summary for [96e][107] NR_unlic_UE_RF, QC, RAN4#96, Aug 2020

- R4-2009165, WF on NR-U PC3 ACLR and in-band emissions, Huawei, RAN4#95e, May 2020

- R4-2008124, NR-U In-band emissions requirement, QC, RAN4#95e, May 2020

- R4-1913800, Emission Mask Considerations for NR-U DL single 20 MHZ channel and wideband carrier operation modes, Charter, RAN4#93, Nov 2019

- R4-1915979, WF on NR-U spectrum emission mask, Nokia, RAN4#93, Nov 2019

- R4-1910535, WF on Emission Requirements for NR-U, Nokia, RAN4#92, Aug 2019

- R4-2002753, WF on remaining UE Tx requirements for NR-U, QC, RAN4#94e, Feb 2020

- R4-2005221, WF on NR-U UE Tx Requirement, Skyworks, RAN4#94bis, Apr 2020

- R4-1805133, Post-antenna connector gain impact on Pcmax and testing, Ericsson, RAN4#86bis, Apr 2018