3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305465

# Online, April 17-April 26, 2023

Title: 	Discussions on 4TX BS demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.4.5.2

Document for:	Discussion

Background

As per [1], RAN4 left many open issues on 4Tx BS demodulation requirements definition. In this paper, we will provide our views on these open issues.

Simulation results

In this section we provide our initial simulation results for some candidate options for bandwidth and MCS. The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions for 4Tx PUSCH requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0, FFS on 4 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

The simulation results are captured in Table 2-2:

Table 2-2: Summary of simulation results for 4Tx PUSCH requirements

| SCS | Bandwidth | Propagation conditions | MCS | Antenna | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 15kHz | 5MHz | TDLA30-10 | 16 | 4T4R | 16.8 |
|  |  |  |  | 4T8R | 8.2 |
|  |  |  | 17 | 4T4R | 17.0 |
|  |  |  |  | 4T8R | 9.0 |
|  |  |  | 20 | 4T4R | 20.6 |
|  |  |  |  | 4T8R | 11.5 |
|  |  | TDLC300-100 | 12 | 4T4R | 12.4 |
|  |  |  |  | 4T8R | 5.1 |
|  |  |  | 16 | 4T4R | 19.9 |
|  |  |  |  | 4T8R | 9.8 |
|  | 20MHz | TDLA30-10 | 16 | 4T4R | 17.6 |
|  |  |  |  | 4T8R | 8.6 |
|  |  |  | 17 | 4T4R | 17.7 |
|  |  |  |  | 4T8R | 9.0 |
|  |  |  | 20 | 4T4R | 23.3 |
|  |  |  |  | 4T8R | 12.2 |
|  |  | TDLC300-100 | 12 | 4T4R | 12.8 |
|  |  |  |  | 4T8R | 5.2 |
|  |  |  | 16 | 4T4R | 19.9 |
|  |  |  |  | 4T8R | 9.9 |
|  | 50MHz | TDLA30-10 | 16 | 4T4R | 18.6 |
|  |  |  |  | 4T8R | 8.9 |
|  |  |  | 17 | 4T4R | 18.6 |
|  |  |  |  | 4T8R | 9.3 |
|  |  |  | 20 | 4T4R | 25.1 |
|  |  |  |  | 4T8R | 12.5 |
|  |  | TDLC300-100 | 12 | 4T4R | 12.8 |
|  |  |  |  | 4T8R | 5.2 |
|  |  |  | 16 | 4T4R | 20.6 |
|  |  |  |  | 4T8R | 10.5 |

Discussions

Bandwidth

One open issue is bandwidth, we have following options:

- For 15kHz: - 5MHz requirement will be introduced - 20MHz and 50MHz for initial simulation, and then choose one for performance requirements definition. - For 30kHz: 10MHz and 100MHz requirement will be introduced

It is open for 15kHz SCS, based on our simulation results above, compared to 20MHz CBW,  there is about 0.3dB degradation for TDLA30-10 and about 0.6~0.7 dB degradation for TDLC300-100 for 50MHz CBW. From the point view of minimum requirements definition, we suggest to use 50MHz.

Proposal 1: For 15kHz SCS, consider 5MHz and 50MHz CBW for performance requirements definition.

MCS configuration

One open issue is MCS derivation, candidate options are captured as follows:

Agreement: Use MCS table 1 for the evaluation. The following MCS can be used as starting point for evaluation. - TDLA30-10: MCS16, MCS17 and MCS20 - TDLB100-400: MCS2 - TDLC300-100: MCS12 and MCS16

It has been agreed to use QPSK for propagation conditions with TDLB100-400, for test coverage, it is more reasonable to choose 64QAM for propagation conditions with TDLA30-10 and 16QAM for propagation conditions with TDLC300-100. Based on our simulation results above, SNR is quite close to 20dB for TDLC300-100 with MCS16, so it is better to consider MCS12 for TDLC300-100. For TDLA30-10, it’s better to consider MCS17 since MCS16 is corresponding to 16QAM and target SNR for MCS20 is quite high.

Proposal 2: Consider following MCS configuration:

- TDLA30-10: MCS17

- TDLB100-400: MCS2

- TDLC300-100: MCS12

Antenna configuration

- For initial simulation: include both 4x4 and 4x8 - For the testing of performance requirement, only the performance for the highest supported Rx is used for testing if agreed to define performance requirements for both 4x4 and 4x8.

For antenna configuration, we prefer to cover 4x4 and 4x8 considering different BS implementation.

Proposal 3: Cover both 4x4 and 4x8 for requirements definition

Precoding

One open issue is precoding. We have following options:

Take TPMI index 0 in Table 6.3.1.5-7 of 38.211 for performance requirement definition. FFS on TPMI index 4 in Table 6.3.1.5-7 of 38.211 for vehicular use cases.

Based on our understanding, there is no performance difference for two precoding matrixes if we use TDL channel model for requirements definition. Beamforming channel model has never been considered in RAN4 demodulation requirements definition. We suggest to reuse the legacy configuration.

Proposal 4: Use following precoding matrixes:

Conclusion

In this paper we provide our views on 4TX BS demodulation requirements. The proposals are:

Proposal 1: For 15kHz SCS, consider 5MHz and 50MHz CBW for performance requirements definition.

Proposal 2: Consider following MCS configuration:

- TDLA30-10: MCS17

- TDLB100-400: MCS2

- TDLC300-100: MCS12

Proposal 3: Cover both 4x4 and 4x8 for requirements definition

Proposal 4: Use following precoding matrixes:

- 4   Reference

- R4-2302926 WF on FR1 4Tx demodulation requirement.  Huawei, HiSilicon