3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305469

# Online, April 17 – April 26, 2023

Title: 	Discussions on demodulation requirements for non-collocated EN-DC/NR-CA deployment

Source: 	Huawei, HiSilicon

Agenda item:	5.12.4

Document for:	Discussion

Background

RAN4 should start to discuss the demodulation requirements for non-collocated EN-DC/NR-CA deployment in this meeting. In this paper we will provide our general views on related demodulation requirements.

Discussions

UE type

UE type which can apply to non-collocated EN-DC/NR CA is still on-going discussion in RF session. Type 1 summaries the architecture of each UE type:

Table 2-1: Summary of architecture of each UE type

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | NRCA/ ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | NRCA ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| [3a] | 1 | 4 shared |  | 4 | 4 | 4Rx | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after 2 LNAs out of 4 => common AGC on LNA => 25dB partial range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |
| [3b] | 1 | 4 shared |  | 4 | 4 | 4Rx | NRCA ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding 2LO/BB/Rx and RF split after all 4 LNAs => common AGC on LNA => 25dB partial range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |
| [4a] | 1 | 4 | 6 total | 4 | 4 | 4Rx | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| [4b] | 1 | 4 | 8 total | 4 | 4 | 4Rx | NRCA ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |

Considering Type 2 has been confirmed as one candidate implementation for non-collocated EN-DC/CA deployment and type 3,4 are still under discussions, it is proposed to take type 2 UE as 1st priority and other UE types depends on the progress of RF discussions.

Proposal 1: Take type 2 UE as 1st priority, other UE types depends on the progress of RF discussion.

Impact on demod parts

Type 2/4 UE have two separate receiving chains for each CC which means there is no interference across the two CCs. From demodulation perspective, there is no difference compared to normal EN/DC and CA deployment. No demodulation requirements should be considered for such type UE.

However, for Type 3 UE, since stronger CC and weaker CC shares one AGC which is always adjusted including both power gain and phase on the slot boundary, the last symbol of weaker CC is distorted due to the misalignment of arrival of two CCs. That can degrade the PDSCH performance which should be considered.

Observation 1: Compared to normal EN-DC/CA deployment, there is no performance impact for type 2/4 UE.

Observation 2: Compared to normal EN-DC/CA deployment, there is large performance impact for type 3 UE.

Hence, we propose the following:

Proposal 2: Don’t define requirements for type 2/4 UE and further discuss how to define requirements for type 3 UE if agreed by RF session.

Conclusion

In this paper we provide our views on demodulation requirements for non-collocated EN-DC/NR-CA deployment. The proposals and observations are:

Proposal 1: Take type 2 UE as 1st priority, other UE types depends on the progress of RF discussion.

Observation 1: Compared to normal EN-DC/CA deployment, there is no performance impact for type 2/4 UE.

Observation 2: Compared to normal EN-DC/CA deployment, there is large performance impact for type 3 UE.

Proposal 2: Don’t define requirements for type 2/4 UE and further discuss how to define requirements for type 3 UE if agreed by RF session.