3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2305557

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Bandwidth part operation without restriction in NR – Option B-1-2

Agenda item:	5.11.2.4

Document for:	Discussion

Introduction

RAN#99 approved a new WID to address the issues on BWP without restriction in [1]. Objectives can be found below:

In this paper we give our input to the discussion related Option B-1-2 and the possible UE, network and system impacts.

Discussion

The solution related to option B-1-2 is based on following assumptions:

- Option B) Perform BM/RLM/BFD based on SSB outside active BWP

  - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

    - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions

And the objective is:

- For Option B-1-2

  - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1):

    - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and

    - UE shall support option (C) NCD-SSB (subject to IoDT availability).

  - The interruption related requirements will be decided and specified in RAN4.

For this solution several conditions have been set:

- In the active BWP of the corresponding carrier(s) to be measured,

  - There is no CSI-RS is configured for RLM/BM/BFD, and

  - There is no NCD SSB is configured for RLM/BM/BFD, and

  - There is no CD SSB is configured for RLM/BM/BFD, and

- UE shall support NCD-SSB

Hence, if either CSI-RS is configured within the active BWP or there is an NCD-SSB within the active BWP, the scenarios are covered by Option-A (CSI-RS is within the active BWP) or Option-C (NCD-SSB is within the active BWP).

This scenario is therefore seen as covering the situation where the UE cannot fulfil the B-1-1 requirement and perform RLM, BFD and BM on the SSB outside the active BWP without causing interruptions.

Our understanding related to B-1-2 is that if the UE cannot perform RLM, BFD and BM on the SSB outside active BWP without causing interruptions, such UE would also either need gaps for performing intra-frequency measurements or cause additional interruptions due to performing intra-frequency measurements.

As RAN4 has defined requirements for gap assisted intra-frequency measurements it does not seem acceptable to allow introduction of interruption based intra-frequency measurements requirements for Option B-1-2. Hence, we conclude that Option B-1-2 would need gaps for performing intra-frequency measurements.

A UE supporting Option B-1-2 need gaps for performing intra-frequency measurements.

For a UE supporting B-1-2, there is a need to have gaps allocated to perform intra-frequency measurements. For B-1-2 to apply, the conditions listed above must be fulfilled. Hence, there is no CSI-RS for performing RLM, BFD or BM within the active BWP. This means measurements for RLM, BFD and BM must be performed on RS outside the active BWP.

Option B-1-2 requires measurements for RLM, BFD and BM to be performed on RS outside the active BWP.

One option is of course that this could be some CSI-RS resource outside the active BWP, but this seems to be a rather unrealistic option and anyway ruled out by the WID. The RS used for RLM, BFD and BM outside the active BWP must be the CD-SSB.

For option B-1-2, The RS outside the active BWP to be sued for RLM, BFD and BM is the CD-SSB.

One option is that the RS is the NCD-SSB. However, using NCD-SSB outside the active BWP would mean following:

- UE would need measurement gaps for performing intra-frequency measurements using the CD-SSB and interruptions for performing RLM, BFD and BM on the NCD-SSB.

- UE would need measurement gaps for performing intra-frequency measurements using the NCD-SSB and interruptions for performing RLM, BFD and BM on the NCD-SSB.

Both options seem rather complex and unattractive from network perspective. However, RAN4 has been tasked to specify support of option B-1-2.

RAN4 has already defined gap assisted intra-frequency measurement gaps based on CD-SSB. The current UE requirements for gap assisted intra-frequency measurement would be readily applicable for this scenario regarding cell detection and measurement requirements.

Using CD-SSB: Current UE requirements for gap assisted intra-frequency measurement can be re-used.

The same UE requirements for gap assisted intra-frequency measurement may likely also be applicable to NCD-SSB. However, as the current requirements are developed assuming CD-SSB is used, RAN4 would need to analyse if any changes and/or clarifications would be necessary to also support NCD-SSB.

Using NCD-SSB: RAN4 would need to analyse if any changes to the current UE requirements for gap assisted intra-frequency measurement when using NCD-SSB.

Additionally, RAN4 would have to analyse and develop UE requirements for allowed interruptions due to performing RLM, BFD and BM. Such requirements are not yet defined.

RAN4 to analyse and develop UE requirements for allowed interruptions due to performing RLM, BFD and BM.

As the support of NCD-SSB is mandatory for the UE when supporting Option B-1-2, but may not be supported by the network, it is clear that RAN4 at least need to develop the necessary requirements for allowed interruptions due to performing RLM, BFD and BM based on CD-SSB.

RAN4 at least need to develop the necessary UE requirements for allowed interruptions due to performing RLM, BFD and BM based on CD-SSB.

RAN4 to decide whether to develop UE requirements for allowed interruptions due to performing RLM, BFD and BM UE requirements based on NCD-SSB.

Conclusion

In this paper we took a further look at bandwidth part operation without restriction in NR – Option B-1-2. We observe following related to option B-1-2:

- A UE supporting Option B-1-2 need gaps for performing intra-frequency measurements.

Option B-1-2 requires measurements for RLM, BFD and BM to be performed on RS outside the active BWP.

For option B-1-2, The RS outside the active BWP to be sued for RLM, BFD and BM is the CD-SSB.

Using CD-SSB: Current UE requirements for gap assisted intra-frequency measurement can be re-used.

And we propose:

- Using NCD-SSB: RAN4 would need to analyse if any changes to the current UE requirements for gap assisted intra-frequency measurement when using NCD-SSB.

- RAN4 to analyse and develop UE requirements for allowed interruptions due to performing RLM, BFD and BM.

RAN4 at least need to develop the necessary UE requirements for allowed interruptions due to performing RLM, BFD and BM based on CD-SSB.

RAN4 to decide whether to develop UE requirements for allowed interruptions due to performing RLM, BFD and BM UE requirements based on NCD-SSB.

- References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Moderator (Vodafone), vivo, Mar. 2023.