3GPP TSG-RAN4 Meeting # 106-bis-e	 R4-2305647

# Electronic Meeting, 17th  –26th  April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | TBD | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for REFSENS for eRedcap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding RF requirements for eRedCap UE in Rel-18 |  |  |  |  |  |  |  |  |
| Summary of change: |  | REFSENS of eRedCap UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No RF requirement for eRedcap UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- START CHANGE ------------------------------------------------------

## 7.3I	Reference sensitivity for RedCap

### 7.3I.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3I.2	Reference sensitivity power level

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE with capability [IE] equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3I.2-1a and Table 7.3I.2-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-1c and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE with capability [IE] equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-1a and Table 7.3I.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-1c and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

Table 7.3I.2-1a: Two antenna port reference sensitivity for eRedcap UE for FDD band

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |
| --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) |
| n1 | 15 | 5,10,15, 20 | -100 |
|  | 30 | 10,15, 20 | -97.1+10log(12/24)= -100.1 |
|  | 60 | 10,15, 20 |  |
| n2 | 15 | 5,10,15,20 | -98 |
|  | 30 | 10,15,20 | -98.1 |
|  | 60 | 10,15,20 |  |
| n3 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
|  | 60 | 10,15,20 |  |
| n5 | 15 | 5,10,15,20 | -98.0 |
|  | 30 | 10,15,20 | -98.1 |
| n7 | 15 | 5,10,15,20 | -98.0 |
|  | 30 | 10,15,20 | -98.1 |
|  | 60 | 10,15,20 |  |
| n8 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
| n12 | 15 | 5,10,15 | -97.0 |
|  | 30 | 10,15 | -97.1 |
| n13 | 15 | 5,10 | -97.0 |
|  | 30 | 10 | -97.1 |
| n14 | 15 | 5,10 | -97.0 |
|  | 30 | 10 | -97.1 |
| n18 | 15 | 5,10,15 | -100.0 |
|  | 30 | 10,15 | -100.1 |
| n20 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
| n24 | 15 | 5,10 | -100.0 |
|  | 30 | 10 | -100.5 |
|  | 60 | 10 |  |
| n25 | 15 | 5,10,15,20 | -96.5 |
|  | 30 | 10,15,20 | -96.6 |
|  | 60 | 10,15,20 |  |
| n26 | 15 | 5,10,15,20 | -97.56 |
|  | 30 | 10,15,20 | -97.86 |
| n28 | 15 | 5,10,15,20 | -98.5 |
|  | 30 | 10,15,20 | -98.6 |
| n30 | 15 | 5,10 | -99.0 |
|  | 30 | 10 | -99.1 |
| n65 | 15 | 5,10,15,20 | -99.5 |
|  | 30 | 10,15,20 | -99.6 |
|  | 60 | 10,15,20 |  |
| n66 | 15 | 5,10,15,20 | -99.5 |
|  | 30 | 10,15,20 | -99.6 |
|  | 60 | 10,15,20 |  |
| n70 | 15 | 5,10,15,20 | -100.0 |
|  | 30 | 10,15,20 | -100.1 |
|  | 60 | 10,15,20 |  |
| n71 | 15 | 5,10,15,20 | -97.2 |
|  | 30 | 10,15,20 | -97.3 |
| n74 | 15 | 5,10,15,20 | -99.53 |
|  | 30 | 10,15,20 | -99.63 |
|  | 60 | 10,15,20 |  |
| n85 | 15 | 5,10,15 | -97.0 |
|  | 30 | 10,15 | -97.1 |
| n100 | 15 | 5 | -100 |
| n105 | 15 | 5,10,15,20 | -97.28 |
|  | 30 | 10,15,20 | -97.3 |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |

Table 7.3I.2-1b: Two antenna port reference sensitivity for eRedcap UE for TDD band and FDD band with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 | TDD |
|  | 30 | 10, 15 | -97.1 +10log10(12/24)=-100.1 |  |
|  | 60 | 10, 15 |  |  |
| n381 | 15 | 5, 10, 15, 20, | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n39 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n40 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -1001 |  |
|  | 60 | 10, 15, 20 |  |  |
| n411, n901 | 15 | 5, 10, 15, 20 | -94.8 | TDD |
|  | 30 | 10, 15, 20 | -98.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n481 | 15 | 5, 10, 15, 20 | -99 | TDD |
|  | 30 | 10, 15, 20 | -99.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n50 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 | TDD |
|  | 30 | 10 | -100.1 |  |
|  | 60 | 10 | -97.5 |  |
| n771,4 | 15 | 10, 15, 20 |  | TDD |
|  | 30 | 10, 15, 20 | -98.6 |  |
|  | 60 | 10, 15, 20 |  |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(25/52)=-99 | TDD |
|  | 30 | 10, 15, 20 | -99.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n791 | 15 | 10, 20 | -95.8 + 10log10(25/52)=-99 | TDD |
|  | 30 | 10, 20 | -99.1 |  |
|  | 60 | 10, 20 |  |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5,10,15,20 | -100 | FDD |
|  | 30 | 10,15,20 | -100.1 |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5,10,15,20 | -100 | FDD |
|  | 30 | 10,15,20 | -100.1 |  |
| n101 | 15 | 5, 10 | -100 | TDD |
|  | 30 | 10 | -100.1 |  |
| n1041,10 | 15 | 20 | -90.7 + 10log10(25/106)=-97 | TDD |
|  | 30 | 20 | -90.8 +10log10(12/51)=-97.1 |  |
|  | 60 | 20 |  |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. |  |  |  |  |

Table 7.3I.2-1c: Uplink configuration for reference sensitivity for eRedcap UE

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
| n1 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 24 |  |
| n2 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 | 101 | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 101 | 101 |  |
| n3 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 101 | 101 |  |
| n5 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n7 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 181 |  |
| n8 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n12 | 15 | 201 | 201 | 201 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n13 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n14 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n18 | 15 | 25 | 251 | 251 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n20 | 15 | 25 | 201 | 202 | 202 | FDD |
|  | 30 |  | 101 | 102 | 102 |  |
| n24 | 15 | 25 | 251 |  |  | FDD |
|  | 30 |  | 121 |  |  |  |
|  | 60 |  | 10 |  |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 | FDD |
|  | 30 |  | 24 | 241 | 241 |  |
|  | 60 |  | 101 | 101 | 101 |  |
| n26 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n28 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
| n30 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n34 | 15 | 25 | 251 | 251 |  | TDD |
|  | 30 |  | 121 | 121 |  |  |
|  | 60 |  | 10 | 18 |  |  |
| n38 | 15 | 25 | 50 | 75 | 100 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n39 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n40 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n41, n90 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n48 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n50 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n51 | 15 | 25 |  |  |  | TDD |
| n53 | 15 | 25 | 251 |  |  | TDD |
|  | 30 |  | 121 |  |  |  |
|  | 60 |  | 10 |  |  |  |
| n65 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 24 |  |
| n66 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 24 |  |
| n70 | 15 | 25 | 251 | 251 | Note 3 | FDD |
|  | 30 |  | 121 | 121 | Note 3 |  |
|  | 60 |  | 101 | 18 | Note 3 |  |
| n71 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n74 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
|  | 60 |  | 51 | 51 | 51 |  |
| n77 | 15 |  | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n78 | 15 |  | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n79 | 15 |  | 251 |  | 251 | TDD |
|  | 30 |  | 121 |  | 121 |  |
|  | 60 |  | 10 |  | 24 |  |
| n85 | 15 | 201 | 201 | 201 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n91 | 15 | 254 | 201,4 |  |  | FDD |
| n92 | 15 | 25 | 201 | 201 | 201 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
| n93 | 15 | 254 | 251,4 |  |  | FDD |
| n94 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n100 | 15 | 25 |  |  |  | FDD |
| n101 | 15 | 25 | 251 |  |  | TDD |
|  | 30 |  | 121 |  |  |  |
| n104 | 15 |  |  |  | 251 | TDD |
|  | 30 |  |  |  | 121 |  |
|  | 60 |  |  |  | 24 |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: Void |  |  |  |  |  |  |

For a RedCap UE equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-4.

For a RedCap UE with capability [IE] equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2-2a shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-5.

Table 7.3I.2-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n2 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n3 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
|  | 60 |  | -95.3 | -93.2 | -92.0 |
| n5 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n8 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n12 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n13 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n14 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.2 | -95.2 |  |
| n20 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n24 | 15 | -100.0 | -96.8 |  |  |
|  | 30 |  | -97.2 |  |  |
|  | 60 |  | -97.5 |  |  |
| n25 | 15 | -97.3 | -94.1 | -92.3 | -91.0 |
|  | 30 |  | -94.5 | -92.5 | -91.2 |
|  | 60 |  | -94.8 | -92.7 | -91.5 |
| n26 | 15 | -98.3 | -95.1 | -93.3 | -92.0 |
|  | 30 |  | -95.5 | -93.5 | -92.2 |
| n28 | 15 | -99.3 | -96.1 | -94.3 | -93.0 |
|  | 30 |  | -96.5 | -94.5 | -93.2 |
| n30 | 15 | -99.5 | -96.3 |  |  |
|  | 30 |  | -96.7 |  |  |
| n65 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n66 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| n74 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n85 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |

Table 7.3I.2-2a: HD-FDD eRedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -97.2+ 10log(12/24)= -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n2 | 15 | -98.8 | -98.8 | -98.8 | -98.8 |
|  | 30 |  | -99.0 | -99.0 | -99.0 |
|  | 60 |  |  |  |  |
| n3 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
|  | 60 |  |  |  |  |
| n5 | 15 | -98.8 | -98.8 | -98.8 | -98.8 |
|  | 30 |  | -99.0 | -99.0 | -99.0 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  |  |  |  |
| n8 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
| n12 | 15 | -97.8 | -97.8 | -97.8 |  |
|  | 30 |  | -98.0 | -98.0 |  |
| n13 | 15 | -97.8 | -97.8 |  |  |
|  | 30 |  | -98.0 |  |  |
| n14 | 15 | -97.8 | -97.8 |  |  |
|  | 30 |  | -98.0 |  |  |
| n18 | 15 | -100.0 | -100.0 | -100.0 |  |
|  | 30 |  | -100.2 | -100.2 |  |
| n20 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
| n24 | 15 | -100.0 | -100.0 |  |  |
|  | 30 |  | -100.2 |  |  |
|  | 60 |  |  |  |  |
| n25 | 15 | -97.3 | -97.3 | -97.3 | -97.3 |
|  | 30 |  | -97.5 | -97.5 | -97.5 |
|  | 60 |  |  |  |  |
| n26 | 15 | -98.3 | -98.3 | -98.3 | -98.3 |
|  | 30 |  | -98.5 | -98.5 | -98.5 |
| n28 | 15 | -99.3 | -99.3 | -99.3 | -99.3 |
|  | 30 |  | -99.5 | -99.5 | -99.5 |
| n30 | 15 | -99.5 | -99.5 |  |  |
|  | 30 |  | -99.7 |  |  |
| n65 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n66 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n70 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n71 | 15 | -98.0 | -98.0 | -98.0 | -98.0 |
|  | 30 |  | -98.2 | -98.2 | -98.2 |
| n74 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n85 | 15 | -97.8 | -97.8 | -97.8 |  |
|  | 30 |  | -98.0 | -98.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |

For a RedCap UE equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2-3 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-4.

For a RedCap UE with capability [IE] equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2-3a shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2-5.

Table 7.3I.2-3: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
| n7 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n8 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n12 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n13 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n14 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n18 | 15 | -97.5 | -94.3 | -92.5 |  |
|  | 30 |  | -94.7 | -92.7 |  |
| n20 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n24 | 15 | -97.5 | -94.3 |  |  |
|  | 30 |  | -94.7 |  |  |
|  | 60 |  | -95.0 |  |  |
| n25 | 15 | -94.8 | -91.6 | -89.8 | -88.5 |
|  | 30 |  | -92.0 | -90.0 | -88.7 |
|  | 60 |  | -92.3 | -90.2 | -89.0 |
| n26 | 15 | -95.8 | -92.6 | -90.8 | -89.5 |
|  | 30 |  | -93.0 | -91.0 | -89.7 |
| n28 | 15 | -96.8 | -93.6 | -91.8 | -90.5 |
|  | 30 |  | -94.0 | -92.0 | -90.7 |
| n30 | 15 | -97.0 | -93.8 |  |  |
|  | 30 |  | -94.2 |  |  |
| n65 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n66 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n70 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n71 | 15 | -95.5 | -92.3 | -90.5 | -89.2 |
|  | 30 |  | -92.7 | -90.7 | -89.4 |
| n74 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n85 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n91 | 15 | -97.5 |  |  |  |
| n92 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
| n93 | 15 | -97.5 |  |  |  |
| n94 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |

Table 7.3I.2-3a: HD-FDD eRedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
| n7 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n8 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
| n12 | 15 | -95.3 | -95.3 | -95.3 |  |
|  | 30 |  | -95.5 | -95.5 |  |
| n13 | 15 | -95.3 | -95.3 |  |  |
|  | 30 |  | -95.5 |  |  |
| n14 | 15 | -95.3 | -95.3 |  |  |
|  | 30 |  | -95.5 |  |  |
| n18 | 15 | -97.5 | -97.5 | -97.5 |  |
|  | 30 |  | -97.7 | -97.7 |  |
| n20 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
| n24 | 15 | -97.5 | -97.5 |  |  |
|  | 30 |  | -97.7 |  |  |
|  | 60 |  | -95.0 |  |  |
| n25 | 15 | -94.8 | -94.8 | -94.8 | -94.8 |
|  | 30 |  | -95.0 | -95.0 | -95.0 |
|  | 60 |  | -92.3 | -90.2 | -89.0 |
| n26 | 15 | -95.8 | -95.8 | -95.8 | -95.8 |
|  | 30 |  | -96.0 | -96.0 | -96.0 |
| n28 | 15 | -96.8 | -96.8 | -96.8 | -96.8 |
|  | 30 |  | -97.0 | -97.0 | -97.0 |
| n30 | 15 | -97.0 | -97.0 |  |  |
|  | 30 |  | -97.2 |  |  |
| n65 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n66 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n70 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n71 | 15 | -95.5 | -95.5 | -95.5 | -95.5 |
|  | 30 |  | -95.7 | -95.7 | -95.7 |
| n74 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n85 | 15 | -95.3 | -95.3 | -95.3 |  |
|  | 30 |  | -95.5 | -95.5 |  |
| n91 | 15 | -97.5 |  |  |  |
| n92 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n93 | 15 | -97.5 |  |  |  |
| n94 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |

Table 7.3I.2-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n18 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
|  | 60 |  | 10 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 50 | 75 | NOTE 1 |
|  | 30 |  | 24 | 36 | NOTE 1 |
|  | 60 |  | 10 | 18 | NOTE 1 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n85 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |

Table 7.3I.2-5: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n7 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
|  | 60 |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n12 | 15 | 25 | 25 | 25 |  |
|  | 30 |  | 12 | 12 |  |
| n13 | 15 | 25 | 25 |  |  |
|  | 30 |  | 12 |  |  |
| n14 | 15 | 25 | 25 |  |  |
|  | 30 |  | 12 |  |  |
| n18 | 15 | 25 | 25 | 25 |  |
|  | 30 |  | 12 | 12 |  |
| n20 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n24 | 15 | 25 | 25 |  |  |
|  | 30 |  | 12 |  |  |
|  | 60 |  | 10 |  |  |
| n25 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n28 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| n30 | 15 | 25 | 25 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
|  | 60 |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
|  | 60 |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 25 | 25 | NOTE 1 |
|  | 30 |  | 12 | 12 | NOTE 1 |
|  | 60 |  | 10 | 18 | NOTE 1 |
| n71 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 50 |
| n74 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n85 | 15 | 25 | 25 | 25 |  |
|  | 30 |  | 12 | 12 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 25 | 25 | 25 |
|  | 30 |  | 12 | 12 | 12 |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------