3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305663

Online, April 17 – April 26, 2023

Source:	Huawei, Hisilicon

Title:	Discussion on Rel-16 time mask of Tx switching

Agenda item:			7.2.1

Document for:	Approval

# 1. Introduction

In RAN1#112, an LS [1] to clarify the Rel-16 UL Tx switching period was sent to RAN4. To align the understanding between RAN4 and RAN1, we provide our view in this contribution.

Agreement - If the gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period, - The time of no UL transmission allocated absorbs the switching period - Neither of the uplink transmissions (the one ending on the switch-from carrier nor the one starting on the switch-to carrier) are interrupted by the switching period. - The setting of uplinkTxSwitchingPeriodLocation has no impact. - Send an LS to RAN4 requesting RAN4 to, in this regard, clarify TS38.101-1 subclauses 6.3A.3.3.2 and 6.3C.3.1 for CA, and SUL based UL Tx Switching, and to TS38.101-3 subclause 6.3B.4.1 for EN-DC.

# 2. Discussion

First of all, in RAN4’s specification, the RRC parameter uplinkTxSwitchingPeriodLocation is used to indicate the frequency domain location of the switching period, either carrier 1 or carrier 2, which is also consistent with RAN2’s description on the RRC parameter

38.101-1 6.3A.3.3.2 Time mask for switching between two uplink carriers … The switching periods described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod. Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1 Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

38.331 UplinkTxSwitching-r16 ::= SEQUENCE { uplinkTxSwitchingPeriodLocation-r16 BOOLEAN, uplinkTxSwitchingCarrier-r16 ENUMERATED {carrier1, carrier2} } uplinkTxSwitchingPeriodLocation Indicates whether the location of UL Tx switching period is configured in this uplink carrier in case of inter-band UL CA, SUL, or (NG)EN-DC, as specified in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of inter-band UL CA or SUL, network configures this field to TRUE for one of the uplink carriers involved in dynamic UL TX switching and configures this field in the other carrier to FALSE. In case of (NG)EN-DC, network always configures this field to TRUE for NR carrier (i.e. with (NG)EN-DC, the UL switching period always occurs on the NR carrier).

From RAN1’s perspective of view, the UE would know which carrier is with the lower priority so that the transmission on which can be sacrificed by omitting the transmission. Thus frankly, the RRC parameter uplinkTxSwitchingPeriodLocation is used to indicate the on which carrier the transmission can be impacted. Back to TS38.101-1 Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b, it illustrate that the transmission on the carrier that switching period doesn’t located in is not impacted by the switching period at all. In other words, the UE does not have to omit transmission on the carrier that is indicated FALSE by uplinkTxSwitchingPeriodLocation.

Observation 1: the RRC signaling uplinkTxSwitchingPeriodLocation is used to indicate the carrier location rather than time domain location of switching period.

Observation 2: The transmission on the carrier that is indicated FALSE by uplinkTxSwitchingPeriodLocation is not impacted by uplink Tx switching.

In the LS from RAN1, if the length between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier is longer than the duration of switching period, the UE has enough flexibility to execute Tx switching on the time domain resource of no UL transmission allocated. In this case, neither of the transmission on switch-from and switch-to bands is impacted by Tx switching. Thus the RRC parameter uplinkTxSwitchingPeriodLocation is invalid. And the time mask to illustrate the frequency domain location of switching period is not applicable.

Observation 3: The time mask of uplink Tx switching to illustrate the frequency domain location of switching period is only applicable to the case that the length between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier is shorter than the duration of switching period.

# 3. Conclusions

This contribution presents the following observations:

Observation 1: the RRC signaling uplinkTxSwitchingPeriodLocation is used to indicate the carrier location rather than time domain location of switching period.

Observation 2: The transmission on the carrier that is indicated FALSE by uplinkTxSwitchingPeriodLocation is not impacted by uplink Tx switching.

Observation 3: The time mask of uplink Tx switching to illustrate the frequency domain location of switching period is only applicable to the case that the length between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier is shorter than the duration of switching period.

# 4. Reference

[1]	R1-2302198, “LS on Rel-16 UL Tx Switching period”, TSG RAN1 Meeting #112, Feb. 27 – Mar. 3, 2023.