3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305729

Electronic Meeting, April 17 – 26, 2023

Agenda Item:	5.14.4.1

Source:	Ericsson

Title:	Discussions on general ATG RRM requirements

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss the frequency band grouping and specification impact of ATG RRM requirements in TS 38.133.

# 2	Discussion

## Frequency bands

The following three frequency bands are agreed to be specified for ATG according to the approved WID [2]:

- Band n1

- Band n78

- Band n79

It is noted that these bands already exist in the NR operating band groups defined in Table 3.5.2-1 in TS 38.133 as shown below:

Table 1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  | NR CCA10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands |
| A | NR_FDD_FR1_A | n1, n18, n24, n70, n744, n91, n92, n93, n94, n100 | NR_TDD_FR1_A | n34, n389, n39, n40, n50, n51, n53, n101 | NR_SDL_FR1_A | n67, n75, n76 | NR_CCA_FR1_A | - |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | n387 | NR_SDL_FR1_B | - | NR_CCA_FR1_B | - |
| C | NR_FDD_FR1_C | n30 | NR_TDD_FR1_C | n48, n771, n78, n79 | NR_SDL_FR1_C | - | NR_CCA_FR1_C | - |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - | NR_CCA_FR1_D | - |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41, n90 | NR_SDL_FR1_E | - | NR_CCA_FR1_E | - |
| F | NR_FDD_FR1_F | n266 | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - | NR_CCA_FR1_F | - |
| G | NR_FDD_FR1_G | n3, n8, n12, n13, n14, n20, n71, n85, n10511 | NR_TDD_FR1_G | n104 | NR_SDL_FR1_G | n29 | NR_CCA_FR1_G | - |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - | NR_CCA_FR1_H | - |
| I | NR_FDD_FR1_I | - | NR_TDD_FR1_I |  | NR_SDL_FR1_I | - | NR_CCA_FR1_I | n46 |
| J | NR_FDD_FR1_J | - | NR_TDD_FR1_J | n478, n96 | NR_SDL_FR1_J | - | NR_CCA_FR1_J | n96 |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is only used for V2X SL service, the band is exclusively used for NR V2X in particular regions. NOTE 8: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. NOTE 9: When this band is only used for WAN service. NOTE 10: Operating bands where operation on carrier frequencies with CCA is supported. NOTE 11: The minimum Io condition is reduced by 0.5 dB when the downlink channel overlap the 612-617 MHz frequency range and the channel bandwidth is 5 MHz. |  |  |  |  |  |  |  |  |

RAN4 needs to discuss whether new frequency band grouping table is needed for ATG. In our opinion, the existing tables with same band notation can be reused provided the UE requirements on the REFSENS do not change for the ATG bands. Otherwise, new ATG specific tables need to be introduced. Following REFSENS agreements were reached in RF session at last meeting [2]:

Issue 2-1-1: REFSENS requirements Agreement: - The REFSENS requirements specified in TS 38.101-1 for each band can be reused for ATG UE

Given that the REFSENS requirements are same for the ATG bands, we propose to use the existing frequency band grouping tables for ATG.

Observation 1: The REFSENS requirements for ATG bands are same as for corresponding legacy bands.

Proposal 1	The frequency band grouping tables from the legacy bands are reused for ATG.

## Specification impact

How to introduce the ATG requirements into TS 38.133 was discussed at last meeting and following options were identified [1]:

Issue 1-1-5: How to involve ATG RRM core requirements in TS38.133 - Option 1: ATG specific RRM requirements are defined in separate sections in specification. - Option 2: Add ATG related requirements in the current corresponding section, similar as HST.

Our view is to define the ATG specific requirements in separate sections since there are important differences between the legacy NR requirements and ATG specific requirements although some requirements are identical. For instance, the legacy requirements contain CA/DC specific requirements in same sections and inter-RAT requirements which are not applicable to ATG. Another important difference is that the ATG is a specialized equipment that is mounted on the aircraft which is not expected to support other features that are supported by the handheld devices. In addition, defining the ATG specific requirements in separate sections facilities specification readability as well as further enhancements to ATG. Therefore, we support option 1.

Proposal 2	ATG specific RRM requirements are defined in separate sections in TS 38.133.

# Conclusion

In this contribution we have discussed the general aspects of ATG requirements based on the issues discussed at last meeting. We have made the following observations:

Observation 1: The REFSENS requirements for ATG bands are same as for corresponding legacy bands.

Proposal 1	The frequency band grouping tables from the legacy bands are reused for ATG.

Proposal 2	ATG specific RRM requirements are defined in separate sections in TS 38.133.

# References

R4-2303226, “WF on NR ATG RRM requirements”, CMCC

R4-2303643, “WF on ATG UE RF requirements”, Huawei, HiSilicon