# 3GPP TSG-RAN WG4 Meeting # 106 bis-e		R4-2305744

# Electronic meeting, April 17 – 26, 2023

Source:	Sony

Title:	Views on 4Rx handheld UE for low NR bands

Agenda item:	4.29.2

Document for:	Approval

- Background

Support of 4Rx and 3Tx for low operating bands was discussed in RAN4 #106 and two WF were agreed [1], [2].

This Tdoc will discuss 4Rx requirement for bands n20, n20, n8, n5, n26 and n71 [3].

- Discussion

As discussed in the WF [1] the antenna is the main difficulty in the support of 4Rx in the below-1GHz-operation. For good TRP/TRS performance coupling between antennas shall be kept low. For smaller form factor, such as handheld devices (compared to FWA), this may be a great challenge. For smart phones, acceptable performance may be obtained in devices with large form factors, but it will be extremely difficult to accommodate with 4Rx within compact phones. Therefore, TRP/TRS performance of 4RX/3TX is expected to be more degraded due to the below-1GHz-operation.

Observation 1	TRP/TRS performance of 4RX/3TX is expected to be more degraded due to the below-1GHz-operation.

The conducted performance (REFSENS) is also degraded due the increased complexity. Maintaining good isolation between RF traces in a smaller form factor, for frequencies < 1GHz, could be challenging. The already specified ΔRIB,4R for n8, n28, n71, n105, targeting FWA [5] is a good starting point for the bands listed in the WID [3] (Table 4.1-1). However, as previously mentioned, the smaller form factor of handsets (compared to FWA) may increase the design challenge, e.g., higher coupling between RF traces. We, therefore, suggest the ΔRIB,4R for the bands in Table 4.1-1 [3] to be further investigated.

Proposal 1	Further investigate the ΔRIB,4R for the bands in Table 4.1-1 in the WID.

- Conclusion

In this contribution, we have discussed ΔRIB,4R for, bands n20, n20, n8, n5, n26 and n71. We have made one observation and proposal:

Observation 1	TRP/TRS performance of 4RX/3TX is expected to be more degraded due to the below-1GHz-operation.

Proposal 1	Further investigate the ΔRIB,4R for the bands in Table 4.1-1 in the WID.

- References

R4-2303522		“WF on 4Rx requirements for low operating bands”, OPPO

R4-2303523		“WF on 3Tx requirements”, OPPO

RP-230161		“Revised WID for Low NR band 4Rx and 3Tx”, OPPO

RP-230160		“Status report for WI 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC”, rapporteur: OPPO

TS 30.101-2		Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R