3GPP TSG RAN WG4 Meeting #106bis-e	R4-2305758

E-meeting, 17 Apr. 2023 – 26 Apr. 2023

Agenda Item:	5.9.2.3

Source: 	Ericsson

Title:	Discussion on other potential enhancement for FR2 SCell activation

Document for:	Discussion

# Introduction

In this contribution, we provide our views on other enhancements, other than L1 and L3 parts. Following issues are deprioritised in last meeting.

| Enhancement part | Issue index in topic summary (R4-2302769) | Enhancement solution |  | RAN4 decision on whether to continue discuss this enhancement solution in R18? |
| --- | --- | --- | --- | --- |
| L3 part | Issue 1-1-1, Issue 1-1-1a | New sub-states for known/unknown state for FR2 SCell |  | Discussed in CR stage |
|  | Issue 1-1-2/1-1-3/1-1-4/1-1-5/1-1-6 | New measurement status indication or report for FR2 SCell activation enhancement |  | Yes, continue discussion |
|  | Issue 1-2-1 | Beam sweeping factor enhancement in L3 part (not related with WI of FR2 multi-Rx chain DL reception) |  | Yes, continue discussion |
|  | Issue 1-3-1/1-3-2 | enhancement of “TFirstSSB_MAX + 15*TSMTC_MAX” part (sample reduction) |  | Deprioritized |
|  | Issue 1-3-3 | Enhancement of “8*Trs” part |  | Merged to issue 1-2-1/2-1-1 |
|  | Issue 1-4-2 | RS and QCL information related enhancement for FR2 unknown SCell activation |  | Deprioritized |
|  | Issue 1-4-1/Issue 1-4-3 | RS and QCL information related enhancement for FR1 unknown SCell activation |  | Yes, continue discussion |
|  | Issue 1-5-1/1-5-2 | Ending point of Scell activation of unknown case in FR2 |  | Deprioritized |
|  | Issue 1-6-1 | use SSB periodicity (e.g. 20ms) instead of SMTC periodicity for FR2 unknown SCell activation |  | Yes, continue discussion |
| L1 part | Issue 2-1-1 | Beam sweeping factor enhancement in L1-RSRP measurement (not related with WI of FR2 multi-Rx chain DL reception) |  | Yes, continue discussion |
|  | Issue 2-1-2 | Skip L1-RSRP measurement of FR2 unknown SCell activation |  | Yes, continue discussion |
|  | Issue 2-1-3 | Prioritization enhancement for L1-RSRP measurement | L1-RSRP measurement is performed in non-DRX mode even DRX is configured. | Yes, continue discussion |
|  |  |  | Whether prioritize L1 over L3 is under discussing | Deprioritized |
|  | Issue 2-2-1 | Skip fine timing tracking for SSB corresponding to the TCI state |  | Deprioritized |
|  | Issue 2-2-2 | TCI activation enhancement |  | Deprioritized |
|  | Issue 2-3-1 | Aperiodic RS for TFineTiming |  | Yes, continue discussion |
|  | Issue 2-3-2/ Issue 2-4-2 | Aperiodic RS for L1-RSRP measurement |  | Deprioritized |
| Others | Issue 3-1-1 | SCell without SSB for inter-band FR2 unknown SCell activation |  | Deprioritized |
|  | Issue 3-1-2/ Issue 3-1-3/Issue 3-1-4 | SCell without SSB for inter-band FR1 unknown SCell activation |  | Deprioritized |
|  | Issue 3-1-5 | known/unknown condition enhancement for FR2 SCell activation |  | Deprioritized |
|  | Issue 3-1-6 | Scheduling enhancement for FR2 unknown SCell activation |  | Yes, continue discussion |
|  | Issue 3-1-7 | CBRA for FR2 unknown SCell activation |  | Deprioritized |

# Other enhancements

Issue 3-1-6: Scheduling enhancement for FR2 unknown SCell activation

- Proposals

  - Option 1 (Apple): the early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed.

  - Option 2 (Nokia):

    - The potential to enable earlier data transmission within the activation period shall be studied.

    - The UE shall be scheduled by the network immediately after L1-RSRP reporting.

    - The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell.

NW needs to know about the rough beam information from the UE to start scheduling after L3 measurements. If the L3 measurements results are not available, then RAN4 need to introduce a new report so that UE can report the l3 measurement results after L3 part completion. If such report is introduced, NW can optionally start scheduling the UE after receiving the L3 report as NW can know the beam information and TCI information can be derived based on the beam information. Further, NW may need to start scheduling with lower MCS values if NW starts scheduling UE before CQI report reception. This may depend on whether NW like to schedule UE before receiving the CQI report or not. Therefore, in our understanding, this should not be a mandatory from NW point of view.

- Scheduling enhancement for FR2 unknown SCell may be a NW controlled optional feature. That means, NW may indicate the starting point of the scheduling to UE.

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction and made following proposals:

- Scheduling enhancement for FR2 unknown SCell may be a NW controlled optional feature. That means, NW may indicate the starting point of the scheduling to UE.

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo