3GPP TSG-RAN4 #107	R4-2307043

Incheon, South Korea, 22nd May 2023 – 26th May 2023

Title:	UL 256QAM, phase noise profiles and EVM testing

Source:	Anritsu Limited

Agenda Item:	8.7.2

Document for:	Discussion

# Introduction

The WID “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” [1] describes the need to “investigate and enable UL 256QAM for FR2-1, including “study the gain, operating SNR, phase noise model and implementation aspects” with UE phase noise profiles having a particular impact on the feasibility of UL 256QAM.

The lower frequency part of the phase noise causes a common phase error (CPE) which affects all the subcarriers within a multicarrier symbol similarly, which allows simple CPE compensation based on PTRS. The higher frequency part of the phase noise causes inter-carrier interference (ICI), which can be mitigated by using higher SCS, while ICI compensation even based on PTRS is much more complex.

At the RAN4 #106 meeting, there were some contributions [2][3] regarding UL EVM requirement and the use of PTRS processing to calculate the EVM minimizing the effect of CPE due to phase noise in FR2. Also, some contributions [2][4] proposed new phase noise profiles to make UL 256QAM feasible.

In OFDM based systems, CPE causes a constant rotation in the angle of the modulation constellation, and ICI is causing scattering of the constellation points. As shown in some contributions [2][3], CPE cancellation based on PTRS gives good results while leaving the effect of ICI. Therefore, it is the ICI that limits the SNR/EVM achievable, the phase noise profile at frequencies between a tenth of the SCS and the signal bandwidth has then a great importance for the feasibility of UL 256QAM.

The aim of this contributions is to compare the different phase noise profiles proposed at the #106 and #106bis-e meetings and to see which ones would allow the support of UL 256QAM. The reason being that the 38.803 [5] phase noise profiles (“UE example 1” and “UE example 2”) would not allow the data-aided EVM measurement as they would not allow to have an EVM good enough such as the typical 3.5% EVM requirement.

# Discussion

## EVM determination based on phase noise profile

In this contribution, the SNR/EVM is determined based on the phase noise profile by calculating the phase jitter and converting it to an equivalent SNR/EVM. That approach is different from the common approach [2][3], but as the aim here is to compare the different phase noise profiles proposed between each other while also using as a reference the EVM simulation results for the 38.803 phase noise profiles, we think that method is also relevant. Specifically, it allows to have the improvement brought by the two new phase noise profiles proposed at the #106 meeting compare to the example 1 and example 2 phase noise profiles in the 38.803 TR [5].

A (dBc)=Integrated Phase Noise Power        (1)

PhaseJitterRMS(radians)=2×10A10            (2)

SNR(dB)=-20×log10PhaseJitterRMS           (3)

EVM(%)=100×10-SNR20                                (4)

## EVM calculations based on phase jitter

From the three carrier frequencies in the 38.803 [5],only the following two are considered 29.55GHz, 39GHz.

### 29.55GHz carrier frequency

Figure 1- Comparison of simulation results

| Phase noise profile at 29.55GHz | 38_803 Example1 [5] |  | 38_803 Example2 [5] |  | Qualcomm Min (38_803 Example1, 38_803 Example2) using pole-zero model [7] |  | MediaTek [7] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fmin used in integration (Hz) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) |
| 1.00E+03 | 26.05 | 5.0 | 28.2 | 3.9 | 27.8 | 4.1 | 29.4 | 3.4 |
| 1.00E+04 | 28.34 | 3.8 | 28.5 | 3.8 | 31.8 | 2.6 | 32.3 | 2.4 |
| 1.00E+05 | 28.65 | 3.7 | 28.6 | 3.7 | 32.5 | 2.4 | 32.7 | 2.3 |
| 1.00E+06 | 29.08 | 3.5 | 31.9 | 2.5 | 33.7 | 2.1 | 33.3 | 2.2 |
| 1.00E+07 | 33.3 | 2.2 | 41.6 | 0.8 | 41.9 | 0.8 | 37.5 | 1.3 |
| Note: Fmax is fixed to 100MHz in the calculations. |  |  |  |  |  |  |  |  |

Table 1 - Simulation results for UE example 2 phase noise profile.

The phase noise profiles from MediaTek and Qualcomm improves the SNR by about 3.5dB compare to the case of the Example 1 and Example phase noise profiles.

Observation 1: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 29.55GHz.

### 39GHz carrier frequency

Figure 2- Comparison of simulation results – 2nd set of results

| Phase noise profile at 39GHz | 38_803 Example1 [5] |  | 38_803 Example2 [5] |  | Qualcomm Min (38_803 Example1, 38_803 Example2) [2] – Option1 [6] |  | MediaTek [7] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fmin used in integration (Hz) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) | SNR (dB) | EVM (%) |
| 1.00E+03 | 23.1 | 7.0 | 25.8 | 5.1 | 27.3 | 4.3 | 26.7 | 4.6 |
| 1.00E+04 | 25.0 | 5.6 | 26.1 | 5.0 | 28.7 | 3.7 | 29.4 | 3.4 |
| 1.00E+05 | 25.2 | 5.5 | 26.2 | 4.9 | 29.3 | 3.4 | 29.7 | 3.3 |
| 1.00E+06 | 25.6 | 5.3 | 29.5 | 3.3 | 30.3 | 3.1 | 30.2 | 3.1 |
| 1.00E+07 | 29.8 | 3.3 | 39.2 | 1.1 | 39.2 | 1.1 | 34.4 | 1.9 |
| Note: Fmax is fixed to 100MHz in the calculations. |  |  |  |  |  |  |  |  |

Table 2 - Simulation results for UE example 2 phase noise profile.

Using Min() phase noise profile improves the SNR by about 3.5dB compare to the case of the Example 1 and Example phase noise profiles. The phase noise profile by MediaTek is also giving about 2.5dB improvement of the SNR.

Observation 2: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 39GHz.

# Conclusion

Based on the discussion above, the following can be noted:

Observation 1: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 29.55GHz.

Observation 2: Both the MediaTek and Qualcomm phase noise profiles allow UL 256QAM to be feasible in terms of phase noise profile at 39GHz.

# References

[1] RP-220967, “NR RF requirements enhancement for frequency range 2 (FR2)”, Nokia, RAN#95-e, Mar. 2022

[2] R4-2300707, “On enabling FR2 UL256QAM”, Qualcomm Incorporated, RAN4 #106, Athens, Greece

[3] R4-2301147, “UL 256QAM and CPE compensation based on PTRS”, Anritsu Limited, RAN4 #106, Athens, Greece

[4] R4-2301928, “Views on FR2-1 UL 256QAM”, MediaTek Inc., RAN4 #106, Athens, Greece

[5] 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, version 14.3.0

[6]	R4-2303709, “WF on FR2-1 UL 256QAM”, Xiaomi, RAN4 #106, Athens, Greece

[7]	R4-2306603, “WF on FR2-1 UL 256 QAM”, Xiaomi, RAN4 #106bis-e, Electronic meeting

# Annex 1 – 38-803 Example 1 Phase Noise Profile

| PSD0 | 1585 (32dB) |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | Alpha z,n | fp,m | Alpha p,m |
| 1 | 3.00E+03 | 2.37 | 1 | 3.3 |
| 2 | 5.50E+05 | 2.7 | 1.54E+06 | 3.3 |
| 3 | 2.80E+08 | 2.53 | 3.00E+07 | 1 |

Table 3 – 29.55GHz.

| PSD0 | 2760.85 (34.4dB) |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | Alpha z,n | fp,m | Alpha p,m |
| 1 | 3.00E+03 | 2.37 | 1 | 3.3 |
| 2 | 4.99E+05 | 2.7 | 1.54E+06 | 3.3 |
| 3 | 3.90E+08 | 2.53 | 3.00E+07 | 1 |

Table 4 – 39GHz.

# Annex 2 – 38-803 Example 2 Phase Noise Profile

| PSD0 | 1585 |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | Alpha z,n | fp,m | Alpha p,m |
| 1 | 3.00E+03 | 2.37 | 1 | 3.3 |
| 2 | 5.50E+05 | 2.7 | 1.54E+06 | 3.3 |
| 3 | 2.80E+08 | 2.53 | 3.00E+07 | 1 |

Table 5 – 29.55GHz.

| PSD0 | 2760.85 |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | Alpha z,n | fp,m | Alpha p,m |
| 1 | 3.00E+03 | 2.37 | 1 | 3.3 |
| 2 | 4.99E+05 | 2.7 | 1.54E+06 | 3.3 |
| 3 | 3.90E+08 | 2.53 | 3.00E+07 | 1 |

Table 6 – 39GHz.

# Annex 3 – Phase Noise Profile proposed by Qualcomm

The phase noise profiles shown below are the ones provided by Qualcomm and documented in [7] for 29.55GHz and Option 1 documented in [6] for 39GHz.

| PSD0 | 1584.9 (32dB) |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | Alpha z,n | fp,m | Alpha p,m |
| 1 | 3.00E+03 | 2.37 | 1 | 3.3 |
| 2 | 6E+05 | 4.7 | 8E+05 | 5.3 |
| 3 | 8E+05 | 2 | 1.1E+06 | 2.5 |

Table 7 – 29.55GHz.

No phase noise model using the pole-zero method, please refer to Min(38_803 Example1,38_803 Example2 as in Option1 [6].

Table 8 – 39GHz.

# Annex 3 – Phase Noise Profile proposed by MediaTek

The phase noise profiles shown below are the ones provided by MediaTek and documented in [7].

| PSD0 | 860.4 (29.35dB) |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | Alpha z,n | fp,m | Alpha p,m |
| 1 | 3.00E+03 | 2.37 | 1 | 3.3 |
| 2 | 6.50E+05 | 2.7 | 1.60E+06 | 3.3 |
| 3 | 1.82E+08 | 2.53 | 3.00E+07 | 1 |

Table 9 – 29.55GHz.

| PSD0 | 1498.7 (31.76dB) |  |  |  |
| --- | --- | --- | --- | --- |
| n,m | fz,n | Alpha z,n | fp,m | Alpha p,m |
| 1 | 3.00E+03 | 2.37 | 1 | 3.3 |
| 2 | 6.20E+05 | 2.7 | 1.60E+06 | 3.3 |
| 3 | 2.40E+08 | 2.53 | 3.00E+07 | 1 |

Table 10 – 39GHz.