3GPP TSG-RAN4 #107	R4-2307044

Incheon, South Korea, 22nd May 2023 – 26th May 2023

Title:	Discussion on definition of dualPA-Architecture capability indication for intra-band UL CA

Source:	Anritsu Limited

Agenda Item:	4.1

Document for:	Discussion

# Introduction

In the TS 38.306 [1] is given two definitions of the dualPA-Architecture capability indication/field, one for the CA-ParametersNR IE and another one for the MRDC-Parameters IE. This contribution concerns the one related on the CA-ParametersNR IE as shown below:

#### 4.2.7.4	CA-ParametersNR

[…]

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| dualPA-Architecture For band combinations with single-band with UL CA, this field indicates the support of dual PA and dual LO frequencies for FR1, or dual LO frequencies for FR2. If absent in such band combinations, the UE supports single PA and single LO frequency for all the ULs for FR1, or single LO frequency for all the ULs for FR2. For other band combinations, this field is not applicable. | BC | No | N/A | N/A |
| … | … | … | … | … |

Recently Anritsu studied various RAN4 documents such as WFs and LS related to the dualPA-Architecture capability indication of the CA-ParametersNR IE in order to know if we could make some assumptions for our tester when dualPA-Architecture is signalled, including if there was any relationship between dualPA-Architecture and UL-MIMO or Tx Diversity.

However, reading such documents did not allow to have clear view of the dualPA-Architecture capability indication mainly due our bias due to the name itself with us expecting for FR1 and FR2. The name of the capability includes the word “PA” so it was expected that even in FR2 there is some relation with PA whereas there is actually none. In the 38.306 is given the definition of the dualPA-Architecture capability indication which is used in the 38.101-1 [2] (but not in the 38.101-2 [3]).

It is after contacting Skyworks, OPPO and Ericsson that things got clearer. This contribution presents:

- a summary table of the different Intra-band CA cases with and without TxD or MIMO and shows when dualPA-Architecture capability is signalled based on discussions with Skyworks.

- a draft LS to RAN2 to modify the definition of the dualPA-Architecture capability indication of the CA-ParametersNR IE such that the probability of misunderstanding the definition is minimized.

# Discussion

## When is dualPA-Architecture capability signalled?

In Table 1 is listed the different cases of intra-band CA with the number of PAs and LOs, and if it is a case CA+TxD or CA+UL MIMO.

| Intra-band CA case | UL MIMO or TxD | # PAs | # LOs | Introduction | dualPA-Architecture is signalled | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Contiguous | No | 1 | 1 | 1# R16 | No | This is where 1 PA supports 200MHz Baseline Architecture for contiguous ULCA, From R16 |
| Contiguous | No | 2 | 2 | 1b# R16 as option for PA that do not support up to 200MHz BW | Yes | This is only an agreed implementation option for contiguous ULCA, the baseline architecture used to derive requirement (MPR/A-MPR) is based on 1PA covering the two CC (200MHz) and 1LO |
| Non-contiguous | No | 2 | 2 | 2# R16 | Yes | Apply to Rel-16 onwards for both FR1 and FR2 from RAN4 stand-point (38.101-1/2), but most relevant in actual implementations for FR1 Rel-17 onwards. Needed when gap > largest CC regardless of total BW (only way to meet the in gap SEM mask). From R16 |
| Contiguous | Yes | 2 | 1 | 3# R17, MIMO version of 1# | No | Baseline for MIMO+CA in R17 2CC per PA, 1 Tx path per layer |
| Non-contiguous | No | 1 | 1 | 4# R17, support of in gap exception (needs improved image and carrier) | No | 1PA support 2CC. CC1+gap+CC2<200MHz and Gap is <largest CC |
| Non-contiguous | Yes | 2 | 1 | 5# R17, MIMO version of 4# | No | CC1+gap+CC2<200MHz and Gap is <largest CC |

Table 1 - Simulation parameters

Observation 1: There are currently no cases where TxD or MIMO is used and where dualPA-Architecture capability is reported.

Proposal 1: The definition in the 38.306 could highlight that “dualPA-Architecture capability is not reported for either intra-band CA + TxD or intra-band CA + UL MIMO as a single LO is used.”

Observation 2: The two cases requiring reporting of dualPA-Architecture capability were introduced in Rel-16, no new case was introduced in Rel-17 and there is no plan in Rel-18 to introduce a new case.

## Definition of the dualPA-Architecture capability indication

The dualPA-Architecture capability indication is used in the 38.101-1 [2] (but not in the 38.101-2 [3]) and its definition is in the 38.306 [4], RF engineers do then need to rely on the 38.306 definition.

The current definition may lead to different interpretations due to the name of the capability and the formatting of the definition. It is better to have different sentences for FR1 and FR2, to help understanding and avoid confusion to the TS readers.

Observation 3: Different sentences for FR1 and FR2 may help understand and avoid confusion.

Proposal 2: Have different sentences for FR1 and FR2 to avoid confusion for FR2.

LS on clarification of dualPA-Architecture capability [4] was submitted and agreed during RAN4 #102-e:

dualPA-Architecture capability was originally introduced in Rel-15 to indicate whether UE using one PA or two PAs to support the intra-band UL CA and during the discussion of Rel-16 FR1 RF enhancements RAN4 extended the meaning of this capability to also imply the number of UE LO frequencies in supporting intra-band UL CA is not singular in FR1. During the discussion of Rel-17 FR1 RF enhancements, RAN4 clarified the implication and also applied it to FR2 as well. Therefore, RAN4 would like to respectfully ask RAN2 to extend the meaning of dualPA-Architecture capability in TS38.306 from Rel-16 if there is no NBC issue. The proposed changes are as below for consideration:

| dualPA-Architecture For band combinations with single-band with UL CA, this field indicates the support of dual PA and dual LO frequencies for FR1, or dual LO frequencies for FR2. If absent in such band combinations, the UE supports single PA and single LO frequency for all the ULs for FR1, or single LO frequency for FR2. For other band combinations, this field is not applicable. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

The following definition based on proposal 1 and proposal 2 would allow RF engineers to more clearly understand the dualPA-Architecture capability indication as of now.

| dualPA-Architecture For NR CA band combinations with two UL CCs in the same band, this field indicates for FR1 that the uplink part is supported by one PA and one LO frequency per CC i.e. dual PAs each with an LO/DC location that can be indicated. The same field indicates in case of FR2 that the uplink part is supported by one LO frequency per CC i.e. each having an LO/DC location that can be indicated, it does not indicate any specific number of PAs. If the field is absent for such a band combination, the uplink part is supported by a single PA and one LO/DC location in the case of FR1, and by one LO/DC location in the case of FR2 (no indication of any specific number of PAs). This field does not indicate a specific number of PAs when present or absent in the case of FR2. DualPA-Architecture capability is not reported for either “intra-band CA + TxD” or “intra-band CA + UL MIMO” as a single LO frequency is used. For other NR CA band combinations, this field is not applicable. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 3: Send a LS to RAN2 to request a modification of the definition of dualPA-Architecture capability indication for intra-band UL CA in the TS 38.306.

# Conclusion

Based on the discussion above, the following can be noted:

Observation 1: There are currently no cases where TxD or MIMO is used and where dualPA-Architecture capability is reported.

Observation 2: The two cases requiring reporting of dualPA-Architecture capability were introduced in Rel-16, no new case was introduced in Rel-17 and there is no plan in Rel-18 to introduce a new case.

Observation 3: Different sentences for FR1 and FR2 may help understand and avoid confusion.

Proposal 1: The definition in the 38.306 could highlight that “dualPA-Architecture capability is not reported for either intra-band CA + TxD or intra-band CA + UL MIMO as a single LO is used.”

Proposal 2: Have different sentences for FR1 and FR2 to avoid confusion for FR2.

Proposal 3: Send a LS to RAN2 to request a modification of the definition of dualPA-Architecture capability indication for intra-band UL CA in the TS 38.306.

# References

[1] TS 38.306, “NR; User Equipment (UE) radio access capabilities”, version 17.4.0

[2] 38-101-2, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”, version 18.0.0

[3] 38-101-2, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”, version 18.0.0

[4]	R4-2206503, “LS on clarification of dualPA-Architecture capability”, OPPO, RAN4 #102-e, Electronic meeting

# Annex 1 – Draft LS to RAN2

3GPP TSG-RAN WG4 Meeting #107	R4-23XXXXX

Incheon, South Korea, 22nd May 2023 – 26th May 2023

# Title:	LS on clarification of dualPA-Architecture capability for the CA-ParametersNR IE

# Release:	Rel-17

# Work Item:	NR_RF_FR1_enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name: TBD

#### E-mail Address: TBD

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None

1. Overall Description:

dualPA-Architecture capability was originally introduced in Rel-15 to indicate whether UE using one PA or two PAs to support the intra-band UL CA and during the discussion of Rel-16 FR1 RF enhancements RAN4 extended the meaning of this capability to also imply the number of UE LO frequencies in supporting intra-band UL CA is not singular in FR1. During the discussion of Rel-17 FR1 RF enhancements, RAN4 clarified the implication and also applied it to FR2 as well. Therefore, RAN4 would like to respectfully ask RAN2 to extend the meaning of dualPA-Architecture capability in TS38.306 from Rel-16 if there is no NBC issue. The proposed changes are as below for consideration:

| dualPA-Architecture For NR CA band combinations with two UL CCs in the same band, this field indicates for FR1 that the uplink part is supported by one PA and one LO frequency per CC i.e. dual PAs each with an LO/DC location that can be indicated. The same field indicates in case of FR2 that the uplink part is supported by one LO frequency per CC i.e. each having an LO/DC location that can be indicated, it does not indicate any specific number of PAs. If the field is absent for such a band combination, the uplink part is supported by a single PA and one LO/DC location in the case of FR1, and by one LO/DC location in the case of FR2 (no indication of any specific number of PAs). This field does not indicate a specific number of PAs when present or absent in the case of FR2. DualPA-Architecture capability is not reported for either “intra-band CA + TxD” or “intra-band CA + UL MIMO” as a single LO frequency is used. For other NR CA band combinations, this field is not applicable. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

2. Actions:

To RAN2 group.

ACTION: RAN4 respectfully ask RAN2 to take above into consideration.

3. Date of Next RAN WG4 Meetings:

3GPPRAN4#108		21 – 25 August 2023		Toulouse, France